3GPP TSG-WG SA2 Meeting #166	S2-2412224

18 - 22 November, 2024, Orlando, USA	(revision of S2-2411058)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5004 | rev | 5 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on subscription to UPF event exposure service via SMF during UPF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile], Nokia, ETRI, ZTE, Samsung, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During UPF relocation procedure, whether the UPF event consumers should subscribe the new UPF data to new UPF or whether the old UPF sends the data that has been collected but not sent to the consumer yet is based on the relocation reporting indication from consumer is FFS. Whether new subscription or not: If the UPF event consumer indicates the certain UPF id in subscription request or trigger direct subscription to certain UPF that UPF id indicates, after UPF relocation, the UPF event consumer may not trigger new subscription request to new UPF. If the UPF event consumer indicates the certain DNAI in subscription request, and after UPF relocation the DNAI is changes, for example relocates from one DNAI to another DNAI, the UPF event consumer doesn’t not trigger new subscription request to new UPF in new DNAI. Revision 5: N4 session release in general is not handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the SMF to subscribe the target UPF as optional, and add clarification about why not subscribe any more. Revision 5: N4 session release is handled and not just UPF relocation. Relocation reporting indication is renamed to Remaining dara reporting indication and a Subscription termination reporting indication is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear that the SMF always subscribes the UPF event. Revision 5: It is unclear how N4 session release is handled in terms of reporting remaining data and notifying subscription termination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

##### 4.15.4.5.6	Information flow for subscription to UPF event exposure service via SMF during N4 session release

Figure 4.15.4.5.6-1: Subscription to UPF event exposure service via SMF during N4 session release

This information flow is used in the scenario of N4 session release (e.g. PDU Session release or UPF relocation) for the existing PDU Session, e.g. the source UPF is I-UPF or local PSA UPF which is relocated, or the source UPF is PSA UPF which is relocated in SSC mode 3. Case when SMF relocation happens is not specified in this Release of the specification.

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF data via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UE. The consumer may include a Remaining data reporting indication, which indicates how to handle the data collected by the source UPF when N4 session is released, e.g. discard the collected data or send the collected data to the consumer. The consumer may include a Subscription termination reporting indication.

2.	The SMF subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation or N4 Session Modification with PFCP depending on UPF exposure event and which mechanism applies for each event as described in clause 5.2.26.2.1. The SMF may include the Remaining data reporting indication and the Subscription termination reporting indication.

3.	When the notification event is triggered the source UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

4.	The SMF determines that the N4 session of the UPF needs to be released (e.g., UPF relocation or PDU session release) and releases the N4 session.

5.	If step 2 included Subscription termination reporting indication, the UPF sends the notification to the UPF event consumer by invoking Nupf_EventExposure_Notify service operation, which includes a subscription termination indication to notify that the subscription to the UPF event has been terminated, and may include a cause code specifying the reason for subscription termination, The UPF may report to the consumer any data that has been collected but not sent to the consumer yet based on the Remaining data reporting indication. For example, for periodic reporting type of the UPF event exposure service when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change. Or for event triggered reporting with Reporting suggestion information, when the UPF changes, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet.

6.	In case of PDU Session release the SMF may send a subscription termination indication to the UPF event consumer by invoking Nsmf_EventExposure_Notify service operation and may also include a cause code indicating the reason for subscription termination, e.g., PDU Session release. The procedure terminates at this step.

7-8.	In case of UPF relocation step 6 is omitted and the SMF establishes N4 session with a target UPF. To continue the subscription of the UPF event consumer, if the target UPF related information is matched with the subscription request in step 1, the SMF may re-subscribe to the target UPF based on the subscription from UPF event consumer in step 1. When the notification event is triggered the target UPF will send the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

* * * End of Change * * *