3GPP TSG-WG SA2 Meeting #167	S2-2501621

Athens, Greece, February 17 – 21, 2025

# Title:	[Draft] Reply LS on UUAA Unsubscribe procedure

# Response to:	C4-245436, LS on UUAA Unsubscribe procedure

# Release:	Rel-17

# Work Item:	ID_UAS

Source:	Ericsson [to be SA2]

To:	CT4

Cc:	CT3

Contact Person:

Name:	Shabnam Sultana

E-mail Address:	shabnam.sultana@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: CR#0182, CR#0183, CR#0184

1.	Overall Description

SA2 thanks CT4 for the LS on UUAA Unsubscribe procedure. With this reply SA2 would like to provide a feedback on the unsubscribe operation for Naf/nef_Authentification_ services.

When comes to means to release the implicit subscription from UAS NF /NEF and USS:

- clause 4.4.1.2.2.1 of 3GPP TS 23.256 specifies that the implicit subscription due to a Naf_Authentication service request sent by a UAS NF / NEF is implicitly released by USS:

“When creating an authentication session, the UAS NF/NEF implicitly subscribes to USS about notification related with the authentication/authorization (e.g. re-authenticate, update authorization data or revoke the UUAA authorization). This implicit subscription is implicitly released by USS when the corresponding authentication session is removed (e.g. in the case of re-authentication failure and USS indicating to release network resource, or in the case of authorization revocation).”

- clause 4.4.1.1.2 of 3GPP TS 23.256 specifies that the implicit subscription due to a Nnef_Authentication service request sent by an AMF/SMF is implicitly released by UAS NF / NEF:

“When creating an authentication session, the AMF/SMF implicitly subscribes to NEF about notification related with the authentication/authorization (e.g. re-authenticate, update authorization data or revoke the UUAA authorization). This implicit subscription is implicitly released by UAS NF/NEF when the corresponding authentication association is removed (e.g. in the case of re-authentication failure and USS indicating to release network resource, or in the case of authorization revocation).”

In both cases, the implicit subscription is implicitly removed by the service producer, i.e. USS or UAS NF/NEF, and explicit service operations, which could have been used for this purpose, have been deliberately not specified. In the CT4-mentioned procedures (clause 5.2.2.2 and 5.2.3.2 of TS 23.256), the UUAA context in the UAS NF / NEF can be cleared after the implicit indication, specifically, based on the event exposure event (for example, “PDU Session Status” as defined in Table 4.15.3.1-1 of TS 23.502 or “Registration state changes” as defined in clause 5.2.2.3.1 of TS 23.502) and notification sent by the SMF/AMF to the UAS NF / NEF.

Considering the aforementioned aspects, SA2 clarifies in the attached CRs that the subscriptions are removed using implicit means.

2.	Actions

To CT4:

ACTION: 	SA2 kindly asks CT4 to take into account SA2 feedback.

To CT3:

ACTION: 	SA2 kindly asks CT3 to take into account SA2 feedback.

3.	Date of Next TSG SA WG2 Meetings

SA2#168	April 07 – 11, 2025	Göteborg, Sweden

SA2#169	May 19 – 23, 2025	Fukuoka, Japan