3GPP SA WG2 Meeting #167	 S2-2501836

# Athens, Greece, 17-21 February 2025								                 (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1392 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE LCS privacy check for ground truth data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE LCS privacy is a feature which allows a UE and/or AF to control which LCS clients and AFs are and are not allowed access to UE location information. The UE LCS privacy check was performed by GMLC when the location request is received from LCS clients or AF. In order to perform AI/ML model performance monitoring for LMF-based AI/ML Positioning, the MTLF/LMF may collect the UE location of UEs as ground truth, the UE LCS privacy check is also required before the ground truth collection from UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that LMF need to perform UE LCS privacy check before the ground truth data collection from UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - No UE LCS privacy check when LMF collects UE’s location as ground truth. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 6.2E.4	Procedure for MTLF-based AI/ML model performance monitoring for LMF-based AI/ML Positioning

NWDAF containing MTLF supports to perform the performance monitoring for AI/ML model. When NWDAF containing MTLF determines to monitor the AI/ML model performance, it triggers to collect the input data of PRU(s)/UE(s) by using the procedure of input data collection by NWDAF as specified in clause 6.22.4 of TS 22.273 [39]. But the input data for AI/ML model performance monitoring includes the ground truth data and the calculated location information of PRU(s)/UE(s) that is calculated by LMF-based AI/ML Positioning instead of measurement data of PRU(s)/UE(s). Before collecting the calculated location information from UE(s), LMF needs to check the user consent in UDM. LMF needs to perform the UE LCS privacy check before collecting ground truth data from UE(s). Based on the ground truth data and the calculated location information, NWDAF containing MTLF evaluates the AI/ML model performance to generate the performance monitoring result. The result may trigger NWDAF containing MTLF to retrain the AI/ML model, or sent to LMF to trigger LMF to change the positioning method, e.g. from LMF-based AI/ML Positioning to legacy positioning.

NOTE: 	The user consent check for calculated location information and UE LCS privacy check for ground truth data collection is not applied to PRU(s).

# >>>> NEXT CHANGES<<<<

# >>>>END OF CHANGES<<<<