3GPP TSG SA WG2 Meeting #168	S2-2502870

Gothenburg, Sweden, 2025‑04-07 – 2025-04-11	(revision of S2-220xxxx)

Source:	Apple

Title:	Generation of AF specific UE Identifier

Document for:	Discussion

Agenda Item:	19.49

Work Item / Release:	EDGEAPP / Rel-20

Abstract: This discussion paper analyzes the generation of AF Specific UE identifier in 5GC

# 1. Background

3GPP in Rel-17 has specified NEF service (Nnef_UEId_Get) for use by an AF to retrieve an identifier for a UE that is known to the AF only by its address (IP Address or MAC Address). The service supports retrieving MSISDN or an AF Specific UE Identifier under different conditions as specified in 3GPP TS 23.501 Clause 5.20.

Incoming LS from SA6 (S2-2502845/S6-250286) has raised questions on the assignment and generation of AF Specific UE identifiers. This discussion paper aims to analyze the generation of AF Specific UE identifier in UDM/UDR.

The AF Specific UE Identifier is assumed to be a temporary identifier and the AF shall not keep maintaining a mapping between this identifier and the UE IP address after the retrieval of an AF specific UE Identifier as this mapping may change. The AF specific UE Identifier is a GPSI in the form of an External Identifier and it shall be different for different AFs. The uniqueness of AF specific UE identifier across AFs is ensured by configuration in NEF, UDM/UDR as stated in the NOTE 2 in TS 23.501

NOTE 2: The AF specific UE identifier is ensured to be unique across different AFs as defined in TS 23.003 [19] by configuration. Such configuration is assumed to be coordinated between the different involved entities (e.g. NEF(s) and UDM/UDR).

# 2. Discussion

## 2.1 Generation of AF Specific UE Identifier

SA6 LS asks SA2 to provide insights into the assignment of AF Specific UE Identifiers. In general, preparing a set of identifiers per AF for each subscriber in advance is not required. This approach makes sub-optimal use of identifier space by assigning AF Specific UE Identifiers for all AFs that MNO may have an SLA with, but UE may not be aware of these AFs and may not connect to them. Moreover, this leads to the AF Specific UE identifier being tied to a subscription identifier, thus creating a permanent mapping between them. This appears to be going against the principle that using AF Specific UE Identifier instead of a permanent identifier was to enable an AF to use UE related exposure services where users do not want to share permanent identifiers with that AF.

Observation 1: Pre-assigning AF specific UE Identifiers in advance of an AF request creates permanent mapping between identifiers and UE Subscription identifiers. This is a potential security issue.

The 3GPP TS 23.003 clause 19.7.2 specifies the content of an External Identifier.

The External Identifier shall have the form username@realm as specified in clause 2.1 of IETF RFC 4282 [53]. The username part format of the External Identifier shall contain a Local Identifier as specified in 3GPP TS 23.682 [98]. The realm part format of the External Identifier shall contain a Domain Identifier as specified in 3GPP TS 23.682 [98]. As specified in clause 4 of IETF RFC 4282 [53], the Domain Identifier shall be a duly registered Internet domain name. The combination of Local Identifier and Domain Identifier makes the External Identifier globally unique. The result of the External Identifier form is: "<Local Identifier>@<Domain Identifier>"

The structure allows different MNO configuration for the AF Specific UE Identifier under the constraints detailed in TS 23.501 Clause 5.20.

As one example, an MNO may want to configure Domain Identifiers identifying the AF which requested UE Id retrieval. In this case the identifiers generated for different AFs will have the domain identifier part unique.

Another possibility is to use the domain identifier to represent the MNO but pre-allocate blocks of Local identifiers to different AFs that are authorized to request AF Specific UE identifiers. Since the AF has to be authorized before invoking the NEF service to retrieve AF specific UE identifiers, it is reasonable to assume the blocks of local identifiers are pre-assigned per AF. The UDM/UDR may dynamically generate identifiers within the block allocated to the requesting AF at the time of retrieval.

Both methods require some part of the AF Specific UE identifier to be statically assigned and a remaining part to be dynamically generated. This method ensures there is no wastage of identifier space and no security threat by creating a more permanent mapping to UE’s subscription identifiers.

Observation 2: MNO configuration allows part of the AF Specific UE identifier to be statically assigned and a remaining part to be dynamically generated.

The LS makes an observation that the possibility for returning an error from UDM to NEF (and NEF subsequently sending “404 Not Found” error status code) in case of failure to retrieve AF Specific UE Identifier points to a static assignment of AF Specific UE identifier by the MNO.

On the other hand, stage 3 in 29.522 clause 4.4.32.2 “Retrieve AF specific UE ID service operation” states that UDM would reply to NEF with a "404 Not Found" error status code if AF specific UE ID is not available (see related paragraph from clause 4.4.32.2 copy/pasted below): “If the UDM indicates that the requested UE Identifier is not available in the subscription data, the NEF shall respond to the AF with a "404 Not Found" error status code.”

However, the error in retrieval can occur even if the AF Specific UE Identifier were to be generated dynamically.

The retrieval may fail due to mis-configuration, e.g., AF requesting the identifier does not have an allocated block of identifiers in the MNO configuration (mismatch between NEF and UDM configuration) or running out of identifiers in the allocated block of local identifiers.

Observation 3: Retrieval of AF Specific UE identifier may fail due to different reasons even if they were generated dynamically, and the error response from NEF to AF with a “404 Not Found” seems to be in order.

# 3. Proposals

Based on the above analysis, the following are proposed to be included in a reply LS to SA6.

Proposal 1: Dynamic generation of AF Specific UE Identifier is not ruled out from Stage 2 specification.

Proposal 2: The error code returned by NEF for failure to retrieve AF Specific UE Identifier is required even when AF specific UE Identifier is dynamically generated.