3GPP TSG-SA2 Meeting #168	S2-2502872

Goteborg, Sweden, April 07 – 11, 2025

# Title:	[draft] Reply LS on support for multiple non-3GPP device identifiers

# Response to:	LS (S2-2502806) support for multiple non-3GPP device identifiers

# Release:	Rel-19

# Work Item:	UIA_ARC

Source:                  [Nokia, Will be] SA2

To:                          CT1

Cc:                          CT3

Contact Person:

Srinivas Garikipati

Srinivas.Garikipati@Nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

S2-2503330 (CR 6208)

S2-2503331 (CR 5432)

S2-2503332 (CR 1536)

1. Overall Description:

SA2 provides the following updates to CT1

CT1 technical details

The agreed SA2 CR 5963 (S2-2501225) to 23.501 states:

For the traffic of non-3GPP devices requiring differentiated QoS:

-	...(skip text)

If the PCF indicates to the SMF that the corresponding Non-3GPP Device Identifier is not available for the UE as specified in clause 6.1.3.31 of TS 23.503 [45], the SMF rejects the PDU Session Modification with a cause code to notify the UE that the Non-3GPP Device Identifier is not available for the UE.

From the marked yellow text, there is no consensus in CT1 on:

- whether the multiple non-3GPP device identifiers are allowed in a single PDU session modification procedure;

- whether the SMF rejects the Non-3GPP Device Identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message.

If only a single non-3GPP device identifier is allowed in the PDU SESSION MODIFICATION REQUEST message, the UE/5G-RG must initiate multiple PDU SESSION MODIFICATION REQUEST messages for each non-3GPP device that is connected behind the UE/5G-RG. CT1 discusses that this approach may lead to increased signalling overhead within the system if the UE/5G-RG needs to request QoS differentiation for multiple devices in a short time. For example, consider a scenario where many UE/5G-RG’s in an area are powered cycled and each needs to request QoS differentiation for multiple non-3GPP devices.

The CT1 group would like to highlight that, CT1 also found that TS 29.512 allows the SMF to report multiple non-3GPP device identifiers to the PCF in one message, as referenced in TS 29.512, Table 5.6.2.3-1.

| n3gDevInfos | map(Non3gppDeviceInfo) | O | 1..N | Contains information about the non-3gpp device(s) behind the UE using the PDU Session of the UE. The key of the map is the "n3gDevId" attribute of the Non3gppDeviceInfo data type. | Non3gppDevice |
| --- | --- | --- | --- | --- | --- |

CT1 requests Stage-2 to consider the scenario where multiple non-3GPP device identifiers are removed from the subscription in the UDR and the network therefore needs to stop QoS differentiation for the multiple devices. A key concern is identifying the most efficient mechanism to convey this updated information to the UE.

Given these considerations, CT1 requests SA2 to provide answers to the following questions:

Q1: Whether the multiple non-3GPP device identifiers are allowed in a single PDU SESSION MODIFICATION REQUEST message?

SA2 Answer:

SA2 considers that a UE / 5G-RG may associate traffic of a non-3GPP device to a Non-3GPP Device Identifier and it is very much possible that the UE / 5G-RG may carry out this process for multiple devices at the same time. Hence SA2 considers including multiple Non-3GPP Device Identifier s in a single PDU Session Modification Request. To reflect this change, CRs attached to this LS Reply are agreed.

Q2: For the Non-3GPP device identifier not available scenario, whether the SMF rejects the Non-3GPP device identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message?

SA2 Answer:

SA2 agreed to the concept of including multiple Non-3GPP Device Identifier s. The SMF includes the status of each of the Non-3GPP Device Identifier in response to the PDU Session Modification Request and does not reject the PDU Session Modification Request.

Q3: For the scenario of removal of more than one non-3GPP device identifier from the subscription in the UDR, whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message?

SA2 Answer:

Multiple Non-3GPP Device Identifier s is possible in a single PDU Session Modification Command.

2. Actions:

To CT1.

ACTION:

SA2 kindly asks CT1 to take above SA2 information into account and accordingly drive their specifications.

3. Date of Next SA2 Meetings:

|  | DATES | LOCATION | CITY |
| --- | --- | --- | --- |
| SA2#169: | 19 - 23 May, 2025 | Fukuoka | Japan |
| SA2#170: | 25 - 29 Aug, 2025 | Goteborg | Sweden |

SA2 Meeting calendar can be found at:

https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/