SA WG2 Meeting #168	S2-2503013

# Goteborg, Sweden, 7-11 April, 2025                            (revision of S2-25xxxxx)

Source: 	CATT

Title: 	Resolve EN on AF authorization

Document for: 	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to resolve the EN on AF authorization to TS 23.369.

1	Proposal

It is proposed to resolve the EN on AF authorization to TS 23.369.

# * * * * First change* * * *

5.6	AF authorization to the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as the authorization data for 3rd party AF in the UDR.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT services.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
| Allowed target AIoT devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization to the AF for the AIoT services includes two parts:

-	NEF performs AF request authorization for AF ID, Allowed area and Allowed service operations based on the AF Authorization Data from UDR, service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	AIOTF performs authorization of AIoT service requested by the AF for Allowed target AIoT Devices, by retrieving the AF authorization data stored in the UDR as described in above Table 5.6-1.

# * * * * End of Changes * * * *