SA WG2 Meeting #168	S2-2503016

7 - 11 April 2025, Goteborg, Sweden

Source:	ZTE

Title:	UP building for UE training data collection

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes a way for UP building between NWDAF and UE.

# 1. Discussion

3GPP SA2 Release 20 has approved the study item "AI/ML Core Network Enhancements Phase 2", which includes the research task of "whether and how to support standardized user plane transmission for AI/ML-related UE data collection." To enable the establishment of user plane connections from the UE to NWDAF, this paper proposes a way for UP building.

# 2. Proposal

This paper proposes to add a solution for Key Issue #1 to TR 23.700-04 as follows.

---------- Start of change ----------

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue 1> | <Key Issue #y> |
| #1 | X |  |
| #2 |  |  |

---------- Next change (all new text) ----------

6.x	Solution #X: UE policy enhancement to support data collection from UE

6.x.1	Functional Description

This solution proposes a user plane connection between NWDAF and UE, with some enhancements in the UP building request. The NWDAF received the request from AF and trigger UP building. After UE training data collection finished, NWDA checks the training data and sends them to NEF/AF.

6.x.2	Procedures

UEr

The untrusted AF can provide AI related data collection parameters to core Network as follow:

AF

NEF

NWDAF

UPF

SMF

AMF

1. UP building request

2. URSP matching

3. PDU session establishment

4.N4 session establishment

6. TLS connect build

5. PDU session establishment accept

7. UP binding request

8. UP binding response

9a. PDU session establishment complete

10. UE training data transfer

9b. PDU session establishment complete

11. UE training data check and confirm

12. UE training data transfer

13. UE training data transfer

Figure 6.x.2-1: User plane building between NWDAF and UE

- 0. The NWDAF has received UE training data collection request from AF.

- 1. NWDAF send UP building request to UE via AMF and gNB for AI/ML data collection, the message may contain NWDAF address information, UP binding ID (i.e. to associate the user plane connection to be established with the target UE), purpose of building user plane (e.g. AI/ML user plane data collection), training data type (e.g. CSI, SSB, RSRP), sample number, sample quality threshold (i.e. only samples that quality greater than the threshold can be sent to NWDAF).

- 2. The UE performs URSP rule matching based on the received parameters. For example, based on the parameter indicating "AI/ML user plane data collection indication" and the address information of the NWDAF, the UE matches the corresponding DNN and S-NSSAI in the RSD. Then, it requests the establishment of a PDU session on the matched DNN and S-NSSAI.

- 3. The UE sends a PDU session establishment request to the SMF, and the request message includes the DNN and S-NSSAI.

- 4. The SMF selects the appropriate UPF based on the S-NSSAI and DNN and initiates the N4 session establishment.

- 5. The SMF replies with a PDU session establishment acceptance message to the UE, and the PDU connection service between the UE and NWDAF is established.

- 6. The UE establishes a TLS connection with the NWDAF.

- 7. The UE initiates the user plane connection binding process. It sends a user plane connection binding request to the NWDAF, and the request message includes the user plane binding ID received in step 1.

- 8. The NWDAF replies with a user plane connection binding acceptance message to the UE.

- 9a/9b. The UE sends a user plane connection establishment complete message to the AMF. The AMF then forwards the user plane connection establishment complete message to the NWDAF.

- 10. The UE sends UE training data for AI/ML through the user plane to the NWDAF.

- 11.  The NWDAF check and confirm the UE training data meet the UE data collection requirements from AF.

- 12. The NWDAF sends the AI/ML-related data collected from the UE to the NEF. This message may include one or more data types requested by the AF along with the corresponding data.

- 13. The NEF maps the information received from the NWDAF into external information (e.g. mapping SUPI to GPSI) and sends the training data to the AF.

6.X.3	Impacts on services, entities and interface

The main enhancement for this solution is NWDAF build a user plane with UE. Also, the NWDAF may provide a purpose of building user plane (e.g. AI/ML user plane data collection) and data types/sample information to UE in the UP building request message. Lastly, the NWDAF needs to check and confirm the training data before it sends to NEF/AF.

---------- End of changes ----------