3GPP TSG-SA2 Meeting #168	S2-2503074

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6180 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on SM signalling including non-3GPP device identifiers for QoS differentiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Issue 1: When the UE is requesting QoS differentiation for traffic from a non-3GPP device, it initiates a PDU Session modification including Non-3GPP Device Connection Information. In case of error scenario, when Non-3GPP Device Identifier is not present in the information retrieved from the UDR, then PCF indicates the non-availability of such information to SMF leading to SMF rejecting the PDU Session modification to the UE (TS 23.503 clause 6.1.3.31). This implies that such PDU Session Modification procedure should be exclusively for requesting traffic differentiation. Issue 2: Currently the specification does not describe the case when UE determines that traffic differentiation for a non-3GPP device is no longer required, e.g., due to device disconnection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clarification in clause 5.52.3 that PDU Session Modification for requesting differentiated QoS includes no other requests from the UE. Added clarification in clause 5.52.2 that when UE determines that traffic differentiation is no longer required for certain traffic, it requests deletion of associated packet filters through a PDU Session modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A PDU session modification procedure initiated with multiple requests from the UE may be rejected due to the failure of traffic differentiation request for non-3GPP devices There is ambiguity about how UE indicates that traffic differentiation is no longer required. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.2, 5.52.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 5.52.2	Traffic identification

When a non-3GPP device is connecting to the UE, the UE may bind the Non-3GPP Device Identifier to a non-3GPP device, for the traffic of non-3GPP devices that require differentiated QoS. This binding enables the 5G System to distinguish between the traffic generated by different non-3GPP devices connected through the same UE.

NOTE 1:	How the UE identifies the non-3GPP device and binds the Non-3GPP Device Identifier to a non-3GPP device is implementation specific. How the Non-3GPP Device Identifier(s) that are associated with the UE's SUPI in the 5GC are known to the UE is implementation specific.

NOTE 2:	At any point in time the Non-3GPP Device Identifier can be bound to only one non-3GPP device.

Non-3GPP Device Identifier Information is stored in the UDR and includes a Non-3GPP Device Identifier and QoS Information. The content of the Non-3GPP Device Identifier Information is further described in clause 4.15.6.15 of TS 23.502 [3].

When the UE determines that differentiated QoS for a non-3GPP device is no longer required, the UE may initiate a PDU Session modification to request deletion of packet filters configured for such traffic.

Next Change

### 5.52.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS, the Non-3GPP Device Connection Information may be signalled by the UE as defined in TS 24.501 [47]. When a non-3GPP device is connected to the UE, the UE may include the Non-3GPP Device Connection Information in PDU Session Modification Request to SMF. The SMF forwards the Non-3GPP Device Connection Information to the PCF for policy control.

NOTE 1:	It is up to UE implementation to determine when to initiate PDU Session Modification procedure for updating the Non-3GPP Device Connection Information.

For an Ethernet PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier;

-	MAC address used in PDU session;

-	Optionally, VLAN tag ID that is associated with the non-3GPP device used in PDU session.

For an IPv4 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier;

-	IPv4 Address associated with the non-3GPP device used in PDU session;

-	Optionally, port ranges associated with the non-3GPP device used in PDU session.

For an IPv6 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier;

-	IPv6 Address/prefix(sub) associated with the non-3GPP device used in PDU session;

-	Optionally, port ranges associated with the non-3GPP device used in PDU session.

NOTE 2:	If IPv4v6 PDU Session is applied, it is up to UE implementation to determine to use IPv4 or IPv6 or both Address/prefix(sub) based on the associated traffic of the non-3GPP device and the IP allocation by the network to the PDU Session.

Editor's note:	The detailed encoding of Non-3GPP Device Connection Information will be defined in stage-3.

A PDU Session Modification initiated by the UE for requesting differentiated QoS by including the Non-3GPP Device Connection Information shall include no other requests from the UE.

If the PCF indicates to the SMF that the corresponding Non-3GPP Device Identifier is not available for the UE as specified in clause 6.1.3.31 of TS 23.503 [45], the SMF rejects the PDU Session Modification with a cause code to notify the UE that the Non-3GPP Device Identifier is not available for the UE.

End of changes