3GPP TSG-SA WG2 Meeting #168	S2-2503098

# Goteborg, SE, April 07 - 11, 2025										revision of S2-250

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1617 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Correction of procedures in clause AG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In current procedure NEF sends response to DC AS before DCSF and MF are contacted. If the procedure fails at some point, this requires an extra notification to be send to DC AS. - Unclear how the UE(s) can decide from which BDC (local or remote) to download the DC application if the network triggers establishment of an ADC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Main changes: - Moving response from NEF to DC AS after response from IMS AS is received to avoid potential extra notifications from NEF to DC AS in case an error occurs in the network. - Clarify that the UE(s) receive sufficient inforamtion in the SIP INVITE to decide from which BDC to download the DC application. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unncessary notifications to be sent from network to DC AS if an error occurs during event exposure handling. Unclear behaviour in UE regarding download of DC applications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AG.2.1.1, AG.2.1.2, AG.2.1.3, AG.2.1.4, AG.2.2.1, AG.2.2.2, AG.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>START OF CHANGES<<<<

### AG.2.1.1	Adding A2P Application Data Channel to an existing IMS session

Figure AG.2.1.1-1 depicts the call flow of adding A2P application data channel(s) to an existing IMS session. In this scenario, the MF-2 is used to anchor the application data channel between the UE B and the DC AS.

Figure AG.2.1.1-1: Procedure of adding A2P application data channel(s) to an existing IMS session

An IMS session between UE A and UE B has been established, a bootstrap DC has also been established and the DC AS has received the IMS session ID.

0.	DC AS determines to add A2P application data channel(s) to an existing IMS session, e.g. based on the event notification of the related IMS session to which the DC AS subscribed to.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add A2P application data channel(s) to the existing IMS session. The AF request contains the DC AS identifier as AF ID.

The DC AS includes the session ID of the existing IMS session to be updated, the operation type as Adding, media operation set including the parameters for the media type set as A2P ADC, the UE B as the target of the A2P ADC, the application binding information and the MDC2 media endpoint address (e.g. FQDN or IP address and port number) in Nnef_ImsSessionManagement_Update request. The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified of the session update progress.

NOTE 1:	To be able to initiate data sending to the UE via MF, the DC AS needs to be informed of the reserved media resources. and the DC AS needs to subscribe to the progress of the session update.

2.	The NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE B.

3.	The NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 requiring to add A2P application data channel(s) to the specific IMS session. The received session ID of the existing IMS session, the operation type as Adding, the media type as A2P ADC, the UE B as the target of the A2P ADC, the application binding information and the MDC2 media endpoint address shall be included in the request. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified of the session update progress.

4.	The IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE B and determines whether the data channel request should be notified to DCSF-2.

If UE B does not have subscription of IMS data channel or does not have the IMS DC capability, then the request shall be rejected with appropriate cause and subsequent steps are skipped.

If the same IMS data channel is already established and if there are no other operations included in the request, then the request shall be rejected with an appropriate cause and the following steps are skipped.

AS.

.	If the IMS AS-2 allows the request to proceed, the IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionUpdateEvent. The notification includes the necessary parameters based on the information extracted from media operation set parameters received in step 3 and other stored parameters in the IMS AS-2 if applicable.

5.	After receiving the session event control notification, the DCSF-2 determines the policy how to process the application data channel establishment request based on the related parameters in the notification and/or DCSF-2 service specific policy.

6.	The DCSF-2 determines that the added application data channel media requires the DC to be anchored in MF-2.

7.	The DCSF-2 instructs the IMS AS-2 to set up the A2P application data channel(s) by sending a Nimsas_MediaControl_MediaInstruction request.

8.	Based on the instruction from DCSF-2, the IMS AS-2 interacts with the MF-2 to allocate data channel media resource towards UE B and MDC2 media resource towards DC AS.

The IMS AS-2 also requests MF-2 to update DC AS side MDC2 media resource information using the received MDC2 media endpoint address in step 3.

9.	The IMS AS-2 may respond to Nimsas_MediaControl_MediaInstruction request.

10.	The DCSF-2 may respond to Nimsas_SessionEventControl_Notify.

11.	The IMS AS-2 returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

12.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

13.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel required by the DC AS, application binding information, the existing IMS session ID, together with the other existing media descriptions. The re-INVITE request contains sufficient information for the UE B, e.g., as part of the application binding information, to decide from which BDC (if there is more than one BDC) to download the DC application.

The IMS AS-2 sends the re-INVITE request(s) to the target UE B.

14.	The UE B may download the corresponding DC application signalled in the SDP offer, if not done already and associate it with the requested application DC.

15.	The DC media negotiation is completed and corresponding data channel(s) are established. The UE B returns a 200 OK response with SDP answer for the application data channel(s).

16.	The IMS AS-2 requests MF-2 to update UE B side media resource information based on the received SDP answer for the application data channel from the UE B.

17.	The IMS AS-2 sends ACK to the UE-B.

NOTE 2:	When the session update is successfully concluded, the IMS AS-2 notifies the DCSF-2 about the IMS session events with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7 with the event set to MediaChangeSuccessEvent. This is not depicted in the Figure.

18.	If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in step 1, the IMS AS-2 notifies the NEF on the progress of the session update with the Nimsas_ImsSessionManagement_Notify request. The MF-2 side MDC2 media information is included in the request.

19.	The NEF notifies the DC AS on the progress of the session update with the Nnef_ImsSessionManagement_Notify request, including the MF-2 side MDC2 media information.

20.	The DC AS updates its MF-2 side MDC2 media information and returns with Nnef_ImsSessionManagement_Notify response to the NEF.

21.	The NEF returns a Nimsas_ImsSessionManagement_Notify response to the IMS AS-2.

# >>>>NEXT CHANGE <<<<

### AG.2.1.2	Adding P2P Application Data Channel to an existing IMS session

Figure AG.2.1.2-1 depicts the call flow of adding P2P application data channel(s) to an existing IMS session. In this scenario, the MF-2 is used to anchor the application data channel between the UE B and UE A.

Figure AG.2.1.2-1: Procedure of adding P2P data channel(s) to an existing IMS session

An IMS session between UE A and UE B has been established, the bootstrap DCs have also been established and the DC AS has received the IMS session ID.

0.	DC AS may determine to add P2P application data channel(s) to an existing IMS session based on the event notification of the related IMS Session for which DC AS subscribed to.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add P2P application data channel(s) in the existing IMS session. The AF request contains the DC AS identifier as AF ID.

The DC AS shall include session ID of the existing session, the operation type as Adding, the media type as P2P ADC and the application binding information in Nnef_ImsSessionManagement_Update request. The DC AS includes the media operation set parameters and may include UE B and UE A as the targets of the P2P ADC as well as a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified for the progress of the session update.

NOTE 1:	To be able to initiate data sending to the UE via MF, the DC AS needs to be informed of the reserved media resources. and the DC AS needs to subscribe to the progress of the session update.

2.	The NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving the UE B.

3.	The NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 requiring to add P2P application data channel(s) in the specific session. The received IMS session ID of the existing session, the operation type as Adding, the media type as P2P ADC and the application binding information are included in the request. The NEF may include UE B and UE A as the targets of the P2P ADC, as well as a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified on the progress of the session update.

4.	The IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE B and determines whether the data channel request should be notified to DCSF-2.

If the UE B does not have subscription for IMS data channel or does not have the IMS DC capability, then the request shall be rejected with an appropriate cause and the subsequent steps are skipped.

If the same IMS data channel is already established and if there are no other operations included in the request, then the request shall be rejected with an appropriate cause and the subsequent steps are skipped.

If the IMS AS-2 allows the request to proceed, the IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionUpdateEvent. The notification includes the necessary parameters based on the information extracted from media operation set parameters received in step 3 and other stored parameters in the IMS AS-2 if applicable.

5.	After receiving the session event control notification, the DCSF-2 determines the policy how to process the application data channel establishment request based on the related parameters in the notification and/or DCSF-2 service specific policy. The DCSF-2 determines that the added application data channel requires the DC to be anchored in the MF-2.

6.	The DCSF-2 instructs the IMS AS-2 to set up the P2P application data channel(s) by sending a Nimsas_MediaControl_MediaInstruction request.

7.	Based on the instruction from DCSF-2, the IMS AS-2 interacts with the MF-2 to allocate data channel media resources towards UE B and UE A.

8.	The IMS AS-2 may respond to Nimsas_MediaControl_MediaInstruction request.

9.	The DCSF-2 may respond to Nimsas_SessionEventControl_Notify.

10.	The IMS AS-2 returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

11.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

12.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel, application binding information, the existing IMS session ID, together with the other existing media descriptions. The re-INVITE request contains sufficient information for the UE B, e.g., as part of the application binding information, to decide from which BDC (if there is more than one BDC) to download the DC application.

The IMS AS-2 sends the re-INVITE request(s) to the target UE B.

13.	The UE B may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

14.	The DC media negotiation is completed. The UE B returns a 200 OK response with SDP answer for the application data channel.

15.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel, application binding information, the existing IMS session ID, together with the other existing media descriptions. The re-INVITE request contains sufficient information for the UE A, e.g., as part of the application binding information, to decide from which BDC (if there is more than one BDC) to download the DC application.

The IMS AS-2 sends the re-INVITE request(s) to the target UE A.

16.	The UE A may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

17.	The DC media negotiation is completed. UE A returns a 200 OK response with SDP answer for the application data channel.

NOTE 2:	Steps 12-14 and steps 15-17 can be processed in parallel.

18.	The IMS AS-2 requests MF-2 to update UE B side and UE A side media resource information based on the received SDP answer for the application data channel from UE B and UE A.

19.	The IMS AS-2 sends ACK to the UE-B and UE A.

NOTE 3:	After the session is successfully updated, the IMS AS-2 notifies the DCSF-2 about the IMS session event with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7 with the event set to MediaChangeSuccessEvent. This is not depicted in the Figure.

20.	If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in step 1, the IMS AS-2 notifies the NEF for the progress of the session update with the Nimsas_ImsSessionManagement_Notify request.

21.	The NEF notifies the DC AS for the progress of the session update with the Nnef_ImsSessionManagement_Notify request.

22.	The DC AS may return a Nnef_ImsSessionManagement_Notify response to the NEF.

23.	The NEF may return a Nimsas_ImsSessionManagement_Notify response to the IMS AS-2.

# >>>>NEXT CHANGE <<<<

### AG.2.1.3	Adding P2A2P Application Data Channel to an existing IMS session

Figure AG.2.1.3-1 depicts the call flow of adding P2A2P application data channel(s) to an existing IMS session. In this scenario, the MF-2 is used to anchor the application data channel between the UE B, the UE A and the DC AS.

Figure AG.2.1.3-1: Procedure of adding P2A2P data channel(s) to an existing IMS session

An IMS session between UE A and UE B has been established, bootstrap DCs have also been established and the DC AS has received the IMS session ID.

0.	DC AS determines to add P2A2P application data channel(s) to an existing IMS session, e.g. based on the event notification of the related IMS Session to which DC AS subscribed to.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring adding P2A2P application data channel(s) in the existing IMS session. The DC AS shall include session ID of the existing session, the operation type as Adding, the media type as P2A2P ADC, the application binding information and the MDC2 media endpoint address (e.g. FQDN or IP address and port number) in Nnef_ImsSessionManagement_Update request. The DC AS includes the media operation set parameters and may include UE B and UE A as the targets of the P2A2P ADC, as well as a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified on the progress of the session update. The AF request contains the DC AS identifier as AF ID.

NOTE 1:	To be able to initiate data sending to the UE via MF, the DC AS needs to be informed of the reserved media resources. and the DC AS needs to subscribe to the progress of the session update.

2.	The NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving the UE B.

3.	The NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 requiring to add P2A2P application data channel(s) in the specific session. The received session ID of the existing session, the operation type as Adding, the media type as P2A2P ADC, the application binding information and the MDC2 media endpoint address shall be included in the request. The NEF may include UE B and UE A as the targets of the P2A2P ADC, as well as a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified for the progress of the session update.

4.	The IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE B and determines whether the data channel request should be notified to DCSF-2.

If the UE B does not have subscription of IMS data channel or does not have the IMS DC capability, then the request shall be rejected with appropriate cause and the subsequent steps are skipped.

If the same IMS data channel is already established and if there are no other operations included in the request, then the request shall be rejected with an appropriate cause and the subsequent steps are skipped.

If the IMS AS-2 allows the request to proceed, the IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionUpdateEvent. The notification includes necessary parameters based on the information extracted from Media operation set parameters received in step 3 and other stored parameters in the IMS AS-2 if applicable.

5.	After receiving the session event notification, the DCSF-2 determines the policy how to process the application data channel establishment request based on the related parameters in the notification and/or DCSF-2 service specific policy. The DCSF-2 determines that the added application data requires to anchor the DC in the MF-2.

6.	The DCSF-2 instructs the IMS AS-2 to set up the P2A2P application data channel(s) by sending a Nimsas_MediaControl_MediaInstruction request.

7.	Based on the instruction from DCSF-2, the IMS AS-2 interacts with the MF-2 to allocate data channel media resource towards UE B and UE A and the originating and terminating MDC2 media resource towards DC AS.The IMS AS-2 requests MF-2 to update DC AS side MDC2 media resource information using the received MDC2 media endpoint address (e.g. FQDN or IP address and port number) in step 3.

8.	The IMS AS-2 may respond to Nimsas_MediaControl_MediaInstruction request.

9.	The DCSF-2 may respond to Nimsas_SessionEventControl_Notify.

10.	The IMS AS-2 returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

11.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

12.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel, application binding information, the existing session ID, together with the existing media descriptions. The re-INVITE request contains sufficient information for the UE B, e.g., as part of the application binding information, to decide from which BDC (if there is more than one BDC) to download the DC application.

The IMS AS-2 sends the re-INVITE request(s) to the target UE B.

13	The UE B may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

14.	The DC media negotiation is completed. The UE B returns a 200 OK response with SDP answer for the application data channel.

15.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel, application binding information, the existing session ID with the existing media descriptions. The re-INVITE request contains sufficient information for the UE A, e.g., as part of the application binding information, to decide from which BDC (if there is more than one BDC) to download the DC application.

The IMS AS-2 sends the re-INVITE request(s) to the target UE A.

16.	The UE A may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

17.	The DC media negotiation is completed. The UE A returns a 200 OK respond with SDP answer for the application data channel.

18.	The IMS AS-2 requests MF-2 to update UE B side and UE A side media resource information based on the received SDP answer for the application data channel from UE B and UE A.

19.	The IMS AS-2 sends ACK to the UE-B and UE A.

NOTE 2:	After the session is successfully updated, the IMS AS-2 notifies the DCSF-2 about the IMS session event with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7 with the event set to MediaChangeSuccessEvent. This is not depicted in the Figure.

NOTE 3:	The details on how the DC AS updates its MF-2 side MDC2 media information are out of scope of 3GPP.

20.	If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in step, the IMS AS-2 notifies the NEF for the progress of the session update with the Nimsas_ImsSessionManagement_Notify request. The MF-2 side originating and terminating MDC2 media information is included in the request.

21.	The NEF notifies the DC AS for the progress of the session update with the Nnef_ImsSessionManagement_Notify request, including the MF-2 side originating and terminating MDC2 media information.

22.	The DC AS updates its MF-2 side originating and terminating MDC2 media information, returns a Nnef_ImsSessionManagement_Notify response to the NEF.

23.	The NEF returns a Nimsas_ImsSessionManagement_Notify response to the IMS AS-2.

# >>>>NEXT CHANGE <<<<

### AG.2.1.4	DC AS initiated adding bootstrap data channel to an existing IMS session

Figure AG.2.1.4-1 depicts the call flow of adding bootstrap data channel towards UE B to an existing IMS session. In this scenario, the MF-2 is used to anchor the bootstrap data channel between the UE B and the DCSF2.

Figure AG.2.1.4-1: Procedure of adding bootstrap data channel to an existing IMS session

An IMS voice session between UE A and UE B has been established.

0.	DC AS decides to add a bootstrap DC to the existing IMS session, which is notified based on the event notification of the related IMS session to which the DC AS subscribed to.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add a bootstrap data channel to the existing IMS session.

The DC AS includes the session ID of the existing IMS session to be updated, the operation type as Adding, media operation set including the parameters for the media type set as BDC, the UE B as the target of the BDC, in Nnef_ImsSessionManagement_Update request. The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified of the session update progress.

2.	The NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE B.

3.	If the NEF authorizes the request, the NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 requiring to add A2P application data channel(s) to the specific IMS session. The received session ID of the existing IMS session, the operation type as Adding, the media type as A2P ADC, the UE B as the target of the A2P ADC, the application binding information, and the MDC2 media endpoint address shall be included in the request. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified of the session update progress.

4.	The IMS AS-2 determines whether the data channel request should be notified to DCSF-2 based on subscription information and DC capability of UE B.

If UE B does not have subscription of IMS data channel or does not have the IMS DC capability, or then the request shall be rejected with appropriate cause and subsequent steps are skipped.

If the bootstrap DC is already established, the request shall be rejected with an appropriate cause and the following steps are skipped.

If the IMS AS-2 allows the request to proceed, the IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionUpdateEvent. The notification includes the necessary parameters based on the information extracted from media operation set parameters received in step 3 and other stored parameters in the IMS AS-2 if applicable..

5.	Steps 7 to 10 in clause AC.7.1 applies.

6.	The IMS AS-2 returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

7.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

8.	The IMS AS-2 generates a re-INVITE request in which the SDP offer contains the media information of the bootstrap data channel and existing voice media and sends the request to UE B.

9.	UE B responds 200 OK with SDP answer.

10.	Steps 17 to 18 in clause AC.7.1 applies.

11.	IMS AS sends ACK to UE B to complete the IMS session modification procedure.

12.	If the NEF included a Notification Target Address in the Nimsas_ImsSessionManagement_Update request in step 3, the IMS AS-2 notifies the NEF MediaChangeSuccess event with the Nimsas_ImsSessionManagement_Notify request.

13.	If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in step 3, the NEF notifies the DS AS MediaChangeSuccess event with the Nnef_ImsSessionManagement_Notify request.

14-15.	The DC AS and NEF return responses.

# >>>>NEXT CHANGE <<<<

### AG.2.2.1	Establishing an IMS session with a standalone bootstrap data channel

Figure AG.2.2.1-1 shows the establishment procedure of a standalone data channel session.

Figure AG.2.2.1-1: Establishment procedure of a standalone BDC session

0.	The DC AS decides to establish a standalone data channel session between UE A and UE B. A standalone data channel can be a standalone bootstrap or standalone application data channel. This call flow describes the case of a standalone bootstrap data channel.

1.	The DC AS sends a Nnef_ImsSessionManagement_Create request to the NEF to create a standalone DC session with UE A. The request includes calling ID (UE A), called ID (UE B), media information set including the instructions for the BDC, associated with the standalone IMS DC session and the DC AS identifier as AF ID.

The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Create request to be notified for the progress of the session update.

2.	The NEF queries the HSS for the address of the IMS AS serving the UE B if needed. If the UE B is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Create request from the DC AS with an error response and the subsequent steps are skipped.

3.	The NEF sends a Nimsas_ImsSessionManagement_Create request to the IMS AS-2 requiring to create a standalone DC session.

4.	The IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE to determine whether the data channel request should be notified to DCSF-2. If the user B does not have subscription for IMS data channel, then the request shall be rejected with appropriate cause.

If the IMS AS-2 allows the request to proceed, the IMS AS-2 sends a Nimsas_ImsSessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionCreateEvent. The notification includes the necessary parameters based on the information extracted from parameters received in step 3.

The DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl_MediaInstruction request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The procedures in clause AC.7 are reused to interact with MF.

The Nimsas_MediaControl_MediaInstruction request instructs the IMS AS to originate the indicated data channel(s) from the MF-2 using the MDC1 and/or MDC2 media endpoint address(es) as included in the Nimsas_ImsSessionManagement_Create request.

NOTE:	The type of data channel established is dependent on the instruction given by DCSF.

5.	The IMS AS-2 returns a successful Nimsas_ImsSessionManagement_Create response to the NEF including a Session ID.

6.	The NEF returns a successful Nnef_ImsSessionManagement_Create response to the DC AS.

7.	The IMS AS-2 generates INVITE request(s) in which the SDP offer contains the media information of the data channel(s) required by the DC AS. The IMS AS uses the calling and called parties as indicated in the Nimsas_SessionManagement_Create request in step 3 as targets when sending the INVITE requests.

The IMS AS-2 sends the INVITE request(s) according to the IMS DC session establishment procedures described in clauses AC.7.1 and AC.7.2 to UE A and UE B with the SIP header "display-name" indicating that IMS AS-2 sends the INVITE request on behalf of the callee.

8.	The DC media negotiation is completed and corresponding data channel are established.

9.	The IMS AS notifies the DCSF for the IMS session events with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7.

If the DC AS included a Notification Target Address to the Nnef_ImsSessionManagement_Create request in step 1, the IMS AS notifies the NEF for the progress of the session creation with the Nimsas_ImsSessionManagement_Notify service operation.

10.	The NEF notifies the DC AS for the progress of the session creation with the Nnef_ImsSessionManagement_Notify service operation.

# >>>>NEXT CHANGE <<<<

### AG.2.2.2	DC AS initiated IMS session establishment with a standalone A2P application data channel

Figure AG.2.2.2-1 depicts the call flow of establishing an IMS session with a standalone A2P application data channel between the DC AS and UE A.

Figure AG.2.2.2-1: Establishment procedure of an IMS session with a standalone A2P application data channel

0.	The DC AS decides to establish an IMS session with a standalone A2P application data channel with UE A.

1.	The DC AS sends a Nnef_ImsSessionManagement_Create request to the NEF requiring to create an IMS session with a standalone A2P application DC with UE A.

The DC AS sends the media operation set including the parameters for the media type set as A2P ADC and the UE A as the target of the ADC to NEF. The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Create request to be notified for the progress of the session creation.

2.	If the NEF authorizes the request, the NEF queries the HSS with Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE A.

3.	The NEF sends a Nimsas_ImsSessionManagement_Create request to the IMS AS-1 requiring to create an IMS session with a standalone A2P application DC. The media operation set includes the parameters for the media type set as A2P ADC and the UE A as the target of the A2P ADC. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Create request to be notified of the session create progress.

4.	The IMS AS-1 validates the user subscription data and checks the IMS DC capability of UE A to determine whether the data channel request should be notified to DCSF-1.

If UE A does not have subscription of IMS data channel or does not have the IMS DC capability, then the request shall be rejected with appropriate cause and the subsequent steps are skipped.

If the IMS AS-1 allows the request to proceed, the IMS AS-1 sends a Nimsas_SessionEventControl_Notify request to the DCSF-1 with event set to 3rdPartySessionCreate. The notification includes the necessary parameters based on the information extracted from the media operation set received in step 3 and other stored parameters in the IMS AS-1 if applicable.

5.	After receiving the session event control notification, the DCSF-1 determines the policy how to process the bootstrap data channel establishment request based on the related parameters in the notification and/or DCSF-1 service specific policy.

6.	The DCSF-1 determines that the standalone A2P application data channel media requires the DC to be anchored in MF-1 and creates the originating side MDC1 media information required by MF-1.

7.	The DCSF-1 instructs the IMS AS-1 to set up the standalone A2P application data channel by sending a Nimsas_MediaControl_MediaInstruction request.

9.	Based on the instruction from DCSF-1, the IMS AS-1 interacts with the MF-1 to allocate data channel media resource towards UE A and MDC1 media resource towards the DCSF.

9.	The IMS AS-1 returns the Nimsas_MediaControl_MediaInstruction response to the DCSF-1.

10.	The DCSF-1 returns the Nimsas_SessionEventControl_Notify response to the IMS AS-1.

11.	The IMS AS-1 returns a successful Nimsas_ImsSessionManagement_Create response to the NEF.

12.	The NEF returns a successful Nnef_ImsSessionManagement_Create response with the session ID generated by the IMS AS-1 to the DC AS.

13.	The IMS AS-1 generates an INVITE request in which the SDP offer contains the media information of the standalone A2P application data channel required by the DC AS, binding information and the session ID. The INVITE request contains sufficient information for the UE A, e.g., as part of the application binding information, to decide from which BDC (if there is more than one BDC) to download the DC application.

The IMS AS-1 sends the INVITE request to UE A.

If UE A has not downloaded the DC application specified in binding information, then the UE A needs to download the DC Application.

14.	DC media negotiation may be completed through 18X/PRACK/200 OK(PRACK)/200 OK(INVITE)/ACK procedure.

15.	The IMS AS-1 requests MF-1 to update DC AS side media resource information.

16.	UE A runs the DC application specified in binding information and establish the standalone A2P application data channel with the DC AS.

17.	If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Create request in step 1, the IMS AS-1 notifies the NEF for the progress of the session creation with the Nimsas_ImsSessionManagement_Notify request.

18.	The NEF notifies the DC AS for the progress of the session creation with the Nnef_ImsSessionManagement_Notify request.

19.	The DC AS returns a Nnef_ImsSessionManagement_Notify response to the NEF.

20.	The NEF returns a Nimsas_ImsSessionMangement_Notify response to the IMS AS-1.

# >>>>NEXT CHANGE <<<<

## AG.2.3	Terminating standalone Data Channel session

Figure AG.2.3-1 shows the terminating procedure of a standalone data channel session. A standalone data channel can be a standalone bootstrap or standalone application data channel.

Figure AG.2.3-1: Terminating of a standalone data channel Session

1.	The DC AS sends a Nnef_ImsSessionManagement_Delete request to the NEF requiring terminating a standalone DC session with UE A. The request includes the Session ID to be terminated.

2.	The NEF queries the HSS for the address of the IMS AS-2 serving the specific UE if needed.

3.	The NEF sends a Nimsas_ImsSessionManagement_Delete request to the IMS AS-2 requiring to terminate the standalone data channel session identified by the Session ID.

4.	The IMS AS-2 releases DC resources in the MF-2 and sends the BYE request according to existing IMS procedures to terminate the data channel session.

5.	The IMS AS-2 returns a successful Nimsas_ImsSessionManagement_Delete response to the NEF.

6.	The NEF returns a successful Nnef_ImsSessionManagement_Delete response to the DC AS.

7.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request including the event SessionTerminationEvent to notify DCSF-2 that a session is terminated.

# >>>>END OF CHANGES<<<<