# 3GPP TSG-SA WG2 Meeting #168																S2-2503171

# Goteborg, Sweden, April 07 – 11, 2025

Source:	OPPO

Title:	Discussion on single LCS-UPP connection maintenance

Agenda item:	9.6.2 (5G_eLCS_Ph3)

Document for:	Decision

1. Introduction

As agreed in S2-2502542, in Rel-18 one UE shall have maximum one LCS-UPP connection, but how to maintain one LCS-UPP connection and avoid extra LCS-UPP connection establishment is not resolved.

Currently two alternative ways forward are identified, one is to let the UE make the decision on rejecting extra LCS-UPP connection establishment request, the other is to let the AMF make the decision. The system impacts of the two alternatives are discussed as follows.

2. Discussion

As specified in 6.18.1 and 6.18.2 of TS 23.273, LMF and UE use Namf_Communication_N1N2Transfer and UL NAS TRANSPORT messages to transfer LCS-UPP configuration info (known as UPP-CM messages defined in TS 24.572).

For Namf_Communication_N1N2Transfer message, as specified in 5.2.2.3.1 of TS 29.518, the UPP-CM message is transferred as a N1 message container from LMF to AMF. The AMF can know the N1 message container is “UPP-CM” from the N1MessageClass parameter as specified in 6.1.6.3.5 of TS 29.518. However, the AMF does not know the action of this “UPP-CM” message (i.e. as specified in clause 10.3 of TS 24.572, UPP-CM messages could include several types such as ser plane connection establishment command, ser plane connection release command, etc.).

Similarly, for UL NAS TRANSPORT message, AMF can only know this NAS message includes “UPP-CMI container” as specified in 8.2.10.7 of TS 24.501. AMF does not know the action of this “UPP-CMI” message.

As specified in 6.18.1 and 8.3.2.9 of TS 23.273, AMF can get the connection status from LMF by Nlmf_location_UPNotify message, but only “established, released or move” indication can be received. If the connection establishment procedure failed, the AMF will not be notified.

As specified in clause 10.3 of 24.572, UPP-CM messages are designed specifically to resolve all the LCS-UPP connection related issues including connection establishment, connection establishment failure report and connection release etc.

Based on the information above, we can have the observations as follows:

Observation #1: General NAS message transfer mechanism is used by LMF and UE to transfer the UPP-CM messages. The content of UPP-CM message is transparent to AMF.

Observation #2: AMF can only get the final result from the LMF when the establishment procedure is successful. If the procedure failed, the AMF gets no information.

Observation #3: UPP-CM messages between UE and LMF are used for all the LCS-UPP connection control plane interaction.

Based on the observations above, we can further compare the two alternatives.

For UE decision alternative, the UE should identify extra LCS-UPP connection establishment request from the LMF and reject the request with a proper failure cause in UPP-CM message. The UPP-CM message sent from the LMF should also be enhanced to indicate whether this is a relocated LCS-UPP connection for LMF relocation case. The AMF is not impacted since all these information is transferred within the UPP-CM container.

To conclude, for UE decision alternative, following enhancements are required:

- The UPP-CM message User plane connection establishment command should be enhanced to indicate whether this is caused by LMF relocation.

- The UPP-CM message User plane connection establishment failure message should be enhanced to indicate the cause is LCS-UPP connection maximum number limitation.

- UE side logic should be enhanced to prevent extra LCS-UPP connection establishment request initiated by the UE. The UE should also reject extra LCS-UPP connection establishment command sent from the LMF with proper cause value.

Observation #4: UE decision alternative only requires minor enhancements on UPP-CM messages without impacts on AMF.

For AMF decision alternative, the following Issues should be resolved:

- AMF should reject all the UPP-CM messages transfer requests (blindly rejection without knowing the true action of the UPP-CM messages) from other LMFs if a LMF has already established LCS-UPP connection with the UE. A new LCS-UPP specific failure cause value should be designed as the response to the LMF. However, as specified in 6.1.6.3.6 of 29.518, currently no service specific cause value is defined.

- When several LMFs try to send UPP-CM messages towards the UE in a short period. The AMF should only allow one LMF’s DL UPP-CM message transfer to avoid UE impact. But the AMF does not know whether the allowed LMF will finally suceed to establish the UPP connection. It is not clear how long should the AMF block the other LMFs. And this blocking behavior also needs a LCS-UPP specific failure cause value. If this is not resolved, the UE will accept multiple establishment commands and UE side decision cannot be avoided.

- The AMF logic should be enhanced to handle the LMF relocation case. It is possible that the target LMF decides not to establish LCS-UPP connection with the UE or failed to establish the LCS-UPP connection. Thus the logic of the AMF is that the AMF allows all the DL UPP-CM messages transfer from the target LMF and rejects all the other DL UPP-CM messages from other LMFs. But how long to block the other LMFs’ requests is also not clear since the target LMF may not establish the LCS-UPP connection.

- It should be guaranteed that the source LMF’s connection is already released before receiving the target LMF’s establishment request, or the UE side decision is needed. But how to make AMF ensure this is not clear since currently the source LMF and target LMF interact with the UE independently.

In order to resolve the issues above, a basic AMF solution is as follows:

- New LCS-specific N1N2MessageTransferCause value is required for different cases as discussed for issue 1,2,3,4.

- Several timers for different cases at the AMF is needed to block the other requests are needed for issue 2,3,4 to make sure the UE will not receive extra connection establishment command when a connection is already existing. After timer expires or AMF gets acknowledge of on-going connection establishment procedure failed or abandoned, the AMF should allow new UPP-CM messages transferred to the UE.

Notice that if the timer is not long enough, the UE may get extra connection establishment requests when a LCS-UPP connection is already existing or is being established. Then UE decision is not avoided. But if the timer is too long, user experience will be impacted since it is possible that the UE has no existing LCS-UPP connection but new LCS-UPP connection establishment commands are blocked since the timer is not expired.

Observation #5: New LCS-specific N1N2MessageTransferCause values should be designed to resolve different issues for AMF decision solution.

Observation #6: Complex new timers and different kinds of logic should be added at the AMF side for different issues to prevent UE impacts.

Observation #7: UE impacts cannot be avoided if timers are not long enough while user experience could suffer if timers are too long.

To conclude, the comparison table is proposed as follows:

| Impacts | UE decision | AMF decision |
| --- | --- | --- |
| UE | - New indicator in UPP-CM connection establishment command. - New cause value in UPP-CM connection failure message. - New logic at UE side to support one single connection. | Same requirements as UE decision if timers are not used or not long enough. |
| AMF | None. | 1. New LCS-specific N1N2MessageTransferCause values for different rejecting cases. 2. Different kinds of logic and contexts maintenance for different cases. (at least 4 for the issues listed above) 3. Different pending timers to block the extra requests. (at least 3 for the issues listed above) |
| User Experience | None. | Yes, if timer is used. User experience suffers if timers are too long. |

Conclusion: Based on the observations and information above, it is proposed to use UE decision alternative as way forward.

3. Proposal

The UE decision alternative is proposed as the way forward for single LCS-UPP connection maintenance, following enhancements are required:

1.	The UPP-CM message User plane connection establishment command should be enhanced to indicate whether this is caused by LMF relocation.

2.	The UPP-CM message User plane connection establishment failure message should be enhanced to indicate the cause is LCS-UPP connection maximum number limitation.

3.	UE side logic should be enhanced to prevent extra LCS-UPP connection establishment request initiated by the UE. The UE should also reject extra LCS-UPP connection establishment command sent from the LMF with proper cause value..

Proposals above are provided in S2-2503173.