3GPP TSG-WG SA2 Meeting #168	S2-2503209

# Sweden, Goteborg, 7 – 11 April 2025

Title:	DRAFT LS Reply on Transport Level Marking on N3/N9

Response to:	C4-250629 LS on Transport Level Marking on N3/N9

Release:	Rel-19

# Work Item:	XRM_Ph2

Source:	SA2

To:	CT4

Cc:

#### Contact Person:	Paul Schliwa-Bertling

#### E-mail Address:	paul.schliwa-bertling @ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: CR6193 (TS23.501)

1. Overall Description:

SA2 would like to thank CT4 for their LS on Transport Level Marking on N3/N9, has agreed on the attached CR to clarify the issue in TS 23.501 and would like to provide the following feedback:

Question 1:

For QoS Flows that are configured for PDU Set QoS handling, the SMF may additionally take into account the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a list of transport level packet marking values for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values. When an I-SMF/I-UPF is inserted in the PDU Session, the I-SMF may instruct the I-UPF to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet.

CT4 would like to ask if the highlighted requirement in yellow implies a new requirement for N4 interface specific for XRM_Ph2?

Answer 1: The I-SMF shall be able to provide a new indication to the I-UPF through the N4 interface to request derivation of the transport level packet marking of each outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet, instead of providing a list of transport level packet markings based on importance values.

Question 2: If so, how can the I-SMF learn from the anchor SMF to instruct the I-UPF for a QoS flow to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet?

Answer 2: When the I-SMF is notified that PDU Set handling is configured for the QoS flow by the presence of PDU Set QoS parameters in the QoS profile or a DL PDU Set Information Marking Support Indication, it may instruct the I-UPF to derive the transport level packet marking of the outgoing N3 downlink packet based on the one of the incoming N9 downlink packet.

Question 3: More generally, could SA2 clarify how transport level marking should be performed by I-UPF?

Answer 3: When a new indication to derive the outgoing N3 DSCP value from the value received in the incoming N9 packet is received from the I-SMF in the FAR, the I-UPF shall take the DSCP value received in the outer header of the incoming N9 downlink packets and set that value as the DSCP of the outer header of the outgoing N3 packets.

2. Actions:

To CT4

ACTION:  SA2 kindly asks CT4 to take the above information into account and update their specifications accordingly, if necessary.

3. Dates of Next SA2 Meeting:

SA2#169                     		May 19th – 23th, 2025	                                      Fukuoka, Japan