3GPP TSG-SA WG2 Meeting #168	S2-2503221

# Gothenburg, Sweden, 07-11 April 2025

Source:	Qualcomm Incorporated

Title:	WT1: Solution for providing IMS voice over GEO via User Plane

Agenda item:	20.1.1

Document for:	Discussion/Approval

1. Introduction

SA#107 approved the study for the support of IMS voice over GEO via NB-IoT NTN connecting to EPC.Under WT-1, SA2 agreed to study CP and UP option for the support of IMS voice over GEO via NB-IoT NTN connecting to EPC. This contribution addresses the system architecture, the bearer mapping and the protocol overhead for the UP option.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1.X | 1.Y | 1.Z |  |
| Y |  | X |  |  |

* * * Next Change * * * *

### x.y.z	Solution Y:	Support of IMS voice over GEO via NB-IoT NTN connected to EPC using user plane bearers

#### x.y.z.0	Introduction and principles of the solution.

This solution addresses WT1/Key Issue 1.Y and supports IMS voice call GEO via NB-IoT NTN connected to EPC using UP bearers.

The solution is based on exchanging the IMS signaling and the voice packets over two separate Data Radio Bearers (DRB) for the same PDN Connection.

#### x.y.z.1	Requirements

To support IMS voice service, two SDFs need to be supported: one for IMS signalling and the other for voice packets. In the UP option, these two SDFs are carried on the radio interface over data radio bearers.

The voice packet flow is a real-time flow with stringent delay requirements. Due to a high round trip time over a GEO satellite link, this flow does not lend itself to acknowledged mode transmission. The IMS signalling flow requires high reliability and can tolerate some delay.

The transport of the voice packet flow should be performed with the minimum protocol overhead possible.

#### x.y.z.2	Network architecture

Figure x.y.z.2-1 below shows high level architecture for IMS voice over GEO via NB-IoT NTN connected to EPC using user plane bearers.

Figure x.y.z.2-1: Architecture for IMS voice over GEO via NB-IoT NTN connected to EPC using UP

DRB1 uses RLC AM. DRB2 can use RLC UM or RLC TM.

Editor's note:	The handling of the PDN connection(s) for the IMS signalling and the voice packet flow is FFS.

#### x.y.z.3	PDN types

See section x.y.z.3 in Solution X (ref. S2-2503220).

#### x.y.z.4	Protocol overhead for the voice packet flow

The protocol overhead for the voice flow (DRB2) is discussed below. This overhead is per voice packet.

MAC layer overhead: MAC layer overhead is one byte, since there will be only one MAC PDU per TBS.

RLC layer overhead: For RLC UM it is one byte. For RLC TM it is zero byte.

Editor's note:	Whether RLC UM or RLC TM is sued is FFS based on RAN2 feedback.

PDCP overhead:

PDCP security overhead consists of 4 bytes for MAC (integrity protection) and one byte for the Sequence Number (SN). VoLTE is currently transported without integrity protection in all commercial deployments since User Plane Integrity Protection in E-UTRAN/EPC, even though standardized, is not commercially deployed. If, accordingly, user plane integrity protection is not required, the PDCP overhead could be one byte.

Editor's note:	Whether voice packets can be transported via DRB2 without integrity protection is FFS.

PDCP RoHC overhead:

-	For IP PDN type: RoHC overhead after successful RoHC context establishment is 3 bytes. However, the IP/UP/UTP overhead without RoHC (e.g. during RoHC context (re-)establishment) is at least 30 bytes.

-	For non-IP PDN type (RTP/NIDD), the RTP overhead is one byte (for RTP sequence number).

Table x.y.z.4-1 summarizes the overhead size for different protocol layers and provides the total overhead for the best-case and the worst-case scenario.

Table x.y.z.4-1 Protocol overhead for IMS voice over GEO using user plane

| Protocol layer | Description | Best Case | Worst case | Comments/impacts |
| --- | --- | --- | --- | --- |
| MAC | Single MAC PDU per TBS | 1B | 1B | Supported in the specs. |
| RLC | TM or UM | 0 (TM) | 1B (UM) | RLC TM is not implemented for DRBs. |
| PDCP security | MAC and SN | 1B | 5B | 1B requires MAC suppression. |
| PDCP RoHC |  | 1B | 3B-30B | 1B for PDN type non-IP. 3B for RoHC. 30B during RoHC context (re-)establishment. |
| TOTAL |  | 3B | 10B-37B | Key issues affecting OH: - MAC suppression - IP vs non-IP PDN type - For IP PDN type: RoHC OH during - RoHC context (re-)establishment. |

The overhead data rate depends on the packet rate. For example, for a 3B and a 6B overhead per packet and 50 packets per second (one packet each 20 ms), the overhead data rate would be 1200 bps and 2400 bps, respectively.

* * * End of Changes * * * *