SA WG2 Meeting ##168	S2-2503227

07 - 11 April, 2025, Goteborg, Sweden

Title:	[Draft] Reply LS on ML Model provisioning by OAM to LMF for UE positioning

Response to:	Reply LS on ML Model provisioning by OAM to LMF for UE positioning from SA2 (S2-2502840/ S5-250826)

Release:	Rel-19

Work Item:	AIML_MGT_Ph2

Source:	SA2

To:	SA5

Cc:	-

# Contact Person:

# Name:	Dr. Thomas Belling

E-mail Address:	 thomas (dot) belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: Agreed CR 712 for TS 23.273.

# 1	Overall description

SA2 thanks SA5 for their reply LS on ML Model provisioning by OAM.

Based on the reply provided by SA5, SA2 has agreed the attached CR to reflect that the LMF may also request a trained ML model for LMF-based AI/ML Positioning from OAM and OAM may provision such a model to the LMF.

SA2 would also like ro provide feedback on the following issues raises by SA5 in their LS:

To streamline the management of ML models for UE positioning across their entire lifecycle (including training/re-training, testing, emulation, deployment, and inference), both the training data collection and performance monitoring of the ML model for UE positioning and the inference function needs to be supported in OAM. In this regard, SA5 would need SA2’s input on defining the mechanisms to collect relevant data, specifically:

- Performance measurements (as defined in TS 28.552), and/or

- UE-level measurements (referencing TS 32.422 and TS 28.558).

SA2 has already defined the Nlmf_DataExposure service to enable a service consumer to subscribe to maesurements and ground truth data as required to train a model for LMF based positioning and monitor its accuracy. If the NWDAF trains a model for positioning at the LMF, it uses this service. SA2 recommends that OAM also uses this service.

However, RAN groups have decided not to support the use case of LMF based ML positioning based on UE-level measurements in Rel-19, and SA2 thus removed related procedures. Only a collection of ground-truth data from UEs is thus supported by the Nlmf_DataExposure service.

# 2	Actions

To SA2

ACTION: 	SA2 asks SA5 to kindly take the reply and related agreed SA2 CR into account.

# 3. Date of Next SA2 Meetings:

TSG SA WG2 Meeting #169	19th May – 23rd May 2025		Fukuoka, Japan

TSG SA WG2 Meeting #170	25th May – 29th August 2025	Goteborg, Sweden