SA WG2 Meeting ##168	S2-2503248

07 - 11 April, 2025, Goteborg, Sweden

# Title:	[Draft] Reply LS on Questions on stage 2 requirements for AIML_CN

# Response to:	LS on Questions on stage 2 requirements for AIML_CN (S2-2502816/ C4-250630)

# Release:	Rel-19

# Work Item:	AIML_CN

Source:	SA2

To:	CT4

Cc:	CT3

Contact Person:

# Name:	Dr. Thomas Belling

E-mail Address:	 thomas (dot) belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 1432 for TS 23.288, CR 5429 for TS 23.502

1. Overall Description:

SA2 thanks CT4 for their LS. SA2 has agreed the attached CRs to address the comments of CT4 and would also like to answer the questions raised by CT4 as follows:

1) Table 6.22.2-5 (Data collection from NRF/OAM) in TS 23.288 contains the following Note: " NOTE: If this information is not available in the OAM, it can be obtained from the NRF if available." CT4 observes that the above Note would result in overlapping functionalities in OAM and NRF APIs. Q1: Given that the metrics can be reported by OAM APIs, is it necessary for the NRF APIs to also support reporting the same metrics, and if so, can SA2 explain why?

SA2 Answer 1:  SA2 agrees that all the information listed in table 6.22.2-5 is available via OAM, see clause 5.10 in TS 28.552. SA2 thus removed the NOTE to avoid duplicated functionality.

2) if the answer to Q1 is yes, TS 23.502 suggests using the Nnrf_NFManagement_NFStatusSubscribe service operation for subscribing and reporting the aforementioned metrics: "- NF load status information, capacity and priority information of NFs and NF Services, etc as described in Table 6.x.2-2 and Table 6.x.2-5 of TS 23.288 [50]." The service operation description states: " Description: Consumer can subscribe to be notified of the following: - Newly registered NF along with its NF services. - Updated NF profile. - Deregistered NF." Q2: If the answer to Q1 is yes, this required functionality does not fit with the current scope of the Nnrf_NFManagement_NFStatusSubscribe service operation. CT4 kindly asks SA2 to clarify which service operation is suitable for this purpose?

SA2 Answer 2: SA2 agrees that the intention of the Nnrf_NFManagementStatusSubscribe service operation is a subscription for information available in NF profiles, but the information listed in table 6.22.2-5 is not part of any NF profile. While the information about load, capacity, and priority related to NFs and NF service instances as listed in Table 6.22.2-2 is part of NF profiles, SA2 does not see a need to introduce related filters in the subscription. SA2 thus removed the quoted bullet.

3) Table 6.22.2-1 (UE related Context Data collection) of TS 23.288 specifies: An NF instance may comprise multiple NF service instances supporting the same and/or different service names, e.g. for different network slices, possibly deployed in different Data Centers. Q3: Accordingly, should NF metrics related to signalling storm information be reported per Service Name (i.e. with the NF instance aggregating metrics of all NF service instances supporting the same service name) or per NF service instance?

SA2 Answer 3: SA2 agrees that it is appropriate to report metrics per service instance rather than per service name if such information is available. An NF instance can comprise multiple service instances supporting the same service (name) e.g. for different network slices, and so aggregating all the metrics per service (name) would not allow the consumer to have a correct representation of the traffic handled by all these service instances, e.g. per network slice. SA2 understands that the peer NF can know the NF service instance of the producer via the information provided as part of the binding mechanism. Thus SA2 clarified in Table 6.22.2-1 that Request type designates a NF service instance when available and otherwise NF service name.

4) Clause 5.2.29.2.1 of TS 23.502 specifies: " Abnormal Signalling characteristics: An NF, NF set, Service Type, may be provided as target. A reporting threshold (in terms of absolute transaction or deviation from average, or in terms of a minimum percentage failed transactions) " Q4: Does “Service Type” refer to “service name”? If the answer is yes, should SCP metrics related to signalling storm information be reported per "Service Name" or "Service instance"? If the answer is no, what does "Service Type" refer to?

SA2 Answer 4: SA2 agrees that it is desirable to support reporting per service instance rather than per service name if such information is available for reasons outlined in response to question 3. SA2 understands that the SCP can know the NF service instance of the producer via the information provided as part of the binding mechanism. Thus SA2 clarified in Clause 5.2.29.2.1 of TS 23.502 that the subscription can be per NF service name or per NF service instance and that the reported “Request type” designates a NF service instance when available and otherwise NF service name.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 to take the provided answers into account.

# 3. Date of Next SA2 Meetings:

TSG SA WG2 Meeting #169	19th May – 23rd May 2025		Fukuoka, Japan

TSG SA WG2 Meeting #170	25th May – 29th August 2025	Goteborg, Sweden