3GPP TSG-SA WG2 Meeting #168	S2-2503313

Goteborg, Sweden, April 07 – 11, 2025

Source:	MediaTek Inc.

Title:	Solution for WT#3: Support of UP path adjustments for the UE

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes solution to the key issue for WT#3 on Support of UP path adjustments for the UE.

* * * Start of Change * * * *

6.X	Key Issue #X: Support of UP path adjustments

6.X.1	Description

This solution is to address the issue for the UP path adjustments for the UE based on the energy attributes and energy related information from EIF.

The UP path adjustment is to reselect UPF by SMF based on the energy attributes and energy related information. The trigger events for SMF to reselect a UPF for a PDU Session can be:

-	from PCF due to policy change/update after PCF interacts with EIF

-	from EIF and OAM due to the update of energy attributes and energy related information

6.X.2	Architectural assumption

The architecture can be referred to clause 4.2.18 of TS 23.501 [y1] to enhance the service operations from EIF to other 5G NFs.

6.X.3	UP path adjustment

The UPF can be (re)-selected by SMF as specified in clause 6.3.3 of TS 23.501 [y1] due to e.g., UE mobility, UE traffic offloading. Since SMF can have service operations with EIF to obtain the energy related information, and the SMF can re-select a UPF based on the obtained energy related information. Furthermore, the SMF can also utilize the NRF to reselect a UPF.

Editor’s NOTE: How SMF interacts with NRF with the enhancement to NF profile to reselect a UPF is FFS.

When the SMF decided to re-select a UPF for UE traffic routing, the SMF can trigger PDU Session establishment modification which includes the N4 Session modification procedure in clause 4.4.1.3 of TS 23.502 [y2] to reselect a UPF and update the corresponding N4 rules to the re-selected UPF.

Besides the above, it is possible that after performing the SM policy association modification procedure with PCF, the SMF may also trigger UP path adjustment to reselect a UPF

6.X.4	Procedures for UP path adjustment

The existing procedures as specified in TS 23.502 [y2] such as PDU Session modification procedure can be re-used by SMF to reselect a UPF for UE traffic routing.

6.X.5	Impacts on existing services, entities and interfaces

SMF:

-	may reselect a UPF for a PDU Session for UE traffic routing based on

-	after interacting with EIF

-	after interacting with PCF

-	after interacting with NRF

PCF:

-	generate PCC rules to SMF that may trigger to reselect a UPF after interacting with EIF

EIF:

-	interact with SMF such that SMF may trigger to reselect UPF for UE traffic routing

* * * End of Change * * * *