3GPP TSG-SA2 Meeting #168	S2-2503389

07 - 11 April 2025, Goteborg, SE

Source:	China Telecom

Title:	23.700-19: Key issues for QoS aspect of GEO call

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper proposes new key issue for QoS aspect of GEO call

1 Discussion

-	WT-1: IMS voice call over GEO via NB-IoT NTN connecting to EPC

NOTE 1:	both CP and UP based NB IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2:	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

-	WT-1.1 Whether and how to support QoS for IMS voice service over GEO satellite,

NOTE 3:	Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

NOTE 4:	There will be NO mobility enhancement developed for the NB-IoT NTN in this WT.

NOTE 5:	Depending on operators’ policies, IMS enhancements defined in the WT-1 may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.

The Work Task#1.1 of the FS_5GSAT_Ph4_ARC SID is copied as below:

We observe that existing mechanism for NB-IoT access(including TN and NTN) dose not support GBR bearer and differentiated QoS bearers within a PDN connection, which are legacy QoS mechanism to support IMS voice call. If NB-IoT access needs to follow existing QoS mechanism for IMS voice call over E-UTRAN access, then coordination between SA and RAN is needed, regardless of UP or CP solution.

Based on previous discussion paper from several companies, we can observe that either CP or UP based solution has its pros and cons, e.g. CP has higher QoS than UP, UP has less overhead than CP. In this study we need to evaluate between different solutions, e.g. CP or UP or combined CP/UP, to meet and balance the various requirements as described in TS 22.261 of SA1(e.g. call setup time, transmission overhead).

2 Proposal

It is proposed to agree the new key issue into TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows

* First change (all New)*

5.X	Key Issue #X: QoS support for IMS voice service over GEO satellite

5.Y.1	Description

The existing mechanism for NB-IoT NTN dose not support GBR bearer and differentiated QoS bearers within a PDN connection, which are legacy QoS mechanisms to support IMS voice service, regardless of data transmisstion of NB-IoT NTN is over UP or CP. It can be estimated that to support legacy QoS mechanism for NB-IoT NTN, both EPC and NB-IoT RAN need enhancement.

To support IMS voice service over NB-IoT GEO satellite, the following issues need to be studied:

- Whether and how to support QoS for IMS voice service over NB-IoT NTN GEO access.

NOTE 1:	Both CP, UP and combined CP/UP based NB IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2:	Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

NOTE3:  The evaluation between different solutions should meet and balance the various requirements as described in TS 22.261 of SA1(e.g. call setup time, transmission overhead)

* End of change *