3GPP TSG SA WG2#168	            S2-2503408

Goteborg, Sweden, April 07 – 11, 2025	(revision of S2-250XXXX)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0305 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interaction between SMF and I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As described in DP S2-2503407, it is suggested to clarify that : - How the N4 rule is configured at the UPF controlled by the I-SMF. - How the LOM Supported Indicatoin and LOM Allowed Indication is exchanged between I-SMF and SMF. - LOM support indication is not used at the AMF. It is AMF support or not support LOM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - I-SMF notifies the SDF filter(s) of the offloaded traffic to SMF. Per the received SDF filter(s), the SMF generates N4 information. - When the I-SMF received the LOM Allowed Indication from AMF and its support LOM, then LOM Supported Indication is sent to SMF. And SMF send back the LOM Allowed Indication back to I-SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inefficient mechanism to let the SMF check all EDI/AFTI and also some unnecssary filter are added at the UPF controlled by the I-SMF. - I-SMF need send different indication to SMF per different procedure and unclear the SMF behaviour when it receives the LOM Allowed Indication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1, 6.10.2.3, 6.10.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE

6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF, as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The I-SMF may insert a Local PSA and optional an additional UL CL into the data path due to traffic offloading, e.g., per the result of the DNS message handling. The I-SMF notifies the SDF (Service Data Flow) Filter(s) of the offloaded traffic to the SMF. Per the received SDF Filter(s) information, the SMF generates N4 information and sends it to the I-SMF to configure N4 rules on UL CL and Local PSA.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

2nd CHANGE

6.10.2.3	EAS Discovery Procedure with EASDF for Local Offloading Management

The EAS Discovery Procedure with EASDF for Local Offloading Management follows the same procedure as defined in clause 6.7.2.3, with the following differences:

-	General:

-	VPLMN is replaced by HPLMN;

-	V-EASDF is replaced by EASDF;

H-UPF is replaced by UPF;

-	H-SMF is replaced by SMF;

-	V-SMF is replaced by I-SMF;

-	V-UPF is replaced by I-UPF;

-	VPLMN Specific Offloading Information is replaced by Local Offloading Management Information.

-	Step 1:

-	Clause 6.7.2.2 is replaced by clause 6.10.2.2;

-	For option a), the SMF configures EASDF with the DNS server in the central part of the DN for FQDNs that are unauthorized for Local Offloading Management in the local part of the DN and EASDF may provide an EDNS Client options related to the PSA UPF in messages sent to this DNS server.

-	Step 3:

-	NOTE 2 and NOTE 3 are not applicable.

-	The I-SMF notifies the SDF (Service Data Flow) Filter(s) of the offloaded traffic to the SMF. Per the received SDF Filter(s) information, the SMF generates N4 information and sends it to the I-SMF to configure N4 rules on UL CL and Local PSA.

6.10.2.4	UE mobility for PDU session supporting Local Offloading Management

For a PDU session supporting Local Offloading Management, during mobility events, e.g. Hand-Over or AMF change, the mobility handling is executed same as the respective mobility handling defined in clause 4.23 of TS 23.502 [3] and clause 6.7.2 with the following difference:

-	The retrieval of the Local Offloading Management Allowed indication either as part of UE context where source AMF provides the target AMF with whether Local Offloading Management is allowed for each PDU session of the UE or the SMF selection subscription data from UDM, the new AMF determines the new I-SMF as follows:

-	If the service area of SMF includes the new UE location and the UE location is out of local offloading management service area, the AMF removes the old I-SMF (if available).

-	If the service area of SMF does not include the new UE location and the UE location is out of local offloading management service area, the AMF follows the procedure defined in clause 4.23 of TS 23.502 [3] without supporting Local Offloading Management with the following differences:

-	If the old I-SMF cannot serve the new UE location, the AMF performs I-SMF change procedure described in Figure 4.23.7.3.2-1 of TS 23.502 [3]. During the handover execution phase per clause 4.23.7.3.3 of TS 23.502 [3], the new I-SMF invokes Nsmf_PDUSession_Update Request to the SMF. Then, the SMF sends PCO to the UE including central DNS server address or EASDF IP address selected by SMF.

-	If the old I-SMF can serve the new UE location, the AMF may keep the old I-SMF and send Nsmf_PDUSession_UpdateSMContext Request without local offloading management allowed indication. In this case, the old I-SMF deletes the DNS Context from the old EASDF and sends Nsmf_PDUSession_Update Request without local offloading management support indication to the SMF so that the SMF sends PCO to the UE including central DNS server address or EASDF IP address selected by SMF.

-	If the service area of SMF either includes or does not include the new UE location and the UE location is within local offloading management service area, the AMF changes the existing I-SMF with the new I-SMF that serves the local offloading management service area unless the existing I-SMF can serve the new UE location and local offloading management service area. If there is an old I-SMF which cannot serve the new UE location, the AMF also removes the old I-SMF. When the new I-SMF interact with SMF, if local offloading management allowed indication is received from AMF, the new I-SMF sends local offloading management support indication to the SMF and receives the local offloading management allowed indication from the SMF.

-	To support the local offloading management, the selected I-SMF needs to support Local Offloading Management, serves the new UE location and local offloading management service area; and support the DNN and S-NSSAI associated with the PDU session.

NOTE:	When the UE moves from an AMF which is not upgraded to act based on the Local Offloading Management to an AMF which is upgraded to act based on the Local Offloading Management, the above procedure also applies.

-	For I-SMF insertion/change/removal/reuse, the procedure described for intra (V-)PLMN HO in clauses 6.7.2.6, 6.7.2.7, 6.7.2.8, 6.7.2.9 and 6.7.2.10 is performed with the following differences:

-	removing V-PLMN, H-PLMN, and steps defined for serving PLMN change case;

-	replacing:

-	V-SMF with I-SMF;

-	H-SMF with SMF;

-	V-EASDF with EASDF;

-	V-UPF with I-UPF;

-	HR-SBO with Local Offloading Management (including allowed indication, policy and offload identifiers);

-	enabling the anchor SMF to send PCO to the UE; and

-	not using PLMN ID in Neasdf_DNSContext_Create Request from I-SMF to retrieve EASDF IP address.

End of CHANGES