SA WG2 Meeting #168	S2-2503481

07th – 11th April, 2025, Goteborg, Sweden

Source:	ETRI

Title:	New Solution to KI#1: Policy enhancement for UE-side data collection over UP

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_ AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for for WT#1 of FS_AIML_CN_Ph2.

1. Introduction

The Work Task#1 of the FS_AIML_CN_Ph2 SID is:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

Regarding the work task #1, this contribution proposes the following enhancement to the policy provisioning:

- The UE may indicate “AI/ML for NR air interface operation indication” in the UE policy container.

- The PCF may send “UE-side data collection policy” to the UE. The “UE-side data collection policy” may have “AI/ML for NR air interface operation allowed,” as well as the IP address and port designated for UE-side data collection.

- The PCF may validate the UE’s consent regarding “UE data collection” for AI/ML for NR air interface operation.

2. Proposale

It is proposed to agree the following updates to TR 23.700-04.

*** Start of Change ***

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #y> |
| #X: Policy enhancement for UE-side data collection over UP | X |  |
| #2 |  |  |

6.X	Solution #X:  Policy enhancement for UE-side data collection over UP

6.X.1	Description

The UE must have an IP address and port for the purpose of facilitating UE-side data transfer over the UP. Packets destined for the allocated IP address and port assigned to the UE will be visible and managed by the 5GCN and subsequently transferred to the OTT server, external to the MNO.

It is mandatory for the 5GCN to validate the UE’s consent prior to collecting UE-side data. Only data collected from UEs that have provided explicit consent may be transmitted to the OTT server outside the MNO.

The UE may indicate its readiness for UE-side data transfer by incorporating the “AI/ML for NR air interface operation indication” within the UE policy container of the NAS Mobility Management (MM) message, specifically in the Registration Request.

The PCF may deliver the “UE-side data collection policy” to the UE that has granted consent for UE-side data collection. This “UE-side data collection policy” may include provisions such as “AI/ML for NR air interface operation allowed,” as well as the IP address and port designated for UE-side data collection.

The UE may further utilize the received IP address and port designated for UE-side data collection to transfer its collected UE-side data.

Editor's note: It is FFS how the PCF obtains the IP address and port for UE-side data collection, which may be configured in the PCF or retrieved from in the UDR.

Editor's Note: It is FFS whether the IP address and port pertain to the UPF, OTT servers, or a third network function of 5GCN.

6.X.2	Procedures

Impact on Registration procedure:

The procedure itself remains unchanged; however, the content of the UE policy container will be enhanced as follows:

-	The UE policy container may further include an “AI/ML for NR air interface operation indication.”

Impact on UE Configuration Update procedure:

The procedure itself remains unchanged; however, the content of the UE policy information will be enhanced as follows:

-	The UE policy information may further include “AI/ML for NR air interface operation allowed,” as well as the IP address and port for UE-side data collection.

6.X.3	Impacts on services, entities and interfaces

Impact on UE:

-	The UE may send “AI/ML for NR air interface operation indication” to the 5GCN.

-	The UE may receive “UE-side data collection policy” from the 5GCN.

Impact on PCF:

-    The PCF may create “UE-side data collection policy” and provision it to the UE.

Impact on UDR:

-	The UDR may store “UE-side data collection consent” in subscription data of a UE.

-	The UDR may provision “UE-side data collection consent” to the PCF.

*** End of Change ***