3GPP TSG-WG SA2 Meeting #168	S2-2503517

# Gothenburg, Sweden, 7th Apr – 11th Apr, 2025	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1544 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for multiplexed media traffic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The details for the PCF behavior when generating PCC rules for the case that the multiplexed media flows cannot be separated in UL direction need to be specified. Without this specification, the AF would not know with which QoS the UL traffic of the multiplexed media flows is handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of details for PCF behavior when generating PCC rules for the case that the multiplexed media flows cannot be separated in UL direction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The QoS for the UL traffic of the multiplexed media flows is not known to the AF and thus, the AF cannot perform the necessary actions/adjustments when the multiplexed media flows cannot be separated in UL direction. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.1.3.27.7	Traffic identification and differentiated QoS for multiplexed media flows

Traffic identification and differentiated QoS for multiplexed media flows is as defined in clause 5.37.11 of TS 23.501 [2] and enables differentiated QoS handling for (S)RTP media flows that are multiplexed on the same end-to-end transport layer connection.

When the UE indicates the support for (S)RTP Multiplexed Media Identification Information in IP Packet Filters to SMF at PDU Session establishment, the SMF provides this UE capability information to PCF.

In order to request differentiated QoS for multiplexed media flows from the PCF, the AF has to provide per media flow the (S)RTP Multiplexed Media Identification Information (see TS 23.501 [2] clause 5.7.6.2) in the Flow Description and the specific QoS requirements (described in clause 6.1.3.22).

The PCF determines the PCC rules and QoS parameters for the multiplexed (S)RTP media flows as follows:

-	When the UE capability information indicates support for (S)RTP Multiplexed Media Identification Information in IP Packet Filters, or the AF request is for the downlink direction only, the PCF generates a separate PCC rule for every media flow in the same way it is described in clause 6.1.3.22. Each PCC rule includes the QoS parameters of the corresponding media flow and it’s (S)RTP Multiplexed Media Identification Information in the IP Packet Filter set.

-	In all other cases (i.e. the AF request includes media flows with an uplink direction and the UE has not indicated support for (S)RTP Multiplexed Media Identification Information in IP Packet Filters), the PCF may, based on operator configuration, reject the AF request indicating lack of UE support for (S)RTP Multiplexed Media Identification or generate PCC rules and notify the AF that the differentiated QoS handling for multiplexed media flows is not supported by the UE.

NOTE 1:	In both cases the AF can provide explicit QoS requirements for the end-to-end transport layer connection in uplink direction by requesting unidirectional media flows (i.e. the downlink media flow(s) separated from the combined uplink media flow).

When the PCF generates PCC rules, the following applies:

-	For the downlink direction of the media flows, the PCF generates separate PCC rules in the same way it is described by the first bullet.

-	For the uplink direction of the media flows, the PCF generates PCC rule(s) (i.e. a single PCC rule or separate PCC rules) with the QoS parameters set according to operator configuration apart from the GBR parameter which is set according to the individual media flow bitrate requirement (in the case of separate PCC rules) or set to the combined media flow bitrate requirements (in the case of a single PCC rule).

NOTE 2:	The operator configuration is expected to contain the QoS parameters to be used for the combined uplink direction of the media flows according to the SLA between operator and application service provider.

NOTE 3:	It is assumed the PSA UPF supports the IP Packet Filter with (S)RTP Multiplexed Media Identification Information.

# * * * * End of changes * * * *