3GPP SA WG2#168	S2-2503649

Gothenburg, Sweden, 7-11 April, 2025

Source:	Tejas Networks

Title:	Discussion on Nx reference point interface between AIOTF and AIoT RAN

Spec:	3GPP TS 23.369

Agenda item:	19.14.2

Work Item / Release: AmbientIoT-ARC / Rel-19

Document for:	Approval

Abstract of the contribution: This contribution is to discuss the choice of reference point interface between AIOTF and AIoT RAN.

1. Introduction

This contribution is to decide on the reference point interface between AIOTF and AIoT RAN. The Nx reference point interface was previously introduced in 3GPP TS 23.369 to connect AIOTF with  AIoT RAN.

As the per the outcome of the Ambient IoT study in RAN WG3, some decisions listed in Annexure 1 were taken by the working group and provided as an LS to SA2#168 in S2-2502832. Based on the working assumption mentioned in the LS, the following observations are made with reference to the system architecture for AIoT.

[Observation-1] Both direct and indirect path communication between the AIOTF and AIoT RAN will be carried out using information containers in NGAP.

[Observation-2] In case of indirect path, the communication over the Nx reference point between AIOT RAN and AIOTF will be over NGAP between AIOT RAN and AMF followed by the use of Naiotf interface between AMF and AIOTF.

[Observation 3]: In case of direct path, communication over the Nx reference point shall be over NGAP between AIOT RAN and AIOTF. This necessitates two NGAP end-points in AIOT RAN, one for N2 and other for Nx.

We recommend the use of single interface i.e., the N2 interface between the 5GC and the AIoT RAN, over which Nx messages may be transported. This retains the architectural sanctity of the 5G end-to-end architecture in its present form without introducing multiple NGAP end-points in the system. This recommendation also provides the additional flexibility of incorporating a limited set of N2 functionality within the AIOTF to support RAN only deployments without a significant impact to the 5G system architecture.

In view of the above discussion, we request that the working assumption made by RAN3 be reevaluated and the use of a new protocol (AIoT-RC) over NGAP be considered instead. Since NGAP is a stateful protocol, the proliferation of NGAP within the network is not recommended. Moreover, using NGAP between the AIoTF and AIoT RAN is unsuitable for migration to future architectures which may employ service-based interfaces between RAN and core. Therefore, we propose the following:

[Proposal 1]: The working assumption of including AIOTF information containers in NGAP be reevaluated by RAN3, with a recommendation from SA2 to consider the use of new protocol over NGAP.

[Proposal 2]: Retain the reference point Nx between AIOTF and AIoT RAN.

2. Proposal

It is proposed to agree the following TP to 3GPP TS23.369:

* * * * First Change * * * *

### 4.2.2	Architecture for AIoT RAN connectivity

#### 4.2.2.1	General

Editor's note:	The term AIoT RAN needs to be aligned with RAN WGs throughout the TS.

Editor's note:	Whether the connection to the AIOTF should be Nx or N2 is to be determined.

5GS system architecture for AIoT supports the following paths to access an AIoT RAN:

-	Direct Connectivity: AIOTF communicates with AIoT RAN directly.

-	Indirect Connectivity via AMF: AIoT RAN and the AIOTF communicate indirectly via an AMF.

Figure 4.2.2.1-1 depicts the complete non-roaming architecture showing the overall 5GS architecture for support of AIoT, including both the Indirect Connectivity and Direct Connectivity.

Figure 4.2.2.1-1: Non-roaming AIoT System Architecture

NOTE 1:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na and N2 as depicted using dashed lines, as all deployments might not use them.

* * * * Next Change * * * *

Figure 4.2.2.1-2 depicts the complete non-roaming AIoT system architecture, using the reference point representation.

Figure 4.2.2.1-2: Non-roaming AIoT System Architecture (AIoT RAN Readers) in reference point representation

NOTE 2:	For the sake of clarity of the point-to-point diagrams, the AF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDR, NEF and NRF as necessary.

NOTE 3:	For clarity, the UDR and its connections with other NFs, e.g., ADM, are not depicted in the point-to-point and service-based architecture diagrams. For more information on the ADM data storage architectures refer to clause 4.5.8.

NOTE 4:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na, N2 and Nx as depicted using dashed lines, as all deployments might not use them.

The architectures in the following clauses showing parts of the overall AIoT architecture specific to each connectivity option:

-	Direct Connectivity: the AIOTF uses Nx to access AIoT RAN, and is described in clause 4.2.2.2.

-	Indirect Connectivity via AMF: the AIOTF uses an AMF which uses N2 to access AIoT RAN, and is described in clause 4.2.2.3.

NOTE 4: For both, direct and indirect connectivity, the interface between AIOTF and AIoT RAN is Nx. Nx is transported over N2. The placement of the N2 end-point is left to the implementation. It can be collocated with AMF (in case of indirect path communication) or collocated with AIOTF (in case of direct path communication). This implementation flexibility is provided to support AIoT implementations without a full fledged 5GC network.

NOTE 5:	A deployment that only uses, e.g., the Direct Connectivity only does not need to deploy the NFs, reference points and service-based interfaces associated with the Indirect Connectivity, and vice-versa.

* * * * End of Changes * * * *

3. Annexure 1 : Excerpt of RAN3 LS to SA WG2 :

RAN3 would like to inform SA2 and SA5 about the following RAN3 progress related to Architecture and OAM aspects:

- Support both the direct and indirect communication options between the gNB and the AIOTF in Topology 1.

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.