SA WG2 Meeting #168	S2-2503686

07 - 11 April, 2025, Goteborg, Sweden	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.369: AIoT Service Operation Result Aggregation

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper adds AIoT service operation result aggregation principal.

1. Discussion

The TR 23.700-13 has concluded the aggregation of AIoT responses received from multiple AIoT devices can be performed by NG-RAN based on the aggregation assistance information received from the AIOTF. This paper adds AIoT service operation result aggregation principal as a high level feature in clause 5 of TS 23.369.

2. Proposal

It is proposed to capture the following changes into TS 23.369.

* * * * Start of Changes * * * *

5.X	AIoT Service Operation Result Aggregation

The NG-RAN may perform service operation result aggregation based on the assistance information received from the AIOTF. An AIoT service operation with a specific correlation ID may involve many AIoT devices, for the purpose of efficient reporting the service operation result the NG-RAN may aggregate the result before sending it to the AIOTF.

The AIOTF determines the aggregation assistance information based on operator policy and/or request from the AF, which includes:

-	Time interval: the fixed time interval for which NG-RAN collects AIoT device operation responses before reporting the aggregated AIoT responses to the AIOTF.

The reporting based on time interval may potentially happen multiple times until the NG-RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to the NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	If the AIOTF does not provide aggregation assistance information, the aggregation process in the NG-RAN is determined by implementation.

* * * * End of Changes * * * *