SA WG2 Meeting #168	S2-2503687

07 - 11 April, 2025, Goteborg, Sweden	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.369: Clarification on AIoT Device Profile Data

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper adds AIoT device temporary ID in the profile data for AIoT devices.

1. Discussion

Regarding the following EN related to AIoT device ID security mechanism, SA3 has concluded to use temporary device ID to protect the permanent device ID. So the EN can be removed, and the profile data for AIoT device includes the temporary device ID which is stored in the ADM. The temporary device ID can be retrieved by the AIOTF to use in the AIoT service operations.

Editor's note:	Whether ADM support ID security mechanism pending on SA WG3 decision.

2. Proposal

It is proposed to capture the following changes into TS 23.369.

* * * * Start of Changes * * * *

5.5	AIoT Device Profile Management

The ADM may hold operator’s profile data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the corresponding profile data (e.g. Device ID or credentials) is stored external to the network.

The AIoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in TS 23.502 [4] clause 5.2.3, it is stored in the ADM (AIoT Data Management) network entity that exclusively supports management of AIoT Device’s profile data. The AIoT Device Permanent ID is the primary key for AIoT device profile data in the ADM.

The AIoT Device temporary ID is allocated by the AIoTF and stored in the ADM as a part of device profile data. The temporary device ID is retrieved by the AIOTF to use in the AIoT service operations.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Uniquely identifies the AIoT Device. |
| AIoT Device Temporary ID | Identify the AIoT Device temporarily during in the AIoT service operations |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |

Editor’s note:	SA WG2 will align security related materials with SA WG3 decision later.

* * * * End of Changes * * * *