SA WG2 Meeting #168	S2-2503692

07 - 11 April, 2025, Goteborg, Sweden	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.369: Address EN for AIoT Reader Selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper addresses the ENs regarding the RAN reader and the AMF selection.

1. Discussion

The paper addresses the following ENs for AIoT Reader selection in clause 5.3:

Editor’s note:	Whether and how the AIOTF chooses the RAN reader(s) is FFS.

Editor’s note:	How AIOTF selects the AMF and the AIoT RAN in the indirect path is FFS.

Some description regarding the selection of RAN readers and the AMF based on the serving area information and the target area in the AIoT service request is added. The ENs can be removed.

2. Proposal

It is proposed to capture the following changes into TS 23.369.

* * * * Start of Changes * * * *

5.3	AIoT Reader Selection

The AIOTF selects AIoT RAN node(s) and, optionally, a list of the RAN readers. The AIOTF may be configured with AIoT RAN node and optional its serving RAN readers and their serving area information by OAM. When receiving the AIoT service request from the NEF and the request includes the target area information, the AIOTF selects the AIoT RAN node(s) based on the target area information and the AIoT RAN node’s serving area information. Additionally, if the serving area of one or multiple RAN readers matches the target area in the AIoT service request, the AIOTF selects the RAN readers and sends the service request to the target AIoT RAN with the selected RAN reader information.

When the AMF supporting AIoT capability registers to the NRF, it will include the AIoT capability indication and its serving area for AIoT service. The AIOTF selects the AMF based on the AMF’s AIoT capability and the target area information and the AMF’s serving area for AIoT service. The AIOTF sends the AIoT service request to the selected AMF and the AMF transfers the service request to the target AIoT RAN.

The AF may provide the expected target area information in the AIoT service request. The NEF may convert the target area information received from the AF to internal area information. The NEF queries the NRF to obtain serving AIOTF(s) based on internal area information. Then, the NEF forwards the AIoT service request, including the internal area information, to the AIOTF.

Editor’s note:	How the NEF converts target area information to internal area information and the format of the internal area information is FFS.

Editor’s note:	It is FFS how to handle the case that the AF provides other information than target area information, for the purpose of reader selection. In such case how the NEF and/or AIOTF use it to select the RAN reader.

The AIOTF may have AIoT RAN information and the optional RAN reader information. For direct path, the AIOTF obtains the information from AIoT RAN by the NGAP or OAM configuration.

Editor’s note:	AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of AIoT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

NOTE:	RAN reader information is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) to determine targeted RAN reader(s) directly.

* * * * End of Changes * * * *