3GPP TSG-WG SA2 Meeting #168	S2-2503696

Goteborg, SE, 7 – 11 Apr, 2025	Revision of S2-250XXXX

Source:	China Mobile

Title:	Update for AIoT Service Procedures

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT/ Rel-19

Abstract: This paper update the AIoT service procedures.

1. Introduction/Discussion

This contribution is proposed to update the Inventory procedure in 6.2.2 in TS 23.369.

2. Text Proposal

It is proposed to approve the following changes.

FIRST CHANGE

6.2	AIoT Service Procedures

6.2.1	General

Clause 6.2.2 provides the procedure for AIoT Inventory. Clause 6.2.3 provides the procedure for AIoT Command.

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires additional details, alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices])  service operation request to the NEF.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF performs AF authentication to determine whether it has the permission to execute the inventory operation. If the authentication is successful, the NEF selects the AIOTF based on AF ID or target area information by NRF discovery or local configuration to handle the request.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF performs AF authentication to determine whether it has the permission to execute the inventory operation in the target area or for target device(s). If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the AIoT Device Identification information to be included in the paging message sent by AIoT RAN.

AIOTF performs Reader Selection, see clause 5.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message, RAN Reader ID list or RAN service area ID list and assistance information to the selected AIoT RAN.

NOTE: The AIOTF groups the Reader IDs and AIoT RAN service areas according to the AIoT RAN Node ID, where Reader IDs and AIoT RAN service areas within the same group correspond to the same AIoT RAN Node ID.

Editor's note:	AIoT Device Identification information needs further definition.

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The AIoT RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly. If any of the selected readers is down, the AIoT RAN rejects the service operation and the reject result should be informed to the AF.

9.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation.

If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	AIoT RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s).

Editor's note:	Whether only one, or more than one report is generated by the AIOTF to send to the AF needs clarification.

13.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s).

END OF CHANGE