SA WG2 Meeting #168	   S2-2503763

7-11 April, 2025, Goteborg, Sweden

Title:	[draft] Reply LS on multiplexing capability negotiation

Response to:	LS on multiplexing capability negotiation (S2-2502807/C1-250885)

Release:	Rel-19

Work Item:	NG_RTC_Ph2

Source:	China Mobile (will be SA2)

To:	CT1

Cc:

Contact person:	Yi Jiang

jiangyi at Chinamobile dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA2 thanks CT1 for the LS on multiplexing capability negotiation.

SA2 discussed the procedures of capability negotiation for data channel multiplexing and provides the answers to the questions in the incoming LS as follows:

On the requirement of the P-CSCF:

Q1: During the capability negotiation between UE and IMS network, what is the consideration to require the P-CSCF to indicate its capability to the home IMS network of the UE?

SA2 answer: The capability indication of P-CSCF is used by S-CSCF to determine that the P-CSCF will not reject the session with SDP including multiplexed DC media or remove multiplexed DC media due to not allowed or not supporting DC multiplexing.

Q2A: what the S-CSCF will indicate in the Feature-Caps header field included in 200 OK response to REGISTER request?

SA2 answer: The S-CSCF will not include the Feature-Caps header field indicating support of DC multiplexing in 200 OK response if the P-CSCF does not indicate its capability.

Q2B: what the P-CSCF will do if the Feature-Caps header field included in 200 OK response to REGISTER request indicates support of data channel multiplexing and the P-CSCF and IMS-AGW do not support data channel multiplexing?

SA2 answer: According to the answers above, the scenario in this question will not happen. If it happens, it is an error case which should be in remit of CT.

On the requirement of the S-CSCF:

Q3: What is the consideration to require the S-CSCF to indicate its capability to the remote IMS network？

SA2 answer: Requiring the S-CSCF indicating its capability is to allow the terminating network knows the multiplexing capability of remote network even if the original SDP offer does not contain multiplexed SDP so that it can determine whether it multiplexes the data channels between originating and terminating IMS networks if the terminating network add data channels to existing IMS session. Without this capability indication, DC multiplexing can only be used when the IMS session originally includes multiplexed SDP offer. If the

Q4: If the P-CSCF and IMS-AGW do not support data channel multiplexing and the MF in the home IMS network of the UE supports data channel multiplexing what the S-CSCF will indicate in the Feature-Caps header field of the INVITE request to the remote network?

SA2 answer: DC multiplexing is peer-to-peer instead of end-to-end. The S-CSCF may include the Feature-Caps header field indicating it’s support of DC multiplexing in the INVITE request even if the P-CSCF and IMS-AGW does not support data channel multiplexing because multiplexing data channels between IMS networks does not require support of P-CSCF and IMS-AGW.

Q5: Can the multiplexed data channel media descriptions within an SDP offer included in the SIP request messages implicitly point out this capability of the IMS network?

SA2 answer: Yes.

2	Actions

To SA2

ACTION: 	SA2 kindly asks CT1 to taken the answers above into account.

3	Dates of next TSG SA WG 2 meetings

SA2#169                 19th  – 23th May 2025                     Japan

SA2#170                 25th  – 29th August 2025                 Sweden