SA WG2 Meeting #168	S2-2503796

07 - 11 April, 2025, Goteborg

Source:	Samsung

Title:	WT#2 use case: Enhancement of analytics for user plane traffic pattern and characteristic

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes a use case to cover Work Task#2 of the FS_AIML_CN_Ph2 SID.

# 1	Discussion

This paper discusses a use case for FS_AIML_CN_Ph2 (SP-250413) Work Task #2 (WT#2), as described below.

Whether and how to enhance the 5GC analytics to support efficient performance user plane. Note 4: This may be based on the analysis of the user plane traffic (e.g. traffic patterns, packet information) and the reaction of the network per the analysis is based on the existing policy framework. Note 5: The study will not address how UPF detects wrong or malformed or duplicated packets. This WT shall have neither RAN or UE impacts. Solutions shall minimize the load on Control Plane NFs due to too much reporting from UPF.

A related use case for the WT#2 is provided in accordance with the SA2 guidance (SP-250418).

# 2	Proposal

It is proposed to include the following changes in TR 23.700-04 V.0.0.

* * * * Start of Changes – All New Text * * * *

### 5.X.1	Use Case #X: Enhancement of analytics for user plane traffic pattern and characteristic

In Rel-19, UPF functionalities such as N6 delay measurement and its use on UP path configuration such as UPF (re)selection are introduced. Such new UPF functionalities and behviour can be utilized for, e.g., enhancement of the analytics for user plane traffic pattern and characteristic. For example, if the NWDAF collects the N6 delay measurement data and generate its related analytics, then it can be utilized by SMF/PCF for session management and also for unexpected user plane performance degradation such as sudden increase of N6 delay. Such analytics can be generated in finer granularity, e.g., per DNAI and application end point pair, per N6 delay measurement protocol/DNAI/application end point, etc.

* * * * End of Changes * * * *