3GPP TSG-SA2 Meeting #168	S2-2503956

# Goteborg, Sweden, 07 – 11 April 2025

Source: 	vivo

Title: 	KI#1: New Solution – UE data collection via DCCF in CN over UP

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#1 to support UE data collection via CN over UP.

1	Discussion

This paper proposes a solution to support UE data collection over UP according to requirements for AI/ML for NR air interface operation with UE-side model training.

Since R17 on, the DCCF has been introduced to CN as an intermediated agent to collect data for AI related usages, wherein the data consumer can be NWDAF, the data source can be CN NFs, AF and OAM. The data collection related signalling is interacted between data consumer and data source(s) via DCCF, and the collected data is reported from the data source(s) to the data consumer via DCCF or MFAF, the detailed procedures are defined as TS 23.288, clause 5A.3 and 6.2.6.3.

In this release, the DCCF can also be used to collect data from UE according to data consumer’s request, wherein the data consumer could be entity in CN or OTT server to train UE-side ML model. The DCCF receives data collection request from the data consumer indicating the target of reporting is some specific UE, group of UE or any UE, then the DCCF sends the data collection request to the target UE via AMF over NAS, meanwhile, UP information for data reporting can also be sent to indicate the UE to utilize user plane for data reporting. The UE selects or established PDU session for this data reporting, and then reports the collected data via the UP connection to the DCCF.

2. Proposal

It is proposed to add the following contents to TR 23.700-04.

Start of Change

Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #x> | <Key Issue #y> |
| #x | x |  |

Next Change (all new text)

6.X	Solution #X: UE data collection via DCCF in CN over UP

6.X.1	Description

This solution is for Key Issue#1 for UE data collection over UP.

Since R17 on, the DCCF has been introduced to CN as an intermediate entity to collect data for AI related usages, wherein the data consumer can be NWDAF, the data source can be CN NFs, AF and OAM. The data collection related signalling is interacted between data consumer and data source(s) via DCCF, and the collected data is reported from the data source(s) to the data consumer via DCCF or MFAF, the detailed procedures are defined as TS 23.288, clause 5A.3 and 6.2.6.3.

In this release, the DCCF can also be used to collect data from UE according to data consumer’s request, wherein the data consumer could be OTT server to train UE-side ML model. The DCCF receives data collection request from the data consumer indicating the target of reporting is some specific UE, group of UE or any UE, then the DCCF sends the data collection request to the target UE via AMF over NAS, UP information for data reporting can also be sent to indicate the UE to utilize user plane for data reporting. The UE selects or established PDU session for this data reporting, and then reports the collected data via the UP connection to the DCCF.

6.X.2	Procedures

6.x.2.1 UE data collection via DCCF in CN over UP

Figure 6.x.2.1-1: UE side data transferring via a User Plane Connection

Editor’s Note: how to support MNO’s controllability and visibility in solution is FFS

- Data consumer (e.g. Model training OTT server) requests UE data collection to DCCF by invoking Ndccf_DataManagement_Subscribe, including Target of reporting (i.e. UE(s), group of UE or any UE) and Notification Target Address, and optionally with Time Window, AOI, etc.

If the OTT server is an untrusted AF, the data collection request should be transferred to the DCCF via NEF.

Editor’s Note: how to discover the DCCF by the data consumer is FFS.

- The DCCF may check the user consent by retrieving the user consent information from UDM using Nudm_SDM_Get. If user consent is not granted, DCCF does not collect data from the UE, the data collection stops here. If the user consent is granted, the DCCF subscribes to UDM to notifications of changes on subscription data type "User consent" for this user using Nudm_SDM_Subscribe.

- The DCCF determines whether the data requested in step 1 are already being collected, if yes, step 4 is performed. Otherwise, step 5 to 11 are performed.

- [Conditional] The DCCF uses Ndccf_DataManagement_Notify to send the UE related data to notification endpoint indicated in step 1.

- [Conditional] The DCCF sends data collection request to the corresponding AMF(s) by invoking Namf_Communication_N1N2Transfer service operation, with requested data information and UP information for data reporting. The UP information for data reporting indicates the UE to utilize user plane for data reporting. TheUP information includes the target address to report data.

Note 1:  The data collection request and UP information for data reporting can be transferred to the UE in two separated steps.

Editor’s Note: whether data collection request and UP information can also be transferred to the UE via UP is FFS.

Editor’s Note: Whether UP information includes other content is FFS.

- [Conditional] For idle UE(s) the AMF triggers service request procedure.

- The AMF forwards the data collection request and UP information for data reporting to the UE(s) via DL NAS message.

- The UE may use the URSP from PCF to select the PDU session, over which the UE will establish a UP connection with DCCF for data reporting. If there is no applicable PDU session for this UE data reporting over the user plane, then the UE establishes the PDU session.

Editor’s Note: When and how the PDU session is established is FFS, e.g. whether the PDU Session is established from the beginning or just after Step 7.

- According to data collection request from DCCF, the UE collects the requested data.

- The UE reports the collected data via the UP connection over PDU session to the DCCF.

- The DCCF uses Ndccf_DataManagement_Notify to send the UE related data to notification endpoint indicated in step 1.

Editor’s Note: whether and where to store the data collected from UE by the DCCF is FFS.

Editor’s Note: whether the DCCF or a different NF with similar functionality will be used is FFS.

6.X.3	Impacts on services, entities and interfaces

DCCF:

-	Sends data collection request to UE via AMF over NAS to collect UE data;

-	Allocates and sends UP information for data reporting to the UE via AMF over NAS.

-  	Receives data collected by the UE via UP connection from the UE;

.

UE:

-	Receives UP information for data reporting originated from DCCF, and establishes UP connection with the DCCF to report data;

-	Receives data collection request originated from DCCF, and collects corresponding data;

- 	Reports the collected data via the UP connection to the DCCF.

End of Changes