3GPP SA2 Meeting #168	S2-2504242

Goteborg, Sweden , April 7 - 11, 2025

Source:	Lenovo

Title:	Solution for WT 1 – General Framework for data collection from the UE

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: .

1. Discussion

LSs received from RAN provide the following information:

-	In RP-242389 provide a summary of the requirements for collecting data from the UE and options to transfer data from the UE to the UE-side server where data are used to train UE-side ML models.

-	In RP-243316, RAN indicating a preference for SA2 to prioritize solutions for Option 2 where data is transferred by the UE to a termination entity inside the core network via a user plane "tunnel" and provide feedback on fulfillment of visibility and controllability requirements. It is important to note that RAN has concluded that transfer of data over CP (for Option 2) meets the visibility and controlability requirements.

In RP-24239 the options discussed in RAN are shown in the table below:

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

RAN also indicates in RP-242389 the requirements to support data collection for UE-side ML models as follows:

1.	The data collected is secured and data integrity and confidentiality for that data is ensured.

2.	User data privacy, anonymity and user consent is respected.

3.	The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer.

4.	MNO has full visibility for standardized data.

5.	The design is futureproof and extendable.

The solution proposed covers the following aspects:

-	Procedure to configure the UE for reporting standardized data for UE-side ML model training

-	Define the entity within the core network that collects data from the UE

-	Procedure for the UE to transfer data to the entity within the core network via user plane

The solution proposed is divided into a:

-	General framework for supporting triggering and configuring transfer of standardized data from the UE via user plane

-	Option where the UE is configured to report standardized data via the control plane and transfer of data via the user plane

-	Option where the UE is both configured to report standardized data and transfer the collected data via the user plane.

2. Proposal

The proposal is captured in TR 23.700-04 as follows:

************************************* FIRST CHANGE **************************************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #X | X |  |

************************************* SECOND CHANGE (all new text)****************************

6.x	Solution X: General framework for supporting transfer of standardised data from the UE via user plane

6.x.1	High Level Principles

The high level principles of the solution are:

- A UE-side server sends a request to receive data from a UE or Group of UEs. The standardised data to report are identified by a specific identifier

- A Data Collection Function receives the UE-side server request and determines the standardised data to retrieve from one or more UEs according to the request from the UE-side server.

- Data Collection Functions prepares configuration information for the UEs where the configuration information indicates to the UE; 1) the standardised data to transfer and 2) validity conditions on when to transfer the data.

- The Data Collection Function sends the configuration information to the UE via an already established session with the Data Collection Function.

-	The UE establishes a UP session with a Data Collection Function based on existing URSP rules. No changes to URSP rules are required.

6.x.2	Description

Triggering and configuring transfer of standardised data from the UE

A data collection function in the network is responsible for triggering a UE to report data and collect data from the UE..or

The architecture is shown in Figure 6.x.2

Figure 6.x.2-1: Triggering and configuring data collection from a UE

The main steps of the data collection procedures are as follows:

-	The data collection function receives from a consumer a request for standardised data from a UE or a group of UEs. The request includes an identifier to indicate a (list of) standardised data which will be needed to be transferred by the UE to the UE-side server for training an UE-side ML model. The request may also include validity information of the requested data (i.e. an area of interest and/or a time window of the collected data by the UE).

Editor’s Note: Whether the DCF functionality is supported in a New NF or existing NF is FFS.

NOTE X:	The standardised data needed to be collected and reported by the UE will be defined by RAN1.

Editor’s Note: Whether the UE side server include in the request information on the data required to be collected by the UE is FFS. Coordination with RAN is needed.

-	The Data Collection Function ensures that the users of UEs have provided user consent for reporting data.

-	The Data Collection Function prepares a configuration for the UEs totransfer standardised data according to the identifier  received by the consumer. The request may include additional information to indicate to the UE how often to report data, size of the transfer of data etc.

The Data Collection Function sends the configuration via user plane).

-	The UE transfers the related standardised data to the Data Collection Function via user plane. In turn the Data Collection Function reports the data to the consumer.

NOTE Y: The trigger for data collection and how the UE collects standardised data is FFS.

6.x.3	Procedures

The architecture for this option in shown in Figure 6.x.3.-1.

Figure 6.x.3-1: Architecture for data collection configuration and reporting via user plane

In this option:

-	UE establishes a session with Data Collection Function which is used for data reporting. The session is via a PDU session established according to a received URSP rules where the URSP rules contains a traffic description the FQDN/IP address of the Data Collection Function and as RSD the DNN/S-NSSAI to use.

Editor's Note: The procedure for the UE to obtain the Data Collection Function address and the trigger for the UE to establish a session with the Data Collection Function is FFS.

-	Once the Data Collection Function receives a request to retrieve data the Data Collection Function constructs configuration information for data to transferred UE (according to the identifier included in the consumer request). The configuration information is included in a UE Data Container and sent to the UE via the established session.

Editor’s Note: Details of the session between the UE and Data Collection Function is FFS

-	The UE logs data according to the configuration information..

Detailed procedure in shown in Figure 6.x.3-2

Figure 6.x.3-2: Data Transfer configuration and reporting via user plane

0.	A UE establishes a session for reporting data with the Data Collection Function

Editor's Note: The procedure for the UE to establish a session is FFS.

1.	A consumer requires data from UE in order to train an AI/ML model used by the UE. The consumer may be an AF, a third party AI/ML training server.

2.	The consumer sends a data request to the Data Collection Function where in the request includes an identifier of the standardised data needed to be transferred by the UE, an area of interest (where data are to be collected) and target UEs.

3.	The Data Collection Function determines what standardised data are needed to be transferred by the UE taking into account user consent information (i.e. from UDM).

4.	The Data Collection Function prepares configuration information to retrieve the standardised data.

5.	The Data Collection Function sends the container in an message to the UE via the established session between the UE and the Data Collection Function.

6.	The data reporting client in the UE checks the configuration information within the UE data container and determines the data to transfer. The UE logs data in assistance with the RAN node. Details are specified in RAN.

7.	The UE includes the logged data in a container and reports via user plane to the Data Collection Function via the established session.

8.	The Data Collection Function reports the data to the consumer.

Editor’s Note: How to support MNO’s visibility in this solution is FFS.

6.X.4	Impacts on services, entities and interfaces

-	Data Collection Function:

-	Receives requests from consumer (UE-side server) to collect and transfer standardised data from the UE.

-	Determines configuration information for the UE to report standardised data.

-	Sends configuration information via an established session between the UE and the Data Collection Function

-	UE

-	Determines standardised data to transfer based on configuration information received from Data Collection Function

-	Establishes a session with the Data Collection Function based on configuration information

************************************** END OF CHANGES *************************************