SA WG2 Meeting #S2-168	S2-2504339

7 - 11 April, 2025, Goteborg, Sweden	(revision of S2-2504302/S2-2503995/S2-2503459)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6223 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the OAM input |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-03-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In this meeting, SA5 send the reply LS (S2-2502843) on energy information collection from OAM to EIF. SA5 clarifies that, “On which gNBs / UPFs EIF wants to collect the energy related information measurement or KPI value(s). EIF shall indicate the Distinguished Name (DN) of concerned gNBs / UPFs;” In the SA2 spec, it uses gNB ID, NF instance ID, e.g. UPF (instance) ID to discover the network entity. But in the SA5, it uses the Distinguished Name (DN) to discover the network entity. They are not simply different in name, their encoding is also different. In the 29.571, the NF instance ID (NfInstanceId) shall be a Universally Unique Identifier (UUID) version 4, e.g. "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" In the 38.413, the gNB ID is BIT STRING (SIZE(22..32)). The Distinguished Name (DN) is defined in the 32.200. It looks like: "DC=a1.operatorNN.com,SubNetwork=CountryNN,ManagedElement=RNC-Gbg-1" So in this stage, the way forward is the EIF is locally configured with mapping between gNB/UPF ID and Distinguished Name (DN). The NWDAF uses the similar mechnism for data collection from OAM (see Note 2 in clause 6.2.3.2 of 23.288) |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EIF is is locally configured with mapping between gNB/UPF ID and DN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not align with SA5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.51.2.2.3	Energy Consumption information collection from OAM

Table 5.51.2.2.3-1 provides the list of information received from OAM. EIF requests the energy consumption and data volume information per NF level from OAM by providing the serving gNB ID(s) and (I-)UPF ID(s) received from SMFs.

NOTE 1:	The management consumer in EIF is configured information to retrieve the energy related information from OAM as defined in 28.554 [209].

The node-level reporting interval from the OAM is the PLMN wide configurable starting time and interval T which is the same as data volume reporting interval from SMFs.

Table 5.51.2.2.2-1: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time interval T based on clause 6.7.3.4.2 of TS 28.554 [209]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time interval T based on clause 6.7.1.1 of TS 28.554 [209]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time interval T based on clause 6.7.3.1 of TS 28.554 [209]. |
| UPF data volume | Data volume consumed at a UPF. |

When the serving gNB and/or the (I-)UPF(s) of the UE are changed, the serving gNB ID and UPF ID will be updated to the EIF through SMF(s). And then, the EIF requests the energy consumption and data volume information from OAM for the updated serving gNB ID and (I-)UPF ID(s).

NOTE 2:	The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs serving by the NF Node.

NOTE 3:	In this Release, only the energy related information of user plane communication is supported and that of control plane signalling is not supported.

The EIF acts as the enforcement point for user consent for collection and processing of energy-related information depending on local regulations. For support of user consent (for collection and processing of energy-related information), the EIF communicates with the UDM as specified in Annex V of TS 33.501 [29].

END OF CHANGES