3GPP TSG-SA2 Meeting #168	S2-2502928

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1611 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of target satellite ID notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Clarify that the PCF is authorizing resources rather than explicitly establishing a user plane path, ensuring consistency with the resource management terminology in 5GS. - In clause 5.4.14 of TS23.501: “In the UE-SAT-UE communication, if the UE's serving satellite changes (e.g. due to UE movement or the satellite……The SMF decides whether to route IMS user plane traffic to the PSA on the ground or retain the UL CL/L-PSA on the satellite based on local configuration and other factors as specified in clause 6.3.3. If the SMF decides to route the IMS user plane traffic to the PSA on the ground ……the SMF provides an early notification without satellite ID via Nsmf_EventExposure_Notify to PCF. The PCF forwards this notification to the P-CSCF via Npcf_PolicyAuthorisation_Notify, indicating that the 5GC user plane needs to fall back to the ground.” Therefore, when the serving satellite of the UE changes, the P-CSCF may not receive the target satellite ID if the SMF decides to select ground fallback. The content in clause AE.3.3 of TS 23.228: “The P-CSCF is notified of the identifier of the target satellite from the PCF when the serving satellite of the UE changes according to clause AE.5.2.” needs to be updated to align with TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarified the capabilities of the PCF. - Updated the notification send to UE when the serving satellite changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The P-CSCF will always receive the target satellite ID when the accessed satellite changes, which is not feasible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.3.2, AE.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AE.3.2	At call setup

The P-CSCF receives from the PCF the satellite identifier of the satellite serving the UE and receives from the P-CSCF serving the remote UE the satellite identifier of the satellite serving the remote UE according to clause AE.5.1 and clause AE.5.3. Subject to operator policy, the P-CSCF uses this information to determine whether or not activate optimized media routing. With the lack of the satellite identifier of the satellite either serving the local UE or the remote UE, the P-CSCF determines not to activate optimized media routing.

NOTE:	In this Release of the specification, how the P-CSCF uses the satellite identifier to derive whether optimized media routing is possible is not specified.

If P-CSCF determines the activation of optimized media routing, the P-CSCF instructs the PCF to authorize the necessary resources according to clause AE.5.1 and clause AE.5.3, so that the PCF proceeds to establish a 5GS user plane resource for optimized media routing.

# * * * * Second change * * * *

AE.3.3	At change of satellite

The P-CSCF may be notified of the identifier of the target satellite from the PCF when the serving satellite of the UE changes according to clause AE.5.2.1 and clause AE.5.2.2. The P-CSCF determines whether optimized media routing continues to be possible between the target satellite and the satellite serving the remote UE based on the identifiers of those satellites. If there is no target satellite identifier in the notification, the P-CSCF determines that optimized media routing is no longer possible.

NOTE:	In this Release of the specification, how the P-CSCF uses the satellite identifier to derive whether optimized media routing can continue is not specified.

If the P-CSCF determines that optimized media routing can continue at change of satellite, the P-CSCF invokes Npcf_PolicyAuthorization_Update service operation to the PCF as defined in clause 4.3.6.3 of TS 23.502 [94] and according to clause AE.5.2.1 to establish a path through the target satellite for optimized media routing.

If the P-CSCF determines that optimized media routing is no longer possible e.g. due to a change of satellite, the P-CSCF executes the ground fallback procedure as per clause AE.5.2.2 to route the media via the ground.

# * * * * End of changes * * * *