# 3GPP TSG-SA WG2 Meeting #168		S2-2503172

# Goteborg, Sweden, April 07 – 11, 2025		(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0706 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| R |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | User plane positioning update for LCS-UPP context synchronization |  |  |  |  |  |  |  |  |  |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the discussion in SA2 #167, during/after the LMF relocation procedure, the UE needs to sync up with the target LMF on the mapping of on-going positioning sessions and LCS-UP connection. It is proposed to update this specification with the following changes: - In LMF relocation procedure, target LMF sends the source LMF’s user plane information along with target LMF’s user plane information for the UE to decide which connection to use for the relocated deffered location sessions. - During the context transfer procedure between source LMF and target LMF, the target LMF decides whether to establish LCS-UP connection. If the establishment of the LCS-UP connection is determined by the target LMF, the user plane (UP) positioning-related location contexts shall be transferred to the target LMF. If the target LMF determines not to establish the LCS-UP connection, the location contexts shall be retained in the source LMF. - After receiving Event report via the user plane connection from the UE with source LMF’s deferred routing identifier, the target LMF will send Event Report Acknowledgment messge to the UE via the user plane connection with indicating the change of LMF and a deferred routing identifier indicating target LMF. - Steps are re-ordered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update LCS-UPP related clauses to support LCS-UPP context synchronization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When LMF is relocated, the UE cannot select the proper LCS-UPP connection to send the reports with proper deferred routing identifier. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.18.3	Modification of User Plane Connection between UE and LMF

The figure 6.18.3-1 shows how a secure user plane connection between UE and LMF is modified. The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF not selecting any Target LMF.

Figure 6.18.3-1: Connection modification between UE and LMFs

1a.	[Conditional] The LMF discovers a need to change LMF or re-establish the user plane connection between UE and LMF or terminate the user plane connection, e.g. after receiving an event report via user plane from UE or after receiving an AMF relocation information from a target AMF (target AMF obtains from source AMF the UE LCS-UP context which indicates UE has maintained LCS-UP connection with the source LMF, the target AMF may inform the source LMF about the AMF change using the Nlmf_Locatoion_UPConfig request). The LMF sends an Nlmf_Location_UPNotify message that includes connection move (termination and establishment) or termination is required and if connection move is requested then message may include target LMF identification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UP Subscribe message. If the LMF is going to terminate the user plane connection, step 1a, step 3, step 6 and step 7 are performed and other steps are skipped.

If the request is to terminate the user plane connection, the AMF releases the LCS-UP context.

NOTE 1:	The LMF change procedure is independent from the SSC mode of the PDU Session with dedicated DNN used for positioning. For SSC mode 2/3, PSA UPF connection with the LMF can be relocated with UE movement and LMF can discover the need to change LMF to reduce the user plane path latency.

1b.	[Conditional] The AMF should perform LMF reselection if a target LMF identification received from source LMF for user plane positioning, or AMF may perform LMF reselection if a UE established a LCS UP connection with the source LMF moves to a new location (which may be out of serving area of source LMF and in serving area of target LMF) and select the target LMF for the current UE location based on LMF service area, LMF user plane positioning capability information and other information listed in clause 5.1. LMF needs to be capable to establish a user plane session for positioning with the UE. After AMF relocation, the target AMF may trigger the LMF reselection and LCS-UP connection modification procedure, e.g. if the source LMF does not perform LMF reselection.

2.	The AMF sends an Nlmf_Location_UPConfig Request towards the source LMF. The message may include a request for the Source LMF to terminate a specific user plane connection to the UE and the Target LMF identification. Alternatively, it may include information about AMF reallocation.

NOTE 2:	AMF relocation does not necessarily always cause LMF reselection, the AMF relocation information can keep the LMF informed.

3-4.[Conditional] The source LMF may invoke an Nlmf_Location_LocationContextTransfer Request service operation towards the target LMF to provide the current location context(s) of the UE, if there is user plane based periodic and triggered UE location events report context(s). The target LMF informs source LMF of whether LCS-UP connection will be established and the location context transfer operation results. If the establishment of the LCS-UP connection ​is determined by the target LMF, the user plane (UP) positioning-related location contexts ​shall be transferred to the target LMF. If the target LMF ​determines not to establish the LCS-UP connection, the location contexts ​shall be retained in the source LMF.

5.	[Conditional] After the location context transfer, step 2-8 of figure 6.18-1-1 are performed between AMF (or Target AMF), UE and Target LMF with addition that UE also terminate connection to Source LMF. The user plane information transmitted in step 2 of figure 6.18.1-1 also includes the user plane positioning address of the source LMF. Upon successful establishment of the user plane connection, the UE ​shall send messages associated with the LCS-UP connection of the source LMF ​via the LCS-UP connection established for the target LMF. Upon receiving the ​LCS Event Report (indicating the source LMF) from the UE via the established LCS-UP connection, the target LMF ​shall send an ​LCS Event Report Acknowledgment message to the UE via the user plane, indicating an LMF change notification and including a deferred routing identifier associated with the target LMF, after which the UE ​shall replace the original deferred routing identifier with the newly received identifier in subsequent LCS Event Reports..

6.	[Conditional] If the user plane connection to source LMF is still active, the source LMF terminates the connection to the UE.

7.	The LMF sends Nlmf_Location_UPConfig Response message to AMF to confirm connection termination or acknowledge change of AMF. If this procedure is used for termination, the AMF will release the LCS-UP context after receiving the response message.

# * * * * End of changes * * * *