3GPP SA WG2 Meeting #168	S2-2503231

Gothenburg, Sweden, 7-11 April 2025

# Title:	[Draft] Reply LS on OAM requirements to support regenerative payload transport links

# Response to:	Reply on OAM requirements to support regenerative payload transport links (R3-250895/S2-250abcd & S5-250842/S2-250abcd)

# Release:	Release 19

# Work Item:	NR_NTN_Ph3-Core

Source:	Qualcomm, Vodafone [to be SA2]

To:	RAN3, SA5

Cc:	-

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 reviewed the RAN3 answers in the LS response R3-250895 and would like to provide the following comments:

- Frequency of updates by OAM is not clarified and therefore the signalling optimisation is unproven: SA2 indicated in LS S2-2413030 that the frequency of updates by OAM was not clear. In RAN3’s answer it was indicated that that the frequency of updates is periodic and predictable but it was not defined what the expected frequency is. RAN3 referred to SA5 to decide the frequency. SA2 would like to note that changing TAIs and possibly cell-ids will require “migration” of the UEs that camped on the satellite and will have to be handed over to another satellite: this would incur in “UE associated signalling” several orders of magnitude larger than the “nodal signalling” for migration.

- Coordination across the signalling based and OAM solutions is not clear: In R3-250895 it is stated:  “…both AMF and gNB share the same OAM configuration on supported TAI list of the on-board gNB”. And later on: “Any potential discrepancy between OAM configuration and parameters received via NG signalling, if present, is expected to be resolved via implementation”. It remains unclear how the gNB will know whether the AMF is “correctly configured” via OAM with the TAI list in order to avoid sending the RAN Configuration Update procedure when it crosses the geographic boundary that would trigger change to new TAI. If mismatch occurs, e.g., the AMF is not configured or is “unsynchronised”, it will have impacts in the paging procedures. Therefore, at a minimum there has to be a “handshake” to agree on TAI timetable at time of NG setup.

- Violation of “plug n’ play” principles: since Rel. 8 in 3GPP it was agreed to follow "plug and play" principles which refer to the ability of a multi-vendor RAN node (eNB/gNB) to connect to the network and automatically configure itself. This concept is supported in TS 32.501 and TS 32.508. In the LS response from RAN3 it is mentioned that the AMF and gNB need to share the same OAM configuration for this concept to work, hence violating the above-mentioned principles.

- Change of functional split between RAN and CN: currently the CN need have no knowledge of the geographic area covered by the gNB. Coverage aspects are left to RAN and should not be added to the CN.

Considering the points above, SA2 believes that the functionality of TAI-gNB configuration in AMF via OAM does not provide any material signalling optimisation whereas at the same time it introduces significant AMF complexity, it violates the “plug n’ play” principles for RAN and CN interfaces, and it can lead to significant interoperability issues that RAN3 admits can only be resolved via implementation.

For these reasons SA2 believes that TAI configuration in AMF via OAM should not be pursued in Release 19 without proper study on the system impacts.

The above discussion and proposal also apply to the work on IoT NTN regenerative payload with the MME.

2. Actions:

To RAN3, SA5:

ACTION: 	SA2 kindly asks RAN3 and SA5 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #169		19-23 May 2025	Fukuoka, Japan

TSG-SA2 Meeting #170		25-29 August 2025	Goteborg, Sweden