3GPP TSG-WG SA2 Meeting #168	            S2-2503336

Gotenburg, Sweden, April 7-11, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6210 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transport level packet marking operation at I-SMF/I-UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | <related to LS in from CT4; S2-2502815/C4-250629> SMF determining the transport level packet marking values considering PDU Set Importance is introduced, but I-SMF/I-UPF operation looks like unintendedly defining another operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified I-SMF/I-UPF operation for transport level packet marking values considering PDU Set Importance is same as I-SMF/I-UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstood the transport level packet marking in I-SMF/I-UPF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

5.8.2.7	PDU Session and QoS Flow Policing

ARP is used for admission control (i.e. retention and pre-emption of the new QoS Flow). The value of ARP is not required to be provided to the UPF.

For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF.

For QoS Flows that are configured for PDU Set QoS handling, the SMF may additionally take into account the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a list of transport level packet marking values for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values. When an I-SMF/I-UPF is inserted in the PDU Session, the I-SMF may instruct the I-UPF via same transport level marking in FAR to keep the transport level packet marking value of the outgoing N3 downlink packet forthe incoming N9 downlink packet.

NOTE 1:	It is recommended that the PDU Set Importance based transport level packet marking values applied by the UPF only vary the drop precedence between PDUs in the transport network nodes (e.g. IP routers) on the N3/N9 interfaces. This is needed to avoid causing packet reordering in the transport network when different transport level DSCP marking values are used for a single QoS Flow.

NOTE 2:	The transport level packet marking values are provided on per-QoS Flow basis and it is up to operator deployments to enforce consistency of transport level marking in the transport network.

The SMF shall provide the Session-AMBR values of the PDU Session to the UPF so that the UPF can enforce the Session-AMBR of the PDU Session across all Non-GBR QoS Flows of the PDU Session.

SMF shall provide the GFBR and MFBR value for each GBR QoS Flow of the PDU Session to the UPF. SMF may also provide the Averaging window to the UPF, if Averaging window is not configured at the UPF or if it is different from the default value configured at the UPF.

In the case of 3GPP access, the SMF may decide to activate ECN marking for L4S by PSA UPF for the QoS Flow (see clause 5.37). In this case, the SMF shall send an ECN marking for L4S indicator to PSA UPF.

********** End of Changes **********