SA WG2 Meeting #168	S2-2504211

07 - 11 April, 2025, Goteborg, Sweden                  	(revision of S2-2503987)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1634 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IMS Service Definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper is proposed to modify IMS service definition. In AA.2.4.3.2, Avatar Media Specification is also optional. In AA.2.4.4.2 and AA.2.4.4.3, since the MDC1 media endpoint address is created by DCSF and not by DC AS; it should be removed from the input parameters for BDC creation and update. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification of IMS service definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some incorrect description exists in IMS service definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.3.2, AA.2.4.4.2, AA.2.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AA.2.4.3.2	Nimsas_MediaControl_MediaInstruction service operation

Service operation name: Nimsas_MediaControl_MediaInstruction

Description: This operation provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event and that may require resource reservation in media resource e.g. MF.

For the case when a specific media flow needs to be terminated in MF media resource (i.e. termination of a Data Channel media descriptor offered by one of the IMS subscribers) or originated by the MF (i.e. origination of a data channel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete MF media specification including information required by MF to know how to terminate or originate the media flow.

Inputs, Required: Session ID, Media instruction set

Session ID specifies the IMS session for which the MediaInstruction operation applies.

Media instruction set includes a set of instructions for each media flow to control. Each instruction includes:

-	media ID: used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media or, if this media is originally a data channel multiplexing media and is instructed to be de-multiplexed, one of the de-multiplexed media component. This field shall be null for instructions related to originating new media or for data channel media components de-multiplexed from a data channel multiplexing media which do not reuse the media ID of that data channel multiplexing media.

NOTE 1:	When a data channel multiplexing media is de-multiplexed to multiple data channel media components, which data channel media component reuses the original media ID of the data channel multiplexing media is decided by the consumer by implementation.

-	associated Media ID: This parameter only exists when a multiplexed media is de-multiplexed to multiple media and is used to associate different media streams which are de-multiplexed from a multiplexed media component. This parameter is set to the value of the media ID of the original multiplexed media.

-	Media resource capability: Identify the Media Resource capabilities the Media instruction is intended for (e.g. DC, AR).

-	Media instruction: includes instructions to the producer (IMS AS) for handling the media. The following instructions are supported:

-	"TerminateMedia": Terminate the offered media descriptor of the UE in the mediaResource, i.e. this media descriptor will not be exposed to the other UE.

-	"OriginateMedia": Originate and offer a media descriptor from the mediaResource to the UE. The media ID representing the new media flow will be provided by the IMS AS in the response.

-	"TerminateAndOriginateMedia": Terminate the offered media flow in the mediaResource from the UE and originate the same media flow from the mediaResource to the other UE, i.e. the offered media descriptor of the originating UE will be replaced by the mediaResource provided media descriptor, which is sent towards the other UE.

-	"UpdateMedia": Update a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia"

-	"DeleteMedia": Delete a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia".

-	"RejectMedia": Remove an offered media flow, i.e. the offered media descriptor will be removed both from the offer sent to the remote UE and from the answer returned to the initiator of the offer.

-	DC Media Specification(s): Description of additional media specification information needed for data channel media stream from application layer. If multiple application data channels are multiplexed in the same m line in the SDP, multiple DC media specifications for all multiplexed data channels are included. Each media specification includes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	MDC1/MDC2 media endpoint address of the application layer.

-	Replacement HTTP URL per stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	Data Channel Port.

-	Data Channel SCTP Port.

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies how the media stream should be processed.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Resource URL: URL that allocated by DC AS for MF to retrieve the avatar representation.

-	Rendering mode: identifies the mode of rendering avatar media, i.e. UE centric, network centric with MF rendering, network centric with DC AS rendering.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE 2:	DC Media Specification, AR Media Specification and Avatar Media Specification are optional depending on the scenario and media type.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: operation result indication.

Outputs, Optional: media resource information set:

This set includes entries corresponding to each instruction in the received media instruction set. Each entry in the set includes the media ID (same as received or new for new or de-multiplexed media) and includes the media type according to SDP. Each entry may optionally contain Avatar Media Specification, DC Media Specification and/or AR Media Specification. The DC Media Specification shall exist only with media type "DC" and contains additional media resource information, e.g. allocated media address for MDC2 interface when an offer must be provided to Data Channel Application Server. The AR Media Specification contains media processing instruction for MF on how to process the media. The Avatar Media Specification contains media processing instruction for MF on how to perform network rendering for avatar communication.

NEXT CHANGE

AA.2.4.4.2	Nimsas_ImsSessionManagement_Create service operation

Service operation name: Nimsas_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with the requested media (e.g. standalone data channel media).

NOTE 1:	Only data channel media is supported in this Release.

Inputs, Required: Calling ID, Called ID, Media Information set.

Calling ID: It is the public identity of calling IMS Subscriber or the identity of the consumer NF, e.g. DC AS.

Called ID: It is the public identity of called IMS Subscriber.

Media Information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created. It is allocated by the consumer NF.

-	Media Type: It indicates the media type used in the created session, e.g. Data Channel.

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	Application Binding Information; and

-	Target ID(s): the public identity(ies) of the IMS Subscriber the DC is targeted;

NOTE 2:	For P2P ADC, the Calling ID and Called ID of the session are used as the Target IDs.

-	If P2A or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1.

Outputs, Required: Result indication.

Outputs, Optional:  Session ID allocated by IMS AS.

NEXT CHANGE

AA.2.4.4.3	Nimsas_ImsSessionManagement_Update service operation

Service operation name: Nimsas_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the type of operated media (e.g. Data Channel).

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	Application Binding Information; and

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	If P2A or P2A2P is selected, if the Operation Type indicates adding or updating the media, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted

.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1.

Outputs, Required: Result indication.

Outputs, Optional: None.

END OF CHANGES