SA WG2 Meeting #169	S2-2505513

19 - 23 May 2025, Fukuoka, Japan                                               (Revision of S2-2505022)

Source:	Tencent, Tencent Cloud, vivo

Title:	Solution for KI#1: Data collections for UE-side model training via user plane

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes a way for data collections between UE and core network.

# 1. Discussion

3GPP SA2 Release 20 has approved the study item "AI/ML Core Network Enhancements Phase 2", which includes the research task of "whether and how to support the standardized transfer of standardized data over user plane for UE data collection." To enable the establishment of user plane connections from the UE to an NF, this paper proposes a way for data collections between UE and core network.

One key issue is proposed in TR 23.700-04 to provide solutions for UE data collection to meet the requirements for AI/ML for NR air interface with UE-side model training. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server. This key issue will investigate the following aspects:

-	How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection.

-	How to support in 5GC the full visibility of standardized UE data contents.

-	How 5GC can support the full controllability of standardized UE data transfer.

-	How to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC.

-	Whether and how the UE knows about what the standardized data to be transferred are.

Per online and offline discussions during SA2#169, it was suggested to merge S2-2505022 with solution#1 which is captured in TR 23.700-04.  Therefore, the pCR is revised accordingly i.e. by adding delta on top of solution#1 in TR 23.700-04.  The major changes are as follows:

- add the high level pricinples of the solution

- replace OTT server with UE model training entity server in step 1 and also figure 6.1.3.1-1.

- in step2  add an editor’s note about how DCCF select the target UEs, more details can be added further

- in step 6 add editor’s note about idle mode UE

- clarify the procedure in step 10

- Updated the impacts part

# 2. Proposal

This paper proposes to update the solution #1 for Key Issue #1 to TR 23.700-04 as follows.

---------- Start of change ----------

6.1	Solution #1: UE data collection via DCCF in CN over UP

6.1.1	High level principles

In this solution, there are some high level principles as follows:

- DCCF is utilized to support data collection from UE to data consumer e.g. UE model training entity server.

- When DCCF receives the request from data consumer, it performs UE selection, generates data collection request and provides UP information and sends to UE via AMF with NAS message.

- Based on the UP information, UE may establish UP connection for data collection via a PDU session, which is selected or established based on URSP.

- The UE transfer the data for AI training via UP connection to DCCF and DCCF transfer data to UE model training entity server.

- During data collection and transfer process, UE is aware of what the standardized data to be transferred to participate AI training.

6.1.2	Description

This solution is for Key Issue#1 for UE data collection over UP.

Since Rel-17 onwards, the DCCF has been introduced to CN as an intermediate entity to collect data for AI related usages, wherein the data consumer can be NWDAF, the data source can be CN NFs, AF and OAM. The data collection related signalling is interacted between data consumer and data source(s) via DCCF, and the collected data is reported from the data source(s) to the data consumer via DCCF or MFAF, the detailed procedures are defined in clauses 5A.3 and 6.2.6.3 of TS 23.288 [5].

In this Release, the DCCF can also be used to collect data from UE according to data consumer's request, wherein the data consumer could be UE model training entity server to train UE-side ML model. The DCCF receives data collection request from the data consumer indicating the target of reporting is some specific UE, group of UE or any UE, then the DCCF sends the data collection request to the target UE via AMF over NAS, UP information for data reporting can also be sent to indicate the UE to utilize user plane for data reporting. The UE selects or established PDU session for this data reporting, and then reports the collected data via the UP connection to the DCCF. The UP information includes the target address to report data.

Editor's note:	Whether UP information includes other content is FFS.

6.1.3	Procedures

6.1.3.1	UE data collection via DCCF in CN over UP

Figure 6.1.3.1-1: UE side data transferring via a User Plane Connection

Editor's note:	How to support MNO's controllability and visibility in solution is FFS.

1.	Data consumer (e.g. UE model training entity server) requests UE data collection to DCCF by invoking Ndccf_DataManagement_Subscribe, including Target of reporting (i.e. UE(s), group of UE or any UE) and Notification Target Address, and optionally with Time Window, AOI, etc.

If the UE model training entity server is an untrusted AF, the data collection request should be transferred to the DCCF via NEF.

Editor's note:	how to discover the DCCF by the data consumer is FFS.

- The DCCF check the user consent by retrieving the user consent information from UDM using Nudm_SDM_Get. If user consent is not granted, DCCF does not collect data from the UE, the data collection stops here. If the user consent is granted, the DCCF subscribes to UDM to notifications of changes on subscription data type "User consent" for this user using Nudm_SDM_Subscribe.

Editor's note:	how to select the target UEs for data collection by DCCF is FFS.

3.	The DCCF determines whether the data requested in step 1 are already being collected, if yes, step 4 is performed. Otherwise, step 5 to 11 are performed.

4.	[Conditional] The DCCF uses Ndccf_DataManagement_Notify to send the UE related data to notification endpoint indicated in step 1.

5.	[Conditional] The DCCF sends data collection request to the corresponding AMF(s) by invoking Namf_Communication_N1N2Transfer service operation, with requested data information and UP information for data reporting. The UP information for data reporting indicates the UE to utilize user plane for data reporting. The UP information includes the target address to report data.

NOTE:	The data collection request and UP information for data reporting can be transferred to the UE in two separated steps.

Editor's note:	Whether data collection request and UP information can also be transferred to the UE via UP is FFS.

- [Conditional] For idle UE(s) the AMF triggers service request procedure.

Editor's note:	Whether data collection from UEs in idle is required would need to be checked with RAN.

7.	The AMF forwards the data collection request and UP information for data reporting to the UE(s) via DL NAS message.

8.	The UE may use the URSP from PCF to select the PDU session, over which the UE will establish a UP connection with DCCF for data reporting. If there is no applicable PDU session for this UE data reporting over the user plane, then the UE establishes the PDU session.

Editor's note:	When and how the PDU session is established is FFS, e.g. whether the PDU Session is established from the beginning or just after step 7.

9.	According to data collection request from DCCF, the UE collects the requested data.

10.	The UE reports the collected data via the UP connection over PDU session to the DCCF. The UE may start data transfer when certain condition is met e.g. the related measurement has been accomplished and the requested data is available.

11.	The DCCF uses Ndccf_DataManagement_Notify to send the UE related data to notification endpoint indicated in step 1.

Editor's note:	Whether and where to store the data collected from UE by the DCCF is FFS.

6.1.4	Impacts on services, entities and interfaces

DCCF:

-	Sends data collection request to selected UEs via AMF over NAS to collect UE side model training data.

-	Allocates and sends UP information for data reporting to the UE via AMF over NAS.

-	Receives data collected by the UE via UP connection from the UE and transfers collected data to data consumer

UE:

-	Receives UP information for data reporting originated from DCCF, and establishes UP connection with the DCCF to report data.

-	Receives data collection request originated from DCCF, and collects corresponding data.

-	Reports the collected data via the UP connection to the DCCF.

AMF

-	The AMF transfer the UP information received from DCCF to UE via NAS connection.

---------- End of changes ----------