3GPP TSG-SA WG2 Meeting #169	S2-2505527

Fukuoka, Japan, May 19 – 23, 2025													(was S2-2503532, revision of S2-2505198+S2-2505074)

Source:	Xiaomi, vivo

Title:	KI#1: New Sol: CP based solution for supporting IMS voice over NB-IoT via GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#1.

1. Introduction/Discussion

A solution is proposed to address Key Issue#1 to support IMS voice call over NB-IoT via GEO connecting to EPC using Control Plane CIoT EPS Optimisation.

As specified in TS 23.401, Dedicated bearer, and GBR bearers are not supported in the NB-IoT RAT.

The solution proposes to remove those restrictions mentioned above to achieve IMS voice service over GEO via NB-IoT access connecting to EPC.

2. Proposal

It is proposed to capture the following text in FS_5GSAT_Ph4_ARC TR 23.700-19.

* * * Start of Change (all new text) * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| x | x |  |  |  |

6.X	Solution #x: Support of dedicated bearer for QoS differentiation for CP based IMS voice service over NB-IoT (GEO)

6.x.1	High level principles

The following principles are applied to this solution:

-	Proceduce on Data Transport in Control Plane CIoT EPS Optimisation is used as baseline.

-	EPS enhancement to support dedicated bearer for the IMS voice service over NB-IoT(GEO).

-	IMS signalling is transmitted via the default bearer, IMS voice data is transmitted via the dedicated bearer.

-	Dedicated bearer can be pre-established during IMS PDN connection establishment to reduce delay for the IMS voice service over NB-IoT(GEO).

6.x.2	Description

This solution addresses KI#1 to support IMS voice call over NB-IoT via GEO connecting to EPC

The TS 23.401 specifies dedicated bearer is not supportedover NB-IoT. GBR bearer is not supported in the NB-IoT RAT. These restrictions may not enable NB-IoT to support the QoS requirement of an IMS voice service considering the IMS signalling and the IMS voice data transmission have different QoS requirements (i.e. current IMS voice transmission using GBR bearer with QCI = 1, while IMS signalling using non-GBR bearer with QCI = 5)

The solution proposes to remove those restrictions mentioned above to achieve IMS voice service over NB-IoT via GEO connecting to EPC. The solution is based on CP CIoT EPS Optimisation.

To enable the IMS voice transmission via NB-IoT control plane, an IMS PDN connection containing one default bearer and one dedicated bearer is proposed to transmit the IMS voice service, which can be depicted in the following figure:

Figure 6.x.2-1: voice data via a dedicated EPS bearer

The IMS PDN connection based on IP type, therefore, the protocol stack could be as follows:

Figure 6.x.2-2: Protocol stack for IMS signalling

Figure 6.x.2-3: Protocol stack for IMS voice data

Following procedure is an example to create one default bearer and one dedicated bearer to suppot IMS voice service

6.x.3	Procedures

Figure 6.x.3.1: IMS signalling and voice data delivery in CIoT EPS Optimisation over NB-IoT via GEO satellite

Attach

0. A UE, supporting IMS voice service over NB-IoT(GEO), camps on a NB-IoT(GEO) cell.

1. The UE initiates the Attach procedure by the transmission of an Attach Request to the MME, including the Preferred Network Behaviour indicates "Control Plane CIoT EPS Optimisation is preferred", its supporting of IMS voice over NB-IoT(GEO) indication.

2. The MME procees the Attach request from the UE. If the attach is successful, it sends an Attach Accept message to the UE. The MME indicates Control Plane CIoT EPS Optimisation is supported in the Supported Network Behaviour information. If IMS voice service over NB-IoT is supported when the UE is using the NB-IoT(GEO) and the UE is allowed to use IMS voice over NB-IoT(GEO) based on the UE subscription information , the IMS voice over NB-IoT(GEO) Support indication is contained in the message as well.

If an ESM container is included in the attach request and IMS related APN is provided, an IMS PDN connection is established during the attachment. Otherwise, a separate PDN connection establishment procedure is performed to establish the IMS PDN connection.

PDN connection establishment for IMS voice service

3. Based on the IMS voice over NB-IoT(GEO) Support indication, the UE can initiate a PDN connection establishement for IMS service by using an APN related to the IMS service, and the request PDN type is set as IP.

4. If the IMS PDN connection is allowed to establish, a default bearer for the IMS PDN connection is established.

Moreover, considering the dedicated bearer establishment triggered by IMS call setup will cause additional delays, the dedicated bearer can be pre-established in combination with the default bearer establishment. The NB-IoT (GEO) RAT type is sent to PGW via the create session request. The PGW can make use of this information to grant dedicated bearer to be established for the IMS APN PDN connection. In this case, two EPS bearers (one default bearer identified by EBI-1 and one dedicated bearer identified by EBI-2) are established for the IMS PDN connection over NB-IoT(GEO). To prevent the use of the dedicated bearer before IMS voice session is established, the TFT binding to the dedicated bearer shall be set to block all packets from the UL and DL.

Editor’s note:	Whether new QCI is needed for supporting IMS voice service over NB-IoT NTN via GEO is FFS.

5. The steps 7-12 in Annex F of TS 23.401 are reused.  If both the default bearer and dedicated bearer are activiated, the EBI-1 and EBI-2 are provided to the UE.

After the IMS PDN connection is established, the UE can perform IMS registration as specidifed in clause 5.2.2 of TS 23.228 [x], which is via the default bearer of the IMS PDN connection. Then the UE can initiate IMS voice call setup.

IMS call setup

6. The UE can initiate IMS signalling negotiation with remote UE by using CP CIoT EPS Optimisation as defined in clause 5.3.4B.2. The NAS PDU carries the encrypted uplink IMS signalling and EBI-1 to the eNodeB of the NB-IoT(GEO). The UE maps to NAS PDU carrying the IMS signalling to the SRBx,

6a. the eNodeB of the NB-IoT(GEO), based on configuration, may retrieve the EPS negotiated QoS profile from the MME. The QoS profile may be applied to prioritise between requests from different UEs.

7. The NAS PDU sent in step 6 is relayed to the MME by the eNodeB using a S1-AP Initial UE message.

8. The MME checks the integirity of the incoming NAS PDU and decrypts the data it contains.

9. Based on the received EBI-1, the MME mappes the uplink IMS signalling data to the default bearer  and deliveries it to the P-GW.

10. The Downlink IMS signalling is delivered via the default bearer, details can refer to steps 9-12 of Figure 5.3.4B.2-1 of TS 23.401. The eNB maps the DL traffic carrring the IMS signalling to the SRBx.

11. The  P-CSCF sends diameter AAR to the PCRF which provides the QoS requirement based on the SIP negotiation. The PCRF generates the PCC rule for the IMS voice data and provides it to the P-GW.

12. If the dedicated bearer has already established during the IMS PDN connection establishment, the PCC rule installation may trigger P-GW to initiate dedicated bearer modification as described in clause 5.4.3 of TS 23.401[5] to update the DL TFT. The UL TFT can be updated by the UE locally based on the IMS SDP negotiation result.

Otherwise, if there is no dedicated bearer for the IMS PDN connection, the PCC rule installation may trigger P-GW to initiate the dedicated bearer creation procedure as specified in step 2-11 in clause 5.4.1 of TS 23.401 [5].

13. When the P-GW receives a downlink IMS voice data, it delivers the downlink data to the S-GW and further to MME via the dedicated bearer.

14. The MME encrypts and integrity protects Downlink IMS voice data.

15. The MME sends the encrypt downlink IMS voice data along with EBI-2 using a NAS PDU carried by a Downlink S1-AP message. The eNB maps the DL traffic of IMS voice packet to the SRBy.

16. The NAS PDUwith IMS voice data is delivered to the UE via a Downlink RRC message.

17. The uplink IMS voice data transmission via the dedicated bearer can refer to steps 16-19 of Figure 5.3.4B.3-1 of TS 23.401. The UE maps to UL traffic of IMS voice packet to the SRBy.

Editor’s note: 	Whether new SRB is needed for IMS signalling or IMS voice data transmission is FFS and depends on RAN implementation.

Editor’s note: 	Whether and how to provide QoS to the IMS voice service via SRB is FFS and depends on RAN implementation.

Editor’s note: 	Whether and how to enhance or optimize IMS registration, IMS call setup procedures to enable IMS voice over NB-IoT NTN via GEO is FFS.

6.x.4	Impacts on services, entities and interfaces

UE:

-	Request for PDN connection establishment for IMS voice service when using NB-IoT NTN via GEO

-	Support dedicated bearer when using NB-IoT NTN via GEO for IMS voice service

-	Support to update UL packet filter locally based on the IMS SDP negotiation result.

MME:

-	Support verification of IMS voice call subscription considering NB-IoT (GEO), UE and network capability for supporting IMS voice over NB-IoT(GEO)

-	Provide IMS voice service over NB-IoT(GEO) support indication to UE

-	Support for PDN connection establishment for IMS voice service when using NB-IoT NTN via GEO

-	Support to establish dedicated bearer,  GBR bearer for UE using IMS service over NB-IoT NTN via GEO

S-GW/P-GW:

-	Support creating the dedicated bearer, GBR bearer for IMS voice service over NB-IoT (GEO).

eNB:

-	Support establishing a dedicated SRB for IMS voice data

-	Support creating new SRB for IMS signalling or IMS voice data (depends on RAN study)

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[x]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

* * * End of Change * * * *