3GPP TSG-WG SA2 Meeting #169	S2-2505604

Fukuoka, JP, 19th – 23rd May, 2025	was S2-2505231

Source:	NTT DOCOMO

Title:	Update on ADM function

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	Architecture support of Ambient power-enabled Internet of Things (AmbientIoT-ARC) / Rel-19

Abstract: This paper proposes an update to the functionality of ADM.

1. Introduction/Discussion

According to the discussion during previous meeting, the following Editor' note was left in TS 23.369 clause 7.5.2.

Editor's note: Whether AF authorization data is stored in ADM is FFS.

This pCR proposes to update the storage location of AF authorization data to be managed by ADM in clause 4.5.9. The reason is that ADM is well-suited for storing AF authorization data because it efficiently handles network information and ensures accessibility where needed.

2. Text Proposal

It is proposed to capture the following changes in TS 23.369.

# * * * * First change * * * *

4.5.9	ADM

The ADM supports the following functions:

-	Management of AIoT device profile data.

-	Management of AF authorization data.

# * * * * Next change * * * *

7.5.2	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query

Description: NF service consumer may request the AIoT device profile data or the AF authorization data from the ADM.

Inputs, Required: AIoT Device Permanent ID or AF ID.

Input, Optional: None.

Outputs, Required: the AIoT device profile data or the AF authorization data.

Output, Optional: None.

# * * * * End of changes * * * *