3GPP SA WG2 Meeting #169	S2-2505654

Fukuoka, Japan, 19-23 May 2025

# Title:	Reply LS on clarification for multi-hop UE-to-UE relay discovery using model B

# Response to:	LS on clarification for multi-hop UE-to-UE relay discovery using model B (C1-252562/ S2-2504502)

# Release:	Release 19

# Work Item:	5G_ProSe_Ph3

Source:	SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank CT1 for the LS on clarification for multi-hop UE-to-UE relay discovery using model B (C1-252562/ S2-2504502).

SA2 had discussed the matter and would like to provide the following feedback.

There are two possible cases where the Model B discovery is used for multi-hop UE-to-UE Relay of IP data unit type as described in TS 23.304 clause 6.3.2.6.2:

- when the 5G ProSe End UE or the Multi-hop UE-to-UE Relay knows a specific target relay; and

- when the 5G ProSe End UE or the Multi-hop UE-to-UE Relay wants to discovery any Multi-hop UE-to-UE Relay of the RSC nearby.

Only in case 1) the User Info ID of the target relay needs to be included in the Multi-hop UE-to-UE Relay Discovery Solicitation for IP data unit type message.

For case 2), the User Info ID is not included in the message, and the Multi-hop UE-to-UE Relay processes the received Multi-hop UE-to-UE Relay Discovery Solicitation message as below:

- if the relay supports the multi-hop UE-to-UE relay operation of IP data unit type and the RSC in the received message, the relay replies with a multi-hop UE-to-UE relay discovery response for IP data unit type message; and

- otherwise, the relay ignores the solicitation message.

The 5G ProSe End UE or the Multi-hop UE-to-UE Relay would be aware of the presence of the relay if a response message is received.

Therefore, SA2 decided to keep the existing descriptions in TS 23.304, and would like to request CT1 to align with the design as described above.

2. Actions:

To CT1:

ACTION: 	SA2 would like to ask CT1 to take the above information into account and align the design accordingly.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #170		25-29 August 2025	Goteborg, Sweden

TSG-SA2 Meeting #171		13-17 October 2025	China (TBD)