3GPP TSG-SA2 Meeting #169	S2-2505665

# 19 - 23 May, 2025, Fukuoka, Japan		(revision of S2-2504787)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1471 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ML Model accuracy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3, TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ML Model accuracy information is to represent general performance measurements for ML Model as defined in clause 5C.1, while the ML Model accuracy related information is related to the parameters of the ML Model Accuracy as defined in clause 6.1B.4. For step 1 in clause 6.2E.3.2, the NWDAF containing AnLF is a target NWDAF containing AnLF in analytics transfer procedure, and it should receive ML Model Accuracy Related Information instead of ML Model accuracy information as defined in clause 6.1B.4. For step 1 in clause 6.2E.3.3, the provider of ML accuracy monitoring for a given ML Model is not been defined and it should be the provider of ML Model accuracy information for a given ML Model as defined in clause 6.1B.2.1. For the reference in clause 6.20.2, the TS 23.502 does not include clause 5.8.2.4.2, and it should be the TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the ML Model Accuracy Information to ML Model Accuracy Related Information in clause 6.2E.3.2. Correct the the provider of ML accuracy monitoring for a given ML Model to the provider of ML Model accuracy information for a given ML Model in clause 6.2E.3.3. Correct the reference in clause 6.20.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect name may lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.3.2, 6.2E.3.3, 6.20.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.2E.3.2	Procedures for registering the monitoring of the analytics accuracy of an ML Model

When an NWDAF containing AnLF starts making use of an ML Model and it has the ability either to monitor the analytics accuracy of the ML Model, or to deliver Analytics Feedback Information for the analytics generated by the ML Model, it registers with the NWDAF containing MTLF, that is responsible for training/updating this ML Model.

When the NWDAF containing AnLF is no longer using the ML Model or monitoring the accuracy of the analytics generated by that ML Model for the Analytics ID, it de-registers it with the responsible NWDAF containing MTLF.

Figure 6.2E.3.2-1 illustrates the procedure by which an NWDAF containing AnLF registers with an NWDAF containing MTLF that it is starting to make use and monitor the analytics accuracy of an ML Model. A new Nnwdaf_MLModelMonitor_Register service operation is used for that purpose.

Figure 6.2E.3.2-1: Procedure for ML Model monitoring registration

An NWDAF containing AnLF may start monitoring the accuracy of an ML Model based on local policy or request from its service consumer.

1-2.	The NWDAF containing AnLF sends an Nnwdaf_MLModelMonitor_Register request to an NWDAF containing MTLF (NF ID of the NWDAF containing AnLF, unique identifier of the ML Model, optionally: subscription endpoint of the Nnwdaf_MLModelMonitor_Subscribe service operation at the NWDAF containing AnLF, Analytics ID, Target of Analytics Reporting, Analytics filter). The NWDAF containing MTLF is now aware of the NF ID of the NWDAF containing AnLF that is monitoring the accuracy of that ML Model.

If the NWDAF containing AnLF is a target NWDAF in analytics transfer procedure (as defined in clause 6.1B), based on the ML Model accuracy related information received from source NWDAF containing AnLF, the NWDAF containing AnLF also includes in the Nnwdaf_MLModelMonitor_Register service request the ML Model accuracy transfer indication, which includes the original Subscription Correlation ID for the ML Model Accuracy Information provided by the source NWDAF containing AnLF and the NF ID of the source NWDAF containing AnLF.

NOTE 1:	These parameters support the NWDAF containing MTLF to map the registration of a new NWDAF containing AnLF with an existing subscription for consumption of ML Model Accuracy Information from a previous NWDAF containing AnLF (i.e. source NWDAF containing AnLF which as described in steps 3-4 may provide a termination indication), enabling NWDAF containing MTLF to reassociate the data from the previous subscription to the new the subscription for ML Model accuracy provisioning at the new NWDAF containing AnLF.

3-4.	When the NWDAF containing AnLF is no longer using the ML Model, it sends an Nnwdaf_MLModelMontior_Deregister service operation.

If NWDAF containing AnLF is registered with a NWDAF containing MTLF, is a source NWDAF containing AnLF in an analytics transfer procedure (as defined in clause 6.1B) and is no longer using the ML Model, the NWDAF containing AnLF sends Nnwdaf_MLModelMontior_Deregister service operation request including the ML Model accuracy provisioning termination information, which includes: a termination indication, the termination cause set to analytics transfer and optionally the NWDAF containing AnLF NF ID of the target NWDAF.

NOTE 2:	The ML Model accuracy termination information is used by the NWDAF containing MTLF to determine whether the termination request is from the source NWDAF containing AnLF. If so, the NWDAF containing MTLF will not delete any data immediately upon receiving of a de-registration request. Then the NWDAF containing MTLF is able to associate the data from the source NWDAF containing AnLF to the target NWDAF containing AnLF.

# * * * * Next changes * * * *

#### 6.2E.3.3	Procedures for monitoring the analytics accuracy of an ML Model

An NWDAF containing MTLF, due to the registration of monitoring of the analytics accuracy of an ML Model received from NWDAF containing AnLF and local policies, subscribes to the NWDAF containing AnLF for receiving notifications of either the accuracy of the ML Model, or Analytics Feedback Information of the ML Model. The NWDAF containing MTLF may get the Subscription endpoint address of the NWDAF containing AnLF from the information received in a previous registration or through a service discovery procedure at the NRF.

Figure 6.2E.3.3-1 illustrates the procedure either for monitoring the analytics accuracy of an ML Model or for delivery of Analytics Feedback Information of an ML Model. Nnwdaf_MLModelMonitor_Subscribe and Nnwdaf_MLModelMonitor_Notify service operations are used for the purposes. A service consumer, i.e. an NWDAF containing MTLF, subscribes at a service producer, i.e. an NWDAF containing AnLF, to be notified when either the analytics accuracy of the previously provisioned ML Model is not sufficient, or Analytics Feedback Information is retrieved from analytics consumer NF.

Figure 6.2E.3.3-1: Procedure for monitoring the analytics accuracy of an ML Model

0.	Upon the reception of an Nnwdaf_MLModelMonitor_Register request and based on local policy, the NWDAF containing MTLF determines to subscribe to the Analytics Accuracy Monitoring for the ML Model as defined in clause 5C.1.

1.	The NWDAF containing MTLF sends an Nnwdaf_MLModelMonitor_Subscribe request (Analytics ID(s), unique identifier(s) of the ML Model(s) to be monitored, desired accuracy metrics to be monitored, optionally Reporting Threshold(s), Analytics ID, Target of Analytics Reporting and Analytics filter for each ML Model identifier or Reporting Period) to an NWDAF containing AnLF subscription endpoint.

When the NWDAF containing MTLF determines during the registration process described in clause 6.2E.3.2 that a subscription request for ML Model Accuracy Monitoring to an NWDAF containing AnLF is related to a previous subscription for ML Model Accuracy Information to a different NWDAF containing AnLF (due to changes in the provider of the ML Model accuracy information for a given ML Model, as an effect of analytics transfer among NWDAFs containing AnLF), the NWDAF containing MTLF may use as base for the new subscription request at the new NWDAF containing AnLF the parameters associated with the original subscription identification for the ML Model Accuracy Information that was received in the registration request of the new NWDAF containing AnLF, as described in steps 1-2 of clause 6.2E.3.2.

2.	The NWDAF containing AnLF sends a response to the NWDAF containing MTLF.

3.	The analytics consumer NF may send Analytics Feedback Information to the NWDAF containing AnLF as described in clause 6.1.1.

4.	When step 1 is triggered, the NWDAF containing AnLF may start monitoring the analytics accuracy of the ML Model(s), if it not started yet.

NOTE 1:	The NWDAF containing AnLF can monitor the analytics accuracy in many ways: e.g. comparing predictions of ML Model and its corresponding ground truth data, comparing changes in internal configuration for the analytics ID generation, previous existent records of Analytics Accuracy Information, etc.

5.	The NWDAF containing AnLF determines whether the analytics accuracy of the ML Model is insufficient, i.e. deviation of the output analytics using the trained ML Model from ground truth data (which are collected from Data Producer NF corresponding to analytic ID requested at the time which the prediction refers to) does not meet the analytics accuracy requirement, which indicates the accuracy value is under the Reporting Threshold(s) (which are locally configured or received in the Subscribe request), or the Reporting Period indicated in the Subscribe request is reached.

6.	Either the Analytics Feedback Information is retrieved at step 3 or the NWDAF containing AnLF detects the analytics accuracy of ML Model is insufficient at step 5, the NWDAF containing AnLF sends an Nnwdaf_MLModelMonitor_Notify request to the notification endpoint (e.g. the NWDAF containing MTLF). The Notify request includes either Analytics Feedback Information, or the monitored accuracy information of the ML Model (e.g. unique identifier(s) of the ML Model(s) to be monitored, Analytics ID, Target of Analytics Reporting and Analytics filter for each ML Model identifier, a deviation value which indicates the deviation of the predictions generated using the ML Model(s) from the ground truth data and the network data when the deviation occurs (which can be used by the NWDAF containing MTLF for possible ML Model retraining) and the number of inferences that were performed during the time interval between Nnwdaf_MLModelMonitor_Register request and the Notify request or between the time of last Notification message and the time of the current Notification message) and optionally an indication that the analytics accuracy of the ML Model does not meet the requirement of accuracy for the ML Model.

7.	The NWDAF containing MTLF sends a response.

8.	The NWDAF containing MTLF determines whether the ML Model is degraded or not based on the notification at step 6. If the notification contains Analytics Feedback Information, the NWDAF containing MTLF may determine ML Model degradation based on the procedures as described in clause 6.2E.2. Otherwise when the NWDAF containing MTLF has received the multiple Analytics Accuracy Information, from one or more NWDAFs containing AnLF, it may consider that the ML Model is degraded/to be updated (i.e. enough number Analytics Accuracy Information received from one or more NWDAFs containing AnLF, indicating insufficient analytics accuracy).

NOTE 2:	The actual mechanism for the NWDAF containing MTLF for determining the degradation of the ML Model degradation is an internal procedure of the NWDAF containing MTLF, e.g. the NWDAF containing MTLF calculate a global accuracy based on the Analytics Accuracy Information and the number of inferences received from multiple NWDAFs containing AnLF.

9.	When an ML Model is considered degraded / to be updated at step 8, the NWDAF containing MTLF re-trains the existing ML Model or selects a new ML Model. If the network data was not included in the Nnwdaf_MLModelMonitor_Notify request of step 6, the NWDAF containing MTLF may request data from the NWDAF containing AnLF, ADRF and/or other 5GS entities as specified in clause 6.2 and use the collected data for ML Model retraining. The NWDAF containing MTLF notifies the NWDAF(s) containing AnLF with the updated trained ML Model Information by invoking Nnwdaf_MLModelProvision_Notify service operation, as described in clause 6.2A.

# * * * * Next changes * * * *

### 6.20.2	Input Data

NWDAF collects input data from the SMF and/or UPF. The detailed data are described in Table 6.20.2-1.

Table 6.20.2-1: Collected PDU Session User Plane Traffic Information

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF, UPF | UE ID for the UE that established PDU sessions to an S-NSSAI/DNN. |
| S-NSSAI | SMF, UPF | The S-NSSAI for which the PDU session is established. |
| DNN | SMF, UPF | The DNN for which the PDU session is established. |
| PDU Session Information |  | The detected Service Data Flow, volume measured and Application related information in a PDU Session. |
| > Packet Filter Set | SMF, UPF | Packet Filter Set for the detected Service Data Flow as defined in clause 5.8.2.4.2 of TS 23.501 [2]. |
| > URL list | SMF, UPF | If available, list of URLs extracted from the inspected user plane packets in the Service Data Flow. |
| > Domain Name list | SMF, UPF | If available, list of domain names extracted from the inspected user plane packets in the Service Data Flow. |
| > UL Data volume | SMF, UPF | Measured UL data traffic volume for the Service Data Flow for the duration of the PDU Session. |
| > DL Data volume | SMF, UPF | Measured DL data traffic volume for the Service Data Flow for the duration of the PDU Session. |

NOTE:	Care needs to be taken with regards to load and major signalling caused when requesting Any UE. This could be achieved via utilization of some event filters (e.g. Area of Interest), a specific DNN, S-NSSAI or sampling ratio as part of Event Reporting Information.

The NWDAF collects input data from the UPF either indirectly via the SMF, or directly from the UPF using the "UserDataUsageMeasures" event exposure event as described in clause 4.15.4.5 of TS 23.502 [3]. Further details about input parameters are described in Table 4.15.4.5.1 of TS 23.502 [3].

# * * * * End of changes * * * *