SA WG2 Meeting #S2-169	S2-2505741

19 - 23 May, 2025, Fukuoka, Japan

Title:	Draft Reply LS on N6-Unmarked PDUs

Response to:	S2-250465/S4-250738

Release:	Rel-19

Work Item:	5G_RTP_Ph2 / XRM_Ph2

Source:	SA2(CATT)

To:	SA4, CT4

Cc:	-

Contact Person: 	Chunshan Xiong (chunshan.xiong@cictmobile.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks for the LS from the SA4, SA2 has the following  answer for the LS:

In TS23.501, it is documented that "The 5G-AN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion." i.e., the PSI is mainly used during the congestion control in the NG-RAN. Also, as described in TS23.501, only one pre-configuration PSI value is used for all types of N6-unmarked PDUs.  SA2 thinks providing different PSI values for different N6-unmarked PDUs may help  to improve the congestion control for the PDU Sets.

Now SA2 has identified four potential solutions to provide different PSI values for N6-unmarked PDUs as following:

- Signalling to the PCF of different PSIs and associated Protocol Descriptions from AF as proposed by the SA4 in the LS.

- Preconfigure of different PSIs and associated Protocol Descriptions in the PCF. e.g. based on SLA；

- Preconfigure of different PSIs and associated Protocol Descriptions in the UPF. e.g based on SLA;

- Selects to one PSI (e.g. the most important PSI) from the PSIs of the PDU Sets by the UPF;

SA2 is planning to selection one solution without introducing additional signalling from the above solutions.

2. Actions:

To SA4

ACTION: 	SA2 kindly asks SA4 to take the above information into account and welcome to provide further information on PSI values for N6-unmarked PDUs.

3. Dates of Next SA2 Meetings:

SA2#170	25th – 29th August 2025 		 Goteborg, SE

SA2#171	13th – 17th October 2025 		 TBD, CN