SA WG2 Meeting #169  	S2-2505755

19-23 May 2025, Fukuoka, Japan

Source:	Nokia

Title:	Solution proposal for Key Issue #2

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes a solution for key issue 2.

# 1. Discussion

SA2 and SA have agreed a Rel-20 5GA study item “Study on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML) Phase 2 (FS_AIML_CN_Ph2)” in SP-250413 aiming to produce the new TR 23.700-04.

This includes the following objective:

WT#2: Whether and how to enhance the 5GC analytics to support efficient performance of user plane

Note 4: This may be based on the analysis of the user plane traffic (e.g. traffic patterns, packet information) and the reaction of the network per the analysis is based on the existing policy framework.

Note 5: The study will not address how UPF detects wrong or malformed or duplicated packets. This WT shall   have neither RAN nor UE impacts. Solutions shall minimize the load on Control Plane NFs due to too much reporting from UPF.

The related key issue and use cases in TR 23.700-84 are defined as follows:

## 5.3	Key Issue #2: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane

Use of AI/ML can be studied to provide analytics on user plane traffic patterns and how to use the analytics to improve user plane performance. In this context, it is proposed to study:

-	How to provide analytics (e.g. enhancing existing in TS 23.288 [5] or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

-	Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

-	Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.

### 5.1.1	Use Case #1: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane

The UPF is responsible for managing substantial volumes of uplink and downlink traffic originating from UEs' applications or application servers. It must be robust and resilient against abnormal data traffic, for examples, large amount of abnormal or malicious IP packets, flows, bursts, syn-ack signalling. Without actions to mitigate the impact of such potential abnormal or malicious traffic pattern in the user plane, the UPF may experience performance degradation which can lead to resource inefficiency. Use of AI/ML can be studied:

-	to provide analytics on user plane traffic patterns;

-	to apply mitigation strategies using analytics, e.g. avoid abnormal or malicious traffic;

so as to improve efficient performance of user plane as well as UPF's robustness.

### 5.1.2	Use Case #2: User plane performance optimization with assistance of NWDAF

It is beneficial for the network to know about the traffic pattern in advance in order to optimize user plane performance, e.g. better QoS control. For instance, the network can generate MDBV, Periodicity for TSC service based on the traffic pattern as described in TS 23.501 [2].

Currently, the above mechanism can only be applicable for services with special protocols, e.g. TSN. For other kinds of applications, most of them rely on transport layer protocol to mitigate congestion, e.g. TCP or QUIC as described in RFC 793 [8] and RFC 9000 [9]. Then, the application behaviour (e.g. packet sending rate) is related to the network status, (e.g. the E2E delay and packet loss, etc.). Hence, it is hard to give a precise traffic pattern in advance to the network from the AF/AS.

The analytic capability of NWDAF can generate the statistic or prediction of the traffic pattern for 5GC to do user plane performance optimization, e.g. assistance to QoS control management, in advance.

NOTE 1:	Solutions for this use case should aim at avoiding extensive signalling.

NOTE 2:	Solutions will reuse existing policy and QoS control as much as possible.

NOTE 3:	Solutions shall not include any recommendations by the NWDAF.

# 2. Proposal

Incorporate the following text in TR 23.700-04

* * * 1st Change * * *

## 6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X |  | X | X | X |

* * * 2nd Change (All New Text) * * *

## 6.X	Solution #X: Assistance for User plane performance optimization

### 6.X.1	High level principles

A UPF can be configured to report Information about detected packets complying to certain pattern(s) (e.g. same size, protocol, same target address/port etc) received from multiple external sources, abnormal amounts of traffic received from external source, and/or rate of dropped packets.

An NWDAF as consumer of the UPF notification can use this information to compile statistics and/or analytics related to abnormal user plane observations (e.g.  detection of special packets/ special traffic volumes from specific sources and/or  the detection of dropped packets).

Consumers of related NWDAF analytics could then block abnormal traffic or to increase capacity for legitimate traffic peaks. An IoT Server AF as analytics consumer can also use the information to control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices.

### 6.X.2	Description

A UPF can be configured to report one or several of:

- Information about detected packets complying to certain pattern(s) (identified by any of the following criteria, possibly in combination: same size, protocol, same target address/port, packets identified by a vendor specific algorithm identified by a packet patternID, packets matching search criteria obtained from databases about known malicious packets, etc, possibly only first packet(s) from a source considered) received from multiple external sources. (High rates could be caused e.g. by distributed DOS attack or protocol/application malfunction or misconfiguration)

-	Target Packets or packet patterns are identified by packet pattern ID, or any type of frequently occurring packets can be reported. Target Packets/packet patterns are identified by a special packet pattern ID without standardized values; Possible values are configured in UPF and event exposure consumer (NWDAF). Detection of related packets is not standardized (could apply AI or other algorithms in UPF)

Editor´s note: An AI capability in UPF is ffs. A possible need to further describe the detection of packet patterns identified by packet pattern IDs (e.g. relating to AI in the UPF) is ffs.

Editor´s note: Whether the application ID can be used instead of the packet pattern ID is ffs.

- Characteristics of detected packets patterns are reported (e.g. source or destination ports, packet size, IP Protocol ID (TCP, UDP etc), special packet IDs, observed packet rate)

- Allows to understand normal traffic levels and identify sudden abnormal increases

- Only sudden increases or very high ratees of similar packets are reported (could be caused e.g. by distributed DOS attack or malfunctioning server/router).

- Targets of reporting can be traffic for any UE, certain UEs or UE groups, certain applications, and/or packets received from certain external IP addresses / domains

- Reporting should identify when possible affected UEs, and/or traffic filters for related packet streams, , and/or application IDs to enable SMF/UPF/PCF/AF to take appropriate corrective actions

- (abnormal) amounts of traffic received from external source (could be caused e.g. by DOS attack or protocol/application malfunction or misconfiguration)

- Allows to understand normal traffic levels and identify sudden abnormal increases

- Only external sources with high volume and/or sudden increases are reported

- Targets of reporting could be traffic for certain UEs or UE groups, certain applications, and/or packets received from certain external IP addresses / domains

- Reporting should identify when possible affected UEs, and/or traffic filters for related packet streams and/or application IDs to enable SMF/UPF/PCF/AF to take appropriate corrective actions

3.	rate of dropped packets (e.g. due to QoS enforcements/rate control/invalid uplink target address)

-	only report high rates or sudden increases of dropped packets

- Targets of reporting could be traffic for certain UEs or UE groups certain applications, and/or packets received from certain external IP addresses / domains

- Reporting should identify when possible affected UEs, and/or traffic filters for related packet streams and/or application IDs to enable SMF/UPF/PCF/AF to take appropriate corrective actions

- Cause of packet dropping (e.g. QoS enforcement, closed gates, invalid uplink target address, UE not reachable, Buffer overflow) can be part of the reporting

Editor´s note: Whether some of the suggested events can be combined is ffs.

The existing mechanism for UPF event exposure with subscription via SMF or directly at the UPF are reused and enhanced with new events as described above. For the subscription via SMF, the N4 protocol is also enhanced to enable subscriptions with the new events.

An NWDAF as consumer of the UPF notification can use this information to compile statistics and/or analytics related to user plane observations (e.g.  detection of special packets/ special traffic volumes from specific sources and/or the detection of dropped packets)

-	Subscription/request to/for related Analytics/Statistics can target any UE, a group of UEs, or specific UEs, special Application IDs, traffic filters, SNSSAI, DNN, and can indicate desired type of abnormal user plane observations (e.g.  detection of special types of packets/ special traffic volumes from specific sources and/or the detection of dropped packets), and also desired special packet type (if applicable), target Causes for packet drop (if applicable)

-	Analytics/statistics reports can indicate applicable UEs, Analytics and Application IDs, traffic filters, SNSSAI, DNN, type of abnormal user plane observations, rate/frequency/count of abnormal user plane observations, cause of packet drop (if applicable), expected/observed time period(s) of abnormal user plane observations

A PCF as consumer of NWDAF analytics could then apply PCC rules to block abnormal traffic, or to optimize the user plane performance optimization by adjusting the QoS for instance to increase capacity for legitimate traffic peaks.

A SMF as consumer of NWDAF analytics could then configure UPFs to block abnormal traffic

OAM as consumer of NWDAF analytics could configure SMF and/or UPF to block abnormal traffic or to increase capacity for legitimate traffic peaks.

An AF as consumer of NWDAF analytics and data source could take action to correct abnormal traffic. For instance, in an enterprise network or IoT deployments, an (IOT) server to control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices.

### 6.X.3	Procedures

Figure 6.X.3-1 shows an example procedure where a request for QoS is provided by the AF or UE.

ed

Figure 6.X.3-1: Procedure for Abnormal user plane traffic mitigation.

1.	An NF aiming to mitigate abnormal user plane behaviour may subscribe at the NWDAF for related analytics/statistics (e.g.  detection of packets complying to some pattern/ special traffic volumes from specific sources and/or the detection of dropped packets). Subscription/request to/for related Analytics/Statistics can target any UE, a group of UEs, or specific UEs, special Application IDs, traffic filters, SNSSAI, DNN, and can indicate desired type of abnormal user plane observations (e.g.  detection of special types of packets/ special traffic volumes from specific sources and/or the detection of dropped packets), and also desired special packet type (if applicable), target Causes for packet drop (if applicable).

2.-3 an SMF handling particular UEs is discovered via the UDM

4.-5. SMF(s) or UPF(s) in a PLMN are discovered via the NRF (e.g. all SMFs or UPFs if all UEs are targeted)

6.	The NWDAF subscribes to notifications related to the detection of special packets and/or high traffic volumes from specific sources and/or high rates of dropped packets at the identified UPF(s). The subscription can be indirect via the identified SMF(s) (e.g. if specific UE groups are targeted) or directly at the UPF

7.	One of the UPF(s) detects events as outlined in Clause 6.X.2, for instance special packets patterns, abnormal load from a special source, or a high level of packet dropping.

8.	The UPF(s) notifies the subscribing NWDAF about the detected packets complying to some packets, abnormal load from a special source, or abnormal packet dropping.

9.	The NWDAF generates analytics related to user plane behaviour

10. The NWDAF notifies the analytics consumer (i.e. NF aiming to mitigate abnormal user plane behaviour) about abnormal user plane behaviour.

11. The analytics consumer takes actions to  mitigate abnormal user plane behaviour or to optimize user plane performance. For instance, an AF con reduce traffic to UEs or reconfiguring UEs; a PCF can adjust PCC rules to drop packets or to increase MBR.

### 6.X.4	Impacts on services, entities and interfaces

-	NWDAF: Provide new analytics about user plane observations. Subscribe to related event exposure from the UPF for input data.

-	UPF: Support event exposure for Rate of similar packets, abnormal) amounts of traffic received from external source, rate of dropped packets

-	SMF:  Support receiving requests for event exposure for Rate of similar packets, (abnormal) amounts of traffic received from external source, rate of dropped packets, and configure UPF to expose those events

-	Analytics consumer: Take mitigation or user plane optimization actions based on new analytics about user plane observations. For instance, an AF can reduce traffic to UEs or reconfiguring UEs; a PCF can adjust PCC rules to drop packets or to increase MBR.

*** END of changes ***