SA WG2 Meeting #169	S2-2505776

# May 19 – 23, 2025, Fukuoka, Japan		    (revision of S1-2505532)

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: Support of PDN connection where packets are transported as Non-IP over the GEO satellite and seen as IP by IMS nodes

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#1.

1	Introduction

This contribution proposes a solution for KI#1 on IMS voice service over GEO satellite, in particular addressing message size optimization. In this solution, for UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets.

2	Proposal

It is proposed to add the following text to TR 23.700-19 v010.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| Z | X |  |  |  |

********** Next Change (All new text) **********

6.x	Solution #Z: Support of PDN connection where packets are transported as Non-IP over the GEO satellite and seen as IP by IMS nodes

6.x.0	High level principles

This solution addresses KI#1, in particular message size optimization.

In this solution, for UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets.

This solution is intended to be used together with another solution shown in "Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections".

6.x.1	Description

Principles of this solution are as follows:

- For UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets.

- UE receives an IP address allocated to it from P-GW at establishment of the PDN connection for voice media and uses the IP address when creating contents of SDP messages.

- UE sends to P-GW an IP address allocated in IMS AGW, when the UE receives the IP address in SDP. P-GW uses this IP address when adding IP header to voice media packets.

- P-GW is assumed to know the IP address of P-CSCF by pre-configuration. P-GW uses this IP address when adding IP header to SIP messages.

Editor's note: The current description mentions IP header only. FFS whether to consider UDP/TCP header addition/removal as well. FFS what impact is required if UDP/TCP header addition/removal is also treated.

6.x.2	Procedures

6.x.2.1	General

The procedures in the below are an example and depict the case where the proposed solution is used together with another solution shown in "Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections". This another solution is referred to as the base solution in the below.

6.x.2.2	Establishment of PDN connection for SIP message exchange

Figure 6.x.2.2-1: Establishment of PDN connection for SIP message exchange

Steps of Figure 6.x.2.2-1 of the base solution are applied with the following modifications.

- This step is the same as step 0 of the base solution.

- This step is the same as step 1 of the base solution with the following modifications: The PDN connectivity request additionally contains PDN type=IPv6 and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 2 of the base solution with the following modifications: The Create Session Request additionally contains PDN type=IPv6 and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 3 of the base solution with the following modifications: The Create Session Request additionally contains PDN type=IPv6 and Extended protocol configuration options that include "CN header handling request" (a new indicator).

- This step is the same as step 4 of the base solution.

Steps 5 to 9 correspond to step 5 of the base solution.

- PCRF sends to P-GW the CC-Answer. P-GW recognizes the need of the CN header handling. P-GW receives or assigns an IP address for UE and recognizes that this IP address is to be used for the source address for UL packets for the CN header handling for this PDN connection. P-GW recognizes that an P-CSCF address that the P-GW has known by configuration is to be used for the destination address for UL packets for the CN header handling for this PDN connection based on Access Point Name = ims and PDN connectivity pair ID. (new behavior of P-GW)

- P-GW sends to S-GW the Create Session Response containing PDN Address Allocation that includes the allocated IPv6 address and Extended protocol configuration options that include "CN header handling accepted indication" (a new indicator).

- S-GW sends to MME the Create Session Response containing Control plane only indication, PDN Address Allocation that includes the allocated IPv6 address, and Extended protocol configuration options that include "CN header handling accepted indication" (a new indicator).

- MME sends to UE the Activate Default EPS Bearer Context Request containing Control plane only indication, PDN Address Allocation that includes the allocated IPv6 address, and Extended protocol configuration options that include "CN header handling accepted indication" (a new indicator). UE recognizes the IPv6 address allocated to it.

- UE sends to MME the Activate Default EPS Bearer Context Accept.

- This step is the same as step 6 of the base solution with the following modifications: This PDN connection is set to transfer Non-IP packets between UE and P-GW and the P-GW is expected to add IP header for UL packets and remove IP header for DL packets.

6.x.2.3	IMS registration

Figure 6.x.2.3-1: IMS registration

- A path for SIP message exchange is established as a result of the procedure depicted in clause 6.x.3.2.

- UE sends SIP Register without IP header to MME by using NAS.

- MME sends SIP Register without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to SIP Register without IP header; P-GW adds the UE IP address as the source address and the P-CSCF IP address as the destination address. (new behavior of P-GW)

- P-GW sends SIP Register with IP header to P-CSCF.

- P-CSCF sends SIP Register to S-CSCF.

- P-CSCF receives 200 OK.

- P-CSCF sends 200 OK to P-GW.

- P-GW removes IP header of 200 OK. (new behavior of P-GW)

- P-GW sends 200 OK without IP header to MME on U-plane via S-GW.

- MME sends 200 OK without IP header to UE by using NAS.

6.x.2.4	IMS voice call setup

6.x.2.4.1	MO procedure

NOTE:	Steps 13 to 18 in the below for establishing a PDN connection for voice media can be performed only after UE receives SDP answer (in MO side)

Editor's note: FFS whether SIP timers are negatively impacted.

Figure 6.x.2.4.1-1: IMS voice call setup MO

Steps of Figure 6.x.2.3.1-1 of the base solution are applied with the following modifications.

- This step is the same as step 0 of the base solution.

- This step is the same as step 1 of the base solution with the following modifications: SIP INVITE is without IP header.

Steps 2 to 4 correspond to step 2 of the base solution.

- MME sends SIP INVITE without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to SIP INVITE without IP header; P-GW adds the UE IP address as the source address and the P-CSCF IP address as the destination address. (new behavior of P-GW)

- P-GW sends SIP INVITE with IP header to P-CSCF.

Steps 5 to 8 are the same as steps 3 to 6 of the base solution.

Steps 9 to 11 correspond to step 7 of the base solution.

- P-CSCF sends SIP 183 Session Progress to P-GW.

- P-GW removes IP header of SIP 183 Session Progress. (new behavior of P-GW)

- P-GW sends SIP 183 Session Progress without IP header to MME on U-plane via S-GW.

- This step is the same as step 8 of the base solution with the following modifications: SIP 183 Session Progress is without IP header.

- This step is the same as step 9 of the base solution with the following modifications: UE recognizes an IP address allocated in IMS AGW in SDP answer in SIP 183 Session Progress without IP header. The PDN connectivity request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 10 of the base solution with the following modifications: The Create Session Request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 11 of the base solution with the following modifications: The Create Session Request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 12 of the base solution.

- This step is the same as step 13 of the base solution with the following modifications: P-GW recognizes the need of the CN header handling. P-GW receives or assigns an IP address for UE and recognizes that this IP address is to be used for the source address for UL packets for the CN header handling for this PDN connection. P-GW recognizes that an IMS AGW address that the P-GW has received in "Destination Address for CN header handling" (a new parameter) is to be used for the destination address for UL packets for the CN header handling for this PDN connection based on Access Point Name = ims_media and PDN connectivity pair ID. (new behavior of P-GW)

- This step is the same as step 14 of the base solution.

- This step is the same as step 15 of the base solution with the following modifications: The PDN connection is set to transfer Non-IP packets between UE and P-GW and the P-GW is expected to add IP header for UL packets and remove IP header for DL packets.

- This step is the same as step 16 of the base solution with the following modifications: Exchanged SIP messages are without IP header between UE and P-GW.

Steps 21 to 25 are the same as steps 17 to 21 of the base solution.

- This step is the same as step 22 of the base solution with the following modifications: Exchanged SIP messages are without IP header between UE and P-GW.

- UE sends UL voice media packets without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to UL voice media packets without IP header; P-GW adds the UE IP address as the source address and the IMS AGW IP address as the destination address. (new behavior of P-GW)

- P-GW sends voice media packets with IP header to IMS AGW.

- P-GW receives voice media packets with IP header from IMS AGW.

- P-GW removes IP header of voice media packets. (new behavior of P-GW)

- P-GW sends voice media packets without IP header to UE on U-plane via S-GW.

6.x.2.4.2	MT procedure

NOTE:	Steps 13 to 14 in the below for establishing a PDN connection for voice media can be performed only after UE receives SDP offer (in MT side).

Editor's note: FFS whether SIP timers are negatively impacted.

Figure 6.x.2.4.2-1: IMS voice call setup MT

Steps of Figure 6.x.2.3.2-1 of the base solution are applied with the following modifications.

Steps 0 to 1 are the same as steps 0 to 1 of the base solution.

Steps 2 to 4 correspond to step 2 of the base solution.

- P-CSCF sends SIP INVITE to P-GW.

- P-GW removes IP header of SIP INVITE. (new behavior of P-GW)

- P-GW sends SIP INVITE without IP header to MME on U-plane via S-GW.

- This step is the same as step 3 of the base solution with the following modifications: SIP INVITE is without IP header.

- This step is the same as step 4 of the base solution with the following modifications: SIP 183 Session Progress is without IP header.

Steps 7 to 9 correspond to step 5 of the base solution.

- MME sends SIP 183 Session Progress without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to SIP 183 Session Progress without IP header; P-GW adds the UE IP address as the source address and the P-CSCF IP address as the destination address. (new behavior of P-GW)

- P-GW sends SIP 183 Session Progress with IP header to P-CSCF.

Steps 10 to 12 are the same as step 6 to 8 of the base solution.

- This step is the same as step 9 of the base solution with the following modifications: UE recognizes an IP address allocated in IMS AGW in SDP offer in SIP INVITE without IP header. The PDN connectivity request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 10 of the base solution.

- This step is the same as step 11 of the base solution with the following modifications: The PDN connection is set to transfer Non-IP packets between UE and P-GW and the P-GW is expected to add IP header for UL packets and remove IP header for DL packets.

- This step is the same as step 12 of the base solution with the following modifications: Exchanged SIP messages are without IP header between UE and P-GW.

- UE sends UL voice media packets without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to UL voice media packets without IP header; P-GW adds the UE IP address as the source address and the IMS AGW IP address as the destination address. (new behavior of P-GW)

- P-GW sends voice media packets with IP header to IMS AGW.

- P-GW receives voice media packets with IP header from IMS AGW.

- P-GW removes IP header of voice media packets. (new behavior of P-GW)

- P-GW sends voice media packets without IP header to UE on U-plane via S-GW.

6.x.3	Impacts to Services, Entities and Interfaces

Since the solution is an extension of the solution in 6.X "Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections", the impacts to UE, MME, S-GW, P-GW, PCRF, and P-CSCF are also inherited from that solution. In addition, the following impacts apply.

UE:

- Handling of new indicators "CN header handling request" and "CN header handling accepted indication".

- Extracting an IMS AGW IP address from SDP and informing it to MME by using a new parameter "Destination Address for CN header handling".

MME:

- Transparent forwarding of a new parameter "Destination Address for CN header handling"

S-GW:

- Transparent forwarding of a new parameter "Destination Address for CN header handling"

P-GW:

- Handling of new indicators "CN header handling request" and "CN header handling accepted indication”

- Adding IP header for UL packets and removing IP header from DL packets.

- Receiving an IP address in a new parameter "Destination Address for CN header handling" from S-GW and using it when adding IP header for UL packets as the destination address.

Editor's note: The impacts on P-CSCF is FFS.

********** End of Changes **********