3GPP TSG-WG SA2 Meeting #169	S2-2505799

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-2505338, 5569)

Source:	Huawei, HiSilicon

Title:	Clause 5.4 Assistance Information Update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Clause 5.4 is updated to modify the assistance information based on LS from RAN3 and solve the two Editor’s Notes as well.

# 1. Introduction/Discussion

LS from RAN3 (R3-252481) on Ambient IoT further progress of RAN3 mentions that there will be a “follow-on command indication” in the AIOTF information container in the Inventory Request message. Therefore, the service type in the Assistance Information is redundant and can be deleted accordingly.

There are two Editor’s Notes in clause 5.4.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

The first Editor’s note can be deleted directly since currently there is no other necessary assistance information foreseen and if there are any other requirements from RAN WG(s) to provide more assistance information further, this clause can be updated directly. Therefore, there is no need to keep this Editor’s note.

The second Editor’s note can be solved by adding further clarification in this clause and there is no need to capture the content in a separate clause since it is a parameter belonging to this assistance information.

In terms of the parameters of approximate number of AIoT devices and approximate D2R message size, if AF has provided them, these two values can be derived from AF request. If AF does not provide such information, some default values can be configured at AIOTF to further assist RAN for radio resource allocation as they are approximate value instead of exact value. Hence, configuration can work. Otherwise, RAN has no idea how to allocate the radio resources accordingly, which is not a complete and good design.

Additionally, based on RAN WG3 decision, there will be Inventory Assistance Information and Command Assistance Information in Inventory Request and Command Request respectively. Therefore, in order to make it clear and keep aligned with RAN3 specification in SA2 specification, the content included in this clause will be will be divided into two parts which are Inventory Assistance Information and Command Assistance Information accordingly.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

## 5.4	Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests.

For Inventory or Command service operation, following Inventory Assistance Information is included in the Inventory Request from AIOTF to NG-RAN:

a)	AIoT service type (e.g. Inventory, Command)

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	Expected Inventory Response from the AIoT Device  message size.

d)	Optionally, time interval for AIoT Device response aggregation.

For Command service operation, following Command Assistance Information is included in the Command Request from AIOTF to NG-RAN:

e)	Approximate Command Response message size.

Bullet c) is determined by AIOTF based on the estimated AIoT Device ID length for the AIoT Devices that respond to Inventory.

If not provided by the AF, bullet d) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g., if the AF provided time interval is shorter than a locally configured minimum interval.

The assistance information is used by the NG-RAN for performing service operations, e.g., radio resource allocation by using bullets b), c) and e), AIoT Device responses aggregation by using bullet d).

# * * * * End of changes * * * *