3GPP TSG-SA2 Meeting #169	S2-2506047

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025                   revision of S2-2505662_was_05254

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5505 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Simultaneous connectivity failure notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2, TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It was agreed that an AF provides the information on AF subscription together with the Indication for Simultaneous Connectivity over the source and target PSA at Edge Relocation to receive from SMF the notification indicating that Simultaneous Connectivity is not possible. This CR proposes to add the corresponding missing part in SMF event exposure description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing part for notification of Simultaneous Connectivity failure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-aligned specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.8.3.1	General

Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on PDU Sessions. The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for an Event ID on PDU Session(s);

-	Allow the NWDAF to collect data for network data analytics from SMF as specified in TS 23.288 [50] and from UPF as specified in clause 4.15.4.5;

-	Allow the EIF to collect data from SMF as specified in clause 5.51.2 of TS 23.501 [2];

-	Notifying events on the PDU Session to the subscribed NFs; and

-	Allow consumer NFs to acknowledge or respond to an event notification.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE IP address / Prefix allocation/change: The event notification may contain a new UE IP address / Prefix or an indication of which UE IP address / Prefix has been released.

-	PDU Session Establishment and/or PDU Session Release.

The event notification may contain following information:

-	PDU Session Type.

-	DNN.

-	UE IP address/Prefix.

-	UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed.

The event notification may contain following information:

-	the type of notification ("EARLY" or "LATE").

-	for both the source and target UP path between the UE and the DN, the corresponding information is provided when it has changed:

-	DNAI.

-	UE IP address / Prefix.

-	N6 traffic routing information.

-	Candidate DNAI(s) for the PDU Session.

-	Change of common EAS.

-	Simultaneous Connectivity failure: a notification indicating that the simultaneous connectivity over the source and target PSA at edge relocation is not possible (as described in clause 6.3.4 of TS 23.548 [74])

NOTE 1:	UP path change notification, DNAI and N6 traffic routing information are further described in clause 5.6.7 of TS 23.501 [2].

NOTE 2:	The UP path change notification includes DNAI to represent the change of satellite identifier in case of UE-Satellite-UE (UE-SAT-UE) communication.

NOTE 3:	DNAI value can be derived from the satellite identifier as per operator policy and implementation.

-	QoS Monitoring: the event notification may contain the QoS Monitoring report for the QoS parameter(s) to be measured defined in clause 5.45 of TS 23.501 [2]. Implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) may trigger SMF to send this event notification.

-	Change of Access Type; The event notification contains the new Access Type for the PDU Session. For MA PDU Session the Change of Access Type may include two Access Type information that the user is currently using.

-	Change of RAT Type; the event notification contains the new RAT Type for the PDU Session.

-	PLMN change; The event notification contains the new PLMN Identifier for the PDU Session and may indicate:

-	whether local traffic offload is possible, i.e. mobility of the PDU session either towards HPLMN or towards a VPLMN where HR-SBO is supported and allowed; and

-	DNN and S-NSSAI of HPLMN.

-	Change of Satellite backhaul category; The event notification contains the new Satellite backhaul category for the PDU session.

-	Downlink data delivery status. The event notification contains the status of downlink data buffering in the core network including:

-	First downlink packet per source of the downlink IP traffic in extended buffering and Estimated maximum wait time.

-	First downlink packet per source of the downlink IP traffic discarded.

-	First downlink packet per source of the downlink IP traffic transmitted after previous buffering and/or discarding of corresponding packet(s).

-	QFI allocation: The event notification is sent whenever a new QoS Flow is established and contains:

-	If the Target of Event Reporting is a PDU Session and the QoS Flow is associated with this PDU Session, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI or QoS parameters corresponding to the QoS Flow and the DNN, S-NSSAI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is a SUPI and the PDU Session is associated with this SUPI, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI or QoS parameters corresponding to the QoS Flow and the DNN, S-NSSAI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is an Internal-Group-Id and the PDU Session is associated with this Internal-Group-Id (i.e. the PDU Session belongs to a UE belonging to this Internal-Group-Id), both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI or QoS parameters corresponding to the QoS Flow and the DNN, S-NSSAI, PDU Session ID, SUPI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is any UE, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI or QoS parameters corresponding to the QoS Flow and the DNN, S-NSSAI, PDU Session ID, SUPI corresponding to the PDU Session are also sent.

-	QoS Flow change: The event notification is sent whenever application traffic is bound to an existing QoS Flow or the QoS parameters of the QoS Flow that the application traffic is bound are modified. The Target of Event Reporting may be SUPI, Internal-Group-Id, or any UE. Related event notifications contain parameters as specified for the QFI allocation event.

-	QFI deallocation: The event notification is sent whenever the QoS Flow is released. The Target of Event Reporting may be SUPI, Internal-Group-Id, or any UE. Related event notifications contain parameters as specified for the QFI allocation event.

NOTE 4:	When the consumer NF is the NWDAF, the QFI allocation, QoS Flow change, and QFI deallocation events are used to collect data for analytics as specified in TS 23.288 [50].

-	Total number of Session Management transactions:

-	The total number of Session Management transaction is used to collect the number of SM transactions of a SUPI or Internal Group ID, for example Dispersion Analytics as specified in TS 23.288 [50]. The transaction count is incremented when the NAS transactions from PDU Session Establishment, PDU Session Authentication, PDU Session Modification and PDU Session Release procedures is concluded. Only the periodic reporting mode applies.

-	Information on PDU Session for WLAN (i.e. Access Type is Non-3GPP and RAT Type is TRUSTED_WLAN).

-	User plane status information: The event notification contains:

-	PDU Session ID.

-	User Plane Inactivity Timer (as specified in TS 29.244 [69]).

-	PDU Session status (activated, deactivated).

NOTE 5:	When the consumer NF is the NWDAF, the event user plane status information is used to collect data for UE Communication analytics as specified in TS 23.288 [50].

-	Session Management Congestion Control Experience for PDU Session: The event notification contains the data related to Session Management Congestion Control experience per PDU Session as described in TS 23.288 [50].

-	UE session behaviour trends (see clause 4.15.4.3);

-	UE communications trends (see clause 4.15.4.3);

-	UP with redundant transmission: the event notification indicates if redundant transmission (see clause 5.33.2.2 of TS 23.501 [2]) has been activated or not for the PDU session;

-	User Data Usage Measures (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF. For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of SMF. Consumer receives the events directly from UPF; and

-	User Data Usage Trends (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF. For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of the SMF. Consumer receives the events directly from UPF.

-	Service Signalling characteristics: The event notification includes Request type and number from NF, Number of receiving Session Report from UPFs, State transition information, Timer information, Unexpected operational status indicator, etc. from SMF for Signalling Storm Analytics as specified in clause 6.22.2 of TS 23.288 [50].

When the consumer NF is the NWDAF, the event Information on PDU Session for WLAN is used to collect data for WLAN performance analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the event Session Management Congestion Control Experience for PDU Session is used to collect data for Session Management Congestion Control Experience analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the events QoS Monitoring, User Data Usage Measures and User Data Usage Trends are used to collect data from UPF for analytics as specified in clause 4.15.4.5 and in TS 23.288 [50]. SMF conveys the subscription to UPF on behalf of the NWDAF or the SMF conveys the subscription to I-SMF on behalf of the NWDAF and the I-SMF conveys the subscription to UPF on behalf of SMF.

The consumer NF may request to subscribe the UPF exposure events (including event ID of exposed UPF event of QoS monitoring, User Data Usage Measures and User Data Usage Trends) via SMF indirectly by Nsmf_EventExposure. After receiving this subscription request, the SMF does a third-party subscription onto UPF on behalf of this consumer. The consumer should also provide the subscribed UPF event to SMF.

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for SMF exposure.

Table 5.2.8.3.1-1: Example of Event Filters for SMF exposure events

| Event ID for SMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| DNAI Change | None |
| Candidate DNAI(s) has changed | None |
| PDU Session Release | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| PDU Session Establishment | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QoS Monitoring | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = AoI, value = AoI1> <Parameter Type = UPF Id, value = UPF Id1> <Parameter Type = DNAI, value = DNAI1> |
| QFI allocation | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QFI allocation | <Parameter Type = Application Identifier, Value = Application Identifier1> |
| QoS Flow change | <Parameter Type = Application Identifier, Value = Application Identifier1> |
| QFI deallocation | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| Transaction Count | <Parameter Type = TAI, Value = TA1> (NOTE) <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| User plane status information | <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = SUPI, Value = SUPI1> |
| Information on PDU Session for WLAN | <Parameter Type = Access Type, Value = Non-3GPP> && <Parameter Type = RAT Type, Value = TRUSTED_WLAN> |
| Session Management Congestion Control Experience for PDU Session | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| UP with redundant transmission | <Parameter Type = DNN, Value = DNN1> |
| User Data Usage Measures | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) <Parameter Type = AoI, value = AoI1> <Parameter Type = SSID/BSSID, Value = SSID/BSSID1> |
| User Data Usage Trends | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) <Parameter Type = AoI, value = AoI1> |
| NOTE 1: Optionally the SMF can fetch the location information from the AMF but transaction information correlation at the location can also be achieved without it and through transaction information associated with the requested time period, which corresponds to the UE's time span at the location of interest. NOTE 2: These Parameters are exclusive and only one of them can be provided. |  |

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI), an Internal Group Identifier, an indication that any UE is targeted (e.g. on a specific DNN), or an indication that any PDU session is the target.

An SMF receiving Nsmf_EventExposure Subscription request to subscribe (e.g. to UPF event exposure) for "Any UE" does not consider PDU sessions for which it is acting as I-SMF when it selects the PDU session(s) it has to consider.

When acknowledgment is expected the SMF also provides Notification Correlation Information to the consumer NF in the event notification.

The consumer NF may provide the following event-specific information when acknowledging an event notification:

-	For UP path change event:

-	N6 traffic routing information related to the target DNAI.

NOTE 6:	Acknowledgement to a UP path change event notification is further described in clause 5.6.7 of TS 23.501 [2].

End of change