SA WG2 Meeting #169	S2-2504555

7 - 11 April 2025, Fukuoka, Japan

Source:	ZTE

Title:	NF selection and triggers for UE data collection

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes the triggering procedure and NF selection for UE data collection.

# 1. Discussion

3GPP SA2 Release 20 has approved the study item "AI/ML Core Network Enhancements Phase 2", which includes the research task of "whether and how to support standardized user plane transmission for AI/ML-related UE data collection." This paper proposes a NWDAF selection mechanism for the NWDAF that supports user plane UE data collection and some trigger procedures for the UE training data collection.

# 2. Proposal

This paper proposes to add a solution for Key Issue #1 to TR 23.700-04 as follows.

---------- Start of change ----------

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue 1> | <Key Issue #y> |
| #1 | X |  |
| #2 |  |  |

---------- Next change (all new text) ----------

6.x	Solution #X: NF selection and triggers for UE data collection

6.x.1	High level principles

This solution addresses KI#1 by defining triggering mechanisms and NF selection procedures for UE data collection. The data collection process can be initiated by either a trusted AF or an untrusted AF.

6.x.2	Functional Description

To establish a user plane connection between the UE and NWDAF for AI/ML user data collection, this solution proposes some mechanisms for NWDAF selection and triggers UE training data collection. AF (possibly via NEF) provides UE training data collection request with some parameters related to AI/ML user data collection. The trust AF or NEF needs to select an appropriate NWDAF and sends or forward the training data collection request to the chosen NWDAF.

6.x.2	Procedures

6.x.2.1	NWDAF registration

NRF

NWDAF

1.Nnrf_NFmanagement_NFResiter/NFupdate_Request

2. NF profile storage/update

3. Nnrf_NFManagement_NFRegister/Update_Response

Figure 6.x.2.1: AI/ML data collection capability registration

1. NWDAF registers or updates its AI/ML user plane data collection capability and the time that it support AI/ML user plane data collection capability to NRF.

2. NRF stores or updates the capability

3. NRF returns registration or update result indication.

6.x.2.2	UE training data collection triggered by trust AF

NWDAF

AF

NRF

UDM

1. Nnrf_NFDiscovery_Request

2. Nnrf_NFDiscovery_Response

3. AI/ML training data collection request

4. Parameters verification and User consent check

5. AI/ML training data collection response

Figure 6.x.2.2: Trust AF triggered UE training data collection

- 1. AF invokes NRF services to send a message such as Nnrf_NFDiscovery_Request, which may include the following information: NF Type = "NWDAF", AI/ML user plane data collection capability, Location information (e.g. TAI, AoI), Time information (e.g. data collection time period).

- 2. NRF responds with  Nnrf_NFDiscovery_Response, which includes information about one or more NWDAF(s).

- 3. AF  can invoke NWDAF services such as Nnwdaf_DataManagement_Subscribe or a new service specifically for AI/ML training data collection. The training data collection request may include following parameters:

- Indication for AI/ML-related (user) data collection

- Application information, such as Application ID, AF ID, and Application Address

- UE information, including UE ID (e.g. GPSI), UE Group Information (e.g. Group ID) or indications for data collection from any UE.

- Time-related information, including data collection time window, required data validity period.

- Location/area information for data collection, such TAI, Cell ID.

- Data type, for example, reference signal information (e.g. CSI, SSB) and corresponding RSRP information, UE positioning information (e.g. latitude and longitude), UE antenna orientation information, codebook information, and radio measurement data information

- Data sample information, such as number of data samples and required data quality thresholds

- Subscription and notification address for collected AI/ML-related (user) data information

- 4. NWDAF performs user consent confirmation for one or more parameters in the AF request. This may involve invoking UDM services, where NWDAF sends relevant parameters (e.g. UE ID) to UDM for user consent check and UDM returns the validation result.

- 5. NWDAF responds to AF with a response message, which may include: Assigned AI/ML data collection reference ID or  AI/ML data collection association ID, Authorized UE list or unauthorized UE list, expected data collection completion time or time when collected data will be provided to AF.

6.x.2.3	UE training data collection triggered by untrust AF

AF

UDM

NEF

NRF

NWDAF

1. AI/ML training data collection request

2. Parameters mapping, user consent check, NWDAF selection

3. AI/ML training data collection request

4. AI/ML training data collection response

5. UE training data collection completion time

Figure 6.x.2-3: Untrust AF triggered UE training data collection

1. The untrsuted AF sends a AI/ML training data collection request to NEF, providing parameters listed in the step 3 of 6.x.2.2.

2. NEF may perform various operations, including mapping external parameters to core network internal parameters, such as converting location information into TAI  and Cell ID. It can also invoke UDM services (e.g. Nudm_SDM_Get) for parameter mapping, like converting GPSI to SUPI or mapping an external group ID to an internal group ID. Additionally, NEF is responsible for user consent confirmation for one or more UE IDs in the AF request, by sending the UE ID to UDM for verification and receiving the result. NEF also performs NWDAF selection, where if the selection is based on location information (e.g. TAI), NEF may invoke NRF services and find a NWDAF has user plane AI/ML data collection capability and covers the target TAI.

If it is based on UE information (e.g. UE ID), NEF may invoke UDM services to query the NWDAF serving the UE.  Alternatively, NEF may select NWDAF based on local configurations. Finally, NEF is also responsible for assigning a transaction reference ID or subscription association ID.

3. NEF sends the AI/ML data collection request to NWDAF, with mapped parameters and authorized UE list.

4. The NWDAF sends a response with estimated UE training data collection completion time to NEF.

5. The NEF forwards UE training data collection completion time to AF.The message can also include authorized UE list or unauthorized UE list, types of data collection authorized or not authorized for one UE or group of UE, together with the assigned reference ID or association ID.

6.X.3	Impacts on services, entities and interface

The trust AF is responsible for selecting the appropriate NWDAF, sending data collection request with AI/ML data collection parameters to NWDAF, and receiving a response from NWDAF.

The untrust AF is responsible for sending data collection request with AI/ML data collection parameters to NEF.

The NWDAF registers its AI/ML data collection capability with NRF, receives AI/ML data collection parameters from NEF or trust AF, and performs user consent verification for the provided parameters. It then sends a response back to NEF or trust AF, which may contain a list of authorized/unauthorized UEs and the expected data collection completion time.

The NEF receives AI/ML data collection parameters from untrust AF, performs user consent verification, and carries out NWDAF selection. Additionally, NEF forwards the AI/ML data collection parameters to NWDAF and subsequently receives a response from NWDAF, including the expected data collection completion time. It then sends a response to AF, which may include data collection authorization details and expected data collection completion time.

---------- End of changes ----------