SA WG2 Meeting #169	S2-2504557

# 7 - 11 April 2025, Fukuoka, Japan

Source:	ZTE

Title:	Model Evaluation Metrics Enhancement

Document for:	Agreement

Agenda Item:	30.1

Abstract of the contribution: This discussion paper presents some explanations for metrics of regression model.

Discussion

Extension of ML Model Evaluation Metrics in the Core Network

Currently, the standardized ML Model metric only contains accuracy. In the current TS 23.288 specification, the definition of accuracy (i.e. correct predictions divided by total number of predictions) is only applicable to assess the performance of classification models.

However, in most Analytics defined in TS 23.288, the output consists of continuous variables rather than discrete ones. For instance, in the commercially deployed Observed Service Experience (OSE) analytics,  NWDAF may predict continuous variable like the Service Experience over the Analytics target period.

Figure 1. Comparison of Classification and Continuous Prediction

As shown in Figure 1, continuous prediction and classification prediction are fundamentally different. The accuracy defined in TS 23.288 is not suitable for evaluating regression models.

- Proposals

- To evaluate the performance of classification models, a proposal is to introduce Mean Square Error(MSE), which the most commonly used metric for rating the regression model.

- The MSE can be calculated as follow:

- mlModelMSE = 1ni=1n(Yi −γi)2

Where n is the number of data in sample, Y is the observed value(ground truth data) and γ is the predicted output.