3GPP TSG-SA WG2 Meeting #169	S2-2504558

Fukuoka, Japan, 19th May 2025 – 23rd May 2025

Source:	OPPO

Title:	New Solution for KI#1: Establish the UP connection between UE and 5GC to support the standardized UE data collection.

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for how to provision the UE and how to establish the UP connection between UE and 5GC to support the standardized UE data collection.

- Discussion

SA2 has discussed and approved a R20 FS_AIML_CN_Ph2 SID which is captured in SP-250413. The aim of this study work is to investigate and identify potential architecture and system level enhancements to support AI/ML enhancements. Specifically, the objectives include:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

SA2 has agreed the KI#1 for the WT#1 which is captured in the TR 23.700-04. This key issue aims to provide solutions for UE data collection to meet the requirements for AI/ML for NR air interface with UE-side model training. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server.

To collect UE measurement data, a new network function in the 5GC named Data Collection Function (DCF) is introduced. In order to let UEs know what type of data the UEs need to be measured and how to report the data to the DCF, a solution for how to provison the UE with the UE data collection configuration and how to establish the UP connection between the UE and DCF is needed.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X | x |  |  |  |

* * * Next Change (All new text)* * * *

6.X	Solution #X: Establish the UP connection between UE and 5GC to support the standardized UE data collection

6.X.1	High level principles

This solution is for Key Issue#1 for UE data collection over UP.

The high level principles of the solution are:

- The UE side server/OTT server configure the DCF about the data collection application information including the Event ID and the corresponding data type the UE need to be measured and reported. The detail description is in the clause 6.X.3.1.

- The DCF provision the UE Data Collection configuration to the UE via control plane singalling including the Event ID and the corresponding data type the UE need to be measured and reported.

- The UE side server/OTT server sends the UE data collection request to the DCF including the AoI and the Event ID.

- The DCF selects the UEs to be used for the data colletion and establish the UP based connection with the UEs.

- The DCF sends the Event ID to the UE via UP to tigger the UE to perform the data measurement with the RAN.

- The UE sends the measured data to the DCF via UP with the Event ID.

- The DCF verify and match the UE transmitted data based on the data collection application information stored in the DCF and the Event ID.

- The DCF expose the UE transmitted data to the UE side server/OTT server.

The clarification of the different terminology for the data collection are as following:

Data collection: Refer to the end-to-end procedure including how to provision data collection application information and UE data collection configuration and how the UE side server/OTT server request and receive the UE related data.

Data measurement: The UE interact with the RAN to obtain the data based on the measurement configurations and reference signals. This part is in the RAN scope.

Data transmission: The UE sends the measured data to the DCF via UP.

Data exposure: The DCF sends the data to the UE side server/OTT server.

Data verification: DCF verify/match the data configured to be collected and the data that is received.

6.X.2	Description

To enable the 5GC to collect UE data and exposure the collected UE data to the UE side server/OTT server, a Data Collection Function (DCF) is introduced with the key functionalities as following:

- Receive the Data collection application information provisioned by the UE side server/OTT server.

- Send the UE Data Collection Configuration to the UE.

- Receive data collection requests from UE side server/OTT server.

- Determine the data collection method (e.g., CP-based or UP-based).

- Identify the target UE(s) from which data needs to be collected.

- Establish a user plane connection with the target UEs.

- Send data collection request to the UE.

- Receive UE measured data.

- Match and verify the data transmitted from the UE.

- Expose the collected data to the UE side server/OTT server.

Bullet 1-7 and 10 aim to achieve how 5GC can support the full controllability of standardized UE data transfer.

Bullet 2, 8, 9, 10 aim to achieve how 5GC can support the full visibility of standardized UE data contents.

6.X.3	Procedures

6.X.3.1      UE Data Collection Mapping Table

To minimize the consumption of network and air interface resources during the exchange of data collection requests and configuration among the UE, DCF, and UE side server/OTT server, the UE side server/OTT server is responsible for creating and maintaining a UE Data Collection Mapping Table. The UE Data Collection Mapping Table is included in the data collection application information and UE Data Collection Configuration. The example of the mapping table is:

Table 6.X.3.1-1: UE Data Collection Mapping Table

| Event | Description | Data Type (This is determined by RAN) |
| --- | --- | --- |
| Event ID 1 | This event is used to collect the CSI feedback related data from UE | The data type needs to be collected from UE |
| Event ID 2 | This event is used to collect the Beam management related data from UE | The data type needs to be collected from UE |
| … | … | … |

Before initiating a data collection request, the UE side server/OTT server configures this mapping table across all relevant entities, including the UE and the DCF. When data collection related messages (such as requests/responses or subscriptions/notifications) are exchanged among the UE, DCF and UE side server/OTT server, only the event ID need to be carried. Each node can then refer to the mapping table to determine the corresponding measurement data type.

6.X.3.2      Data collection application information and UE data collection configuration provision

Figure 6.X.3.2-1: Data Collection Application Information and UE data collection configuration provisioning over CP

- DCF subscribes to the UDR on any modification of Data collection application information by invoking Nudr_DM_Subscribe (AF service parameter provisioning information, SUPI, Data Set setting to "Application Data", Data Subset setting to "Service specific information")

- To create a new request, the AF invokes an Nnef_ServiceParameter_Create service operation.

To update or remove an existing request, the AF invokes an Nnef_ServiceParameter_Update or Nnef_ServiceParameter_Delete service operation together with the corresponding Transaction Reference ID which was provided to the AF in Nnef_ServiceParameter_Create response message.

The request includes Data collection application information. The data colletion application information includes the UE Data Collection Mapping Table.

- The NEF authorizes the AF request. The NEF performs the following mappings:

- Map the AF-Service-Identifier into DNN and S-NSSAI combination, determined by local configuration.

- Map the External Application Identifier into the corresponding Application Identifier known in the core network.

In the case of Nnef_ServiceParameter_Create or Update: The NEF stores the AF request information in the UDR as the "Application Data" (Data Subset setting to "Service specific information") together with the assigned Transaction Reference ID.

In the case of Nnef_ServiceParameter_delete: The NEF deletes the AF request information from the UDR.

- The NEF responds to the AF. In the case of Nnef_ServiceParameter_Create response message, the response message includes the assigned Transaction Reference ID.

- UDR notifies DCF on any modification of Data collection application information by invoking Nudr_DM_Notify.

- DCF stores/modifies/removes the Data collection application information based on what is received in Nudr_DM_Notify.

Step 7 onwards can happen in parallel with step 1-6

- UE registers to the network as defined in clause 4.2.2.2.2 of TS 23.502.

- If UE indicates the capability of data collection in the registration request, the AMF checks with UDM on whether the UE is authorised to perform data collection.

- If UE is authorised to perform data collection, the AMF initiates UE Data collection configuration association establishment/modification to the DCF.

- On any modification of Data collection application information received from the UDR, the DCF provides the updated UE Data collection configuration in a container to the AMF by invoking Namf_Communication_N1N2MessageTransfer.

- If UE is in CM-IDLE and reachable by AMF in 3GPP access, the AMF starts the paging procedure by sending a Paging message described in the step 4b of Network Triggered Service Request (in clause 4.2.3.3 of TS 23.502). Upon reception of paging request, the UE shall initiate the UE Triggered Service Request procedure (clause 4.2.3.2 of TS 23.502).

- If the UE is in CM-CONNECTED over 3GPP access or non-3GPP access, the AMF transfers transparently the UE Data Collection configuration received from the DCF to the UE.

- The UE updates the UE Data Collection configuration provided by the DCF and sends the result to the AMF.

- The AMF forwards the response of the UE to the DCF using Namf_Communication_N1MessageNotify.

6.X.3.3      UP connection establishment and UE data collection request between the UE and DCF to support the UE data collection

To enable UP-based data collection, a UP connection need to be established between the UE and the DCF. When the DCF receive the UE data collection request, and if there is no existing connection between the target UE and the DCF, DCF will tigger the UP connection establishment.

Figure 6.X.3.3-1: UP connection establishment and UE data collection configuration

- When the UE side server/OTT server wants to collect the UE data to perform the UE-side model training, it sends a UE data collection request to the DCF. The request may include:

- The Event ID.

- The data collection period, indicate the time window that the data need to be collected from UE.

- The maximum response time, indicate when the collected data need to be response to the UE side server/OTT server.

- The area of interest (AoI) to indicate a certain area the data conusmer want to collect the data to perform model training.

- Data size.

- The required number of the data samples.

Editor´s note: How the UE side server/OTT server discovers the DCF to support the UE data collection is FFS.

- Based on the UE data collection request, the DCF determines which UEs to be selected to perform data collection.

Editor´s note: How the DCF select the UEs to collect the data is FFS.

- The DCF determines to use UP-based connection for UE data collection. DCF may invoke Nnrf_NFDiscovery service operation to retrieve control plane congestion status (e.g. AMF load information). DCF may also invoke Nnrf_NFManagement_NFStatusSubscribe service to subscribe specific AMF load information. Based on AMF load information, DCF may determine to use user plane UE data collection. Steps 4-10 are skipped if there is already a user plane connection context of the target UE in DCF and DCF determines to utilize the user plane connection. The DCF may further determine the protocol to be used for collecting UE data.

- If DCF decides to utilize user plane for UE data collection and there is no established secure user plane connection between the UE and DCF, the DCF sends its user plane information to the AMF via Namf_communication_N1N2MessageTransfer. This information includes the DCF's address, represented as an IP address or FQDN. Additionally, the DCF assigns a UE data collection-UP binding ID to each UE. This ID may also be included in the user plane information sent to the AMF. The UE data collection-UP binding ID serves to bind the target UE with the user plane connection.

- The AMF sends the DCF's user plane information to the UE via NAS message.

- Based on the DCF's user plane information, the UE initiates establishment of the user plane connection with the DCF.

- UE sends an acknowledgement to DCF through AMF to indicate a success of user plane connection establishment.

- AMF sends the acknowledgement received in step 7 to the DCF via Namf_N1messageNotify service.

- DCF indicates AMF in the Ndcf_UEDataCollection_UPNotify message that user plane connection between the UE and DCF has been established.

- The AMF stores the UE data collection-UP context. The UE data collection-UP context indicates which DCF the UE has established a user plane connection with.

- Since the UE maintains a mapping of Event IDs to the corresponding required data type, the DCF only needs to send the Event ID to the UE to trigger the UE perform the data measurement. The DCF sends a data collection request to the UE, which includes the Event ID.

- UE locally maps the Event ID to the specific data types that need to be collected. The UE interacts with the RAN to obtain measurement configurations and reference signalling, then performs the required data collection.

- UE encapsulates the measured data into data packets and sends them to the DCF via the user plane connection. The data packets sent by the UE to the DCF include the Event ID.

- The DCF uses the Event ID provided by the UE and its locally stored mappping information to verify and match the expected data types that UE should be reported. The DCF may performs further formatting for the collected data according to the UE side server/OTT server 's request.

- The DCF sends a data collection response to the UE side server/OTT server including the Event ID and the corresponding collected data.

6.X.4	Impacts on services, entities and interfaces

DCF:

-	Support the Data Collection Application Information and UE data collection configuration provisioning.

-	Select the Target UE to perform the UE data collection.

-	Determine to use UP based connection to support the UE data collection and assign a UE data collection-UP binding ID.

-	Send the UP information to UE to assist UE establish the UP connection.

-	Send the Event ID to the UE via UP

-	Receive UE-side measured data.

-	Match and verify the data transmitted from the UE.

-	Expose the collected data to the UE side server/OTT server.

UE:

-	Receive and maintain the UE data collection configuration.

-	Receive the DCF UP information and establish the UP based connection with the DCF.

-	Support to collect the measurement data and send the collected data to DCF via UP.

* * * End of Change * * * *