3GPP SA WG2 Meeting #169	S2-2504594

Fukuoka, Japan, 19-23 May 2025

# Title:	[Draft] Further reply LS on signalling feasibility of dataset and parameter sharing

# Response to:	S2-2504512 (R2-2503169)

# Release:	Release 19

# Work Item:	NR_AIML_air-Core

Source:	SA2

To:	RAN1, RAN2, RAN3

Cc:	SA3, SA5

Contact Person:

Name:	Juan Zhang

E-mail Address:	juanzhan@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks RAN2 for the Reply LS on signalling feasibility of dataset and parameter sharing in S2-2504512 / R2-2503169. RAN2 listed the following non-OTA candidate solutions and asked SA2 to confirm if the solutions that impacted SA2 can be supported within Rel-19 existing architecture framework.

Table 1. non-OTA candidate solutions

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

SA2 discussed the corresponding solutions and approved the following conclusions.

CN -> UE-side training entity solution

The UE-side training entity is the AF in trusted domain or untrusted domain.

It is already supported the External Exposure of Network Capability by CN as described in clause 5.20 in TS 23.501. It is also defined that AF is allowed to interact with the 3GPP core network to access the Network Exposure Function in clause 6.2.10 in TS 23.501 as below:

- Based on operator deployment, Application Functions considered to be trusted by the operator can be allowed to interact directly with relevant Network Functions.

- Application Functions not allowed by the operator to access directly the Network Functions shall use the external exposure framework (see clause 7.3) via the NEF to interact with relevant Network Functions.

However, SA2 is not clear where is the data source to provide the dataset and parameters to CN and how does CN to retrieve the dataset and parameters that required to transfer to the UE-side training entity.

gNB -> OAM/CN -> UE-side training entity

gNB -> OAM -> UE-side training entity should be discussed in SA5.

For gNB -> CN -> UE-side training entity, as described above, it is feasible to expose information from CN to UE-side training entity. However, it is not supported to transfer data from gNB to CN in Rel-19 SA2 spec.

Considering another option gNB-> OAM -> CN -> UE-side training entity, SA2 thinks it is feasible to transfer data from gNB to OAM. It is already supported to collect information from OAM to CN for different features in SA2. However, it is not currently supported to expose the information from OAM to AF via CN in Rel-19 SA2 spec.

SA2 does not have plan to study this subject in Rel-20 for 5GA (see approved SID in SP-250413).  The status in SA2 for 5GA should not limit the discussion for SA2 in 6G.

2. Actions:

To RAN1, RAN2, RAN3:

ACTION: 	SA2 asks RAN1, RAN2 and RAN3 to consider the above information.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #170		25-29 August 2025	Goteborg, Sweden

TSG-SA2 Meeting #171		13-17 October 2025	China (TBD)