SA WG2 Meeting #169	S2-2504596

May 19 – May 23, 2025, Fukuoka, JP

Source:	Skylo Technologies, ViaSat

Title:	FS_5GSAT_Ph4_ARC, WT1: New Sol: Support of IMS Voice Call over NB-IoT NTN via GEO by using User Plane NIDD Fallback

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC /Rel-20

Abstract of the contribution: This paper proposes a new solution for KI#1-4 of the FS_5GSAT_Ph4_ARCH.

# 1	Discussion

## 5.1	Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

To support IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC, this key issue studies the following issues:

-	System enhancements to support IMS voice service over NB-IoT NTN via GEO satellite connecting to EPC;

-	Whether and how to support QoS for IMS voice service over GEO satellite.

## 5.2	Key Issue #2: IMS enhancement for GEO NB-IoT NTN access

To meet the requirements on experienced data rate and call setup time documented in TS 22.261 [8], this key issue studies:

-	Whether and how to enhance IMS to utilize NB-IoT as IP-CAN;

-	Potential IMS optimization.

## 5.3	Key Issue #3: Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC

This key issue studies:

-	How to support IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC to comply with the relevant requirements defined in TS 22.261 [8] and TS 22.101 [9].

## 5.4	Key Issue #4: Location service for IMS emergency call and regulatory services over NB-IoT NTN

This key issue studies whether existing location services can be used to obtain the UE location (e.g., geographical location) meeting regulatory requirements, when the UE establishes an IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC or other services for which regulatory requirements for obtaining network provided location information exist.

KI#1-4 of FS_5GSAT_Ph4_ARC has been agreed and documented in TR 23.700-19, captured in the text box below. This contribution proposes an end-to-end solution for KI#1-4.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.1.0.

* * * * Start of Changes (all new text) * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| x | x | x | x | x |

## 6.X	Solution #X: Support of IMS call over NB-IoT NTN via NIDD fallback

### 6.X.1	High level principles

Editor's note:	This clause will describe the solution principles.

To enable IMS-based voice services over NB-IoT access through GEO satellite connected to the EPC, the following high-level architectural principles in Clause 6.x.1.1 to 6.x.1.3 are proposed.

#### 6.X.1.1	Separate DRBs for Signalling and Media at RAN

It is proposed to establish separate Data Radio Bearers (DRBs) for signaling and voice media packets due to different transport requirements for these two types of packets. Voice packets require consistent jitter and delay handling and may benefit from prioritization or special treatment in satellite links. This separation also aligns with how IMS networks typically handle control and media, enabling better traffic management and potential for differentiated satellite resource allocation.

#### 6.X.1.2	User Plane CIOT EPS Optimisation with NIDD

NB-IoT channels operating over GEO satellites face significant bandwidth constraints, limited to 180 kHz usable bandwidth. Moreover, these constraints restrict the number of simultaneous voice calls over a non-anchor carrier. To maximize transmission efficiency over these narrowband links, it is proposed to use User Plane CIoT EPS Optimization with Non-IP Data Delivery (UP NIDD) for both signaling and voice media.

Table 6.x.1.2-1: Data Delivery Overhead Comparisons

| Bytes | RTP | UDP | IP | PDCP | RRC | NAS | RLC | MAC | BSR | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPv4 | 12 | 8 | 20 | 1 |  |  | 2 | 1 | 2 | 46 |
| IP with RoHC |  |  | 1 | 1 |  |  | 2 | 1 | 2 | 7 |
| UP NIDD |  |  |  | 1 |  |  | 1 | 1 | 2 | 5 |
| CP NIDD |  |  |  |  | 2 | 11 | 2 | 1 | 2 | 18 |

As shown in Table 6.x.1.2-1 below, UP NIDD provides the lowest protocol overhead, making it especially suitable for constrained and high-latency environments such as GEO-based NTN. UP NIDD, operating over the user plane, also enables the use of AS-level security mechanisms. Although NB-IoT does not support dedicated data radio bearers, the default bearer used by UP NIDD allows for radio-level optimization (e.g., RLC AM for signaling, RLC UM for media), which can be configured for improved reliability and latency control.

IPv4 is not ideal in this context due to the relatively high overhead of protocol headers. IP with RoHC adds additional complexity and CPU processing for both the UE and the network. This makes it less efficient for NB-IoT, where devices are constrained by limited processing capability, power consumption, and memory footprint.

#### 6.X.1.3	Use of SCC-AS architecture for IMS Session Setup

SIP is not suitable for IMS session setup over NB-IoT NTN GEO links for several reasons:

- High signaling overhead (typically 500–1,500 bytes per message);

- Multiple round trips required (INVITE, 180 Ringing, 200 OK, ACK);

- Lack of SIP stack support on typical NB-IoT devices.

Instead, it is proposed to use IMS Service Centralization and Continuity architecture (see clause 4.20 of TS 23.228 and TS 23.292) for IMS session establishment over NB-IoT NTN GEO links. I1 protocol (as defined in 3GPP TS 24.294) is designed specifically for this scenario where the bearer path is CS or constrained, but IMS service control is still required.

The I1 protocol is highly lightweight, with fixed header-length of 7 bytes, making it well-suited for low-bandwidth, high-latency satellite links by significantly reducing transmission time. While originally designed for CS-to-IMS interworking and IMS Centralized Services (ICS), it can be readily adapted for NIDD-to-IMS interworking, as discussed in Clause 6.x.2.1.

I1 is already standardized and supported by the Service Centralization and Continuity Application Server (SCC-AS), and since it eliminates the need for a SIP stack on the UE, it also greatly simplifies device implementation.

### 6.X.2	Description

Editor's note:	This clause will describe high-level solution. (Sub) clause(s) may be added to capture details.

### 6.x.2.1 	General

Support of IMS Service Centralization is specified in TS 23.228 and TS 23.292. IMS service centralization provides communication services such that all services, and service control, are based on IMS mechanisms and enablers. It enables IMS services via CS access. This solution proposes to further expand the architecture to support IMS services when using NB-IoT NTN access with UP NIDD, as shown in Figure 6.x.2.1-1.

Figure 6.x.2.1-1: Overall Architecture for IMS Voice Call over NB-IoT NTN

The I1 protocol enables sessions that originate from or terminate in the NIDD domain to be anchored and controlled by IMS. This ensures service continuity and consistent behavior across both NIDD and IMS domains.

When a voice call is initiated or received by a user with an IMS subscription and NB-IoT NTN (GEO) access, the session falls back to a User Plane (UP) NIDD-based configuration. In this setup:

- Two DRBs are established per IMS session, one for I1 signaling and one for voice media. This separation allows media traffic to receive differentiated treatment, even in the absence of conventional QoS mechanisms.

- An Interworking Function (IWF), specifically the SCC-AS as defined in TS 23.292, receives session signaling over the I1 protocol and translates between I1 and SIP messages, enabling seamless integration with standard IMS components.

- The media parameters included in the I1 message allow the Media Gateway within the IWF to determine whether transcoding is required. By resolving codec mismatches internally, the system avoids traditional end-to-end codec negotiation, significantly reducing call setup latency, especially important in high-delay satellite environments.

The NIDD UP session is anchored at the Service Centralization and Continuity Application Server (SCC-AS), which maintains IMS session control and service logic regardless of the underlying access technology.

All external interfaces of the interworking function comply with existing 3GPP specifications, so no additional standardization is required. Since it operates entirely within the operator’s network domain, it can be deployed without impacting standardized interfaces. In practice, the interworking function can be integrated into the SBC, with the I1 and SIP message translation implemented as a B2BUA (as specified in TS 24.229), effectively providing a SIP trunk between the interworking function and the IMS core.

For transport, the User Plane uses RLC Acknowledged Mode (AM) for the signaling bearer to ensure reliable delivery, and RLC Unacknowledged Mode (UM) for the media bearer to reduce latency. Both bearers can be configured by the eNB as needed.

Because codec negotiation is handled entirely by the interworking function, there is no need for direct capability exchange between endpoints. This simplifies the UE design and enables a seamless, efficient voice experience across diverse network and device types.

### 6.X.2.2 	QoS support

NB-IoT does not support dedicated bearer setup with Guaranteed Bit Rate (GBR), which is commonly used in eMBB and LTE networks to support IMS call setup. To enable differentiated treatment of IMS session traffic over NB-IoT NTN via GEO satellite, alternative strategies must be considered.

One approach is to assign an existing or newly defined QCI to voice traffic over these links. This QCI is applied to the default bearer associated with a dedicated APN configured specifically for IMS services over NB-IoT NTN via GEO. The QCI parameters should be optimized to meet the performance requirements of conversational voice in high-latency, narrowband environments.

It is also recommended to configure separate APNs for signaling and media traffic, each using its own default EPS NIDD UP bearer. This enables the core network to prioritize traffic by assigning higher scheduling priority, applying distinct DSCP markings or packet filters, and optionally routing media traffic through a dedicated PGW optimized for low latency or accelerated processing.

At the RAN level, the DRBs established for IMS traffic over NB-IoT GEO should be treated with elevated priority, including preferential scheduling and minimized packet discard during periods of congestion.

### 6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.3.1 	MO Call Procedures

Fig. 6.X.3.1-1 provides an example flow for MO call procedures.

Figure 6.X.3.1-1: Example flow for MO call procedures

- UE-A access IMS service over NB-IoT NTN via GEO with an indication to use IMS over UP NIDD APN

- In step 1a, the UE-A initiates the attach procedure by the transmission of an Attach Request. The Attach Type indicates that the UE requests a combined EPS/IMSI attach and informs the network that the UE is capable and configured to use UP NIDD fallback for IMS voice via GEO.

In step 1b, steps 3 to 16 of the EPS Attached procedure are performed as specified in TS 23.401.

- In step 2a, the MME sends a Location Update Request towards MSC/VLR via SGs.

In step 2b, the MSC/VLR Server enhanced for ICS performs location update, authentication and obtains subscriber data.

In step 2c, a Location Area Update Accept is returned to the MME.

- Steps 17 to 26 of the EPS Attached procedure are performed as specified in TS 23.401. And the EPS Attach procedure is completed with default bearer, with the DRB for IMS I1 signalling.

- The MSC Server enhanced for ICS decides to initiate IMS registration for this subscriber if the subscriber is not already registered following Clause 7.2.1.2 of TS 23.292.

- A default bearer for RTP media session is established in this step. This bearer supports the RTP media session and utilizes the QoS mechanisms outlined in Clause 6.X.2.2.

- In Step 6a, UE-A initiates a call by sending an ICS Call Initiation Request over I1 reference point containing the UE-B address. In step 6b, the SCC AS sends the SCC AS PSI DN to the UE over the I1 reference point.

- UE-A sends an I1 INVITE to the SCC AS with SCC AS PSI and SDP-MGW, e.g. codec supported, etc.

- The SCC AS invokes a B2BUA, terminating the UE-A Leg and originating a leg for presentation of an IMS session towards UE-B on behalf of UE-A. The SCC AS correlates the signalling session bearer (over UP-NIDD) with the IMS service control signalling session using the SCC AS PSI and creates a SIP INVITE containing the SDP received in the UP-NIDD I1 signalling path. The INVITE request is routed from the SCC AS to the IMS core where standard IMS originated session processing and routes the request onwards to the UE-B.

- Completion of the Service Control Signalling Procedures.

- As described in Clause 6.x.2.1, with the knowledge of both UE-A and UE-B’s codec capabilities, the MGW determines, without further negotiation, whether a transcoding is necessary.

- Media Session continues between two parties UE-A and UE-B.

### 6.X.3.2 	MT Call Procedures

Figure 6.X.3.2-1: MT Call Procedures

Steps 0-4 in Figure 6.X.3.2-1 are identical to steps 0-4 in Figure 6.X.3.1-1.

- An incoming SIP INVITE is received at the IMS core of the UE-A party from UE-B.

- The IMS core (e.g. S-CSCF) executes terminating initial Filter Criteria and forwards the SIP INVITE to the SCC AS.

- The SCC AS sends a Session Progress response to the IMS, and forwards the Session Progress response to UE-B.

- The SCC AS performs Terminating Access Domain Selection (T-ADS) and if informed of the UE-A capability for I1 during IMS registration, chooses the NIDD UP domain for the setup of the media.

- The SCC AS terminates the session from UE-B and establishes a new session over I1 by sending an Incoming Call Request to the UE-A.

The Incoming Call Request may include an indication instructing UE-A to initiate the NIDD UP bearer establishment procedure for a second default bearer to support the RTP media session.

Additionally, the Incoming Call Request may contain the SCC AS PSI DN, allowing the SCC AS to later correlate outgoing service control signalling with the incoming I1 control signalling over the NIDD-UP bearer.

- A default bearer for the RTP media session is established in this step. This bearer supports the RTP media session and utilizes the QoS mechanisms outlined in Clause 6.X.2.2. In step 10a, the MGW determines codec selection for media session and perform transcoding if required.

- In steps 11a-11b, UE-A sends a I1 ALERT to the SCC AS, and the SCC AS forwards SIP 180 Ringing message to UE-B. In steps 11c-11d, UE-A sends I1 SUCCESS message to the SCC AS, and the SCC AS forwards a SIP 200 OK message to UE-B in response to the INVITE received at step 6.

- Media Session continues between two parties UE-A and UE-B.

### 6.X.3.3 	Emergency Call Procedures

If the SCC-AS detects an IMS emergency call over NIDD-UP via GEO, e.g. through emergency service indication in the initial I1 message or based on the dialed emergency number from an NB-IoT UE accessing via GEO NTN, it may interwork with IMS for routing the emergency session to the PSAP.

If emergency registration is performed using the I1 interface toward the SCC-AS, and the SCC AS then proceeds to send the emergency session initiation request to the E-CSCF in the IMS core using standard SIP signaling, by following the architecture in Figure 6.X.2.1-1.

If the SCC AS is required to handle anonymous emergency calls (e.g., due to local regulations), it shall skip the IMS registration step and directly send the anonymous emergency session initiation to the E-CSCF using SIP. The identity used may be derived from configuration or replaced with a temporary anonymous identifier as per IMS emergency call procedures.

If the emergency session is tagged with eCall indication from the UE (e.g., via a special I1 information element), the SCC-AS may include the IMS eCall indication in the SIP INVITE to the E-CSCF. The Minimum Set of emergency related Data (MSD) shall be transferred via the User Plane NIDD media bearer. The SCC AS and IMS shall transport the MSD data transparently end-to-end from UE to the PSAP, maintaining the format generated by the UE.

For a UE that has established a NIDD bearer with the SCC AS, but has not yet performed any form of logical registration for IMS service control (e.g., via I1), the SCC AS may follow a fallback behavior equivalent to a non-enhanced interworking node, potentially routing the call using pre-configured emergency handling mechanisms or default service logic.

It is further proposed that existing location services can be used to obtain the UE location (e.g., geographical location) meeting regulatory requirements, when the UE establishes an IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

### 6.X.4	Impacts to Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

MSC/VLR:

- Add the support of UP NIDD Fallback to support both Non-ICS UE and ICS UE

* * * * End of Changes * * * *