3GPP TSG-SA WG2 Meeting #169	S2-2504604

Fukuoka, Japan, 19th May 2025 – 23rd May 2025

Source:	OPPO

Title:	New Solution for KI#5: Sensing result delivery to the consumer

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution on Sensing result delivery to the consumer.

- Discussion

KI#5 have been defined in TR 23.700-14 as below:

This key issue addresses the exposure of sensing results to support sensing services. The aspects to be studied in this key issue include: - Study what types of sensing results with or without the sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the sensing service consumer. - How the Core Network exposes sensing results to the sensing service consumer (e.g., one time, periodic or event trigger).

For sensing result exposure, new sensing result may be derived from sensing data. Data collection often triggers sensing measurements. However, since multiple requests from diverse applications may seek identical results, reusing stored outcomes avoids redundant processing. This approach not only streamlines operations but also aligns with energy efficiency goals by minimizing unnecessary data generation and computation.

Sensing data storage preserves historical and real-time environmental context. Stored data ensures consistency for cross-service validation (e.g., AI training, network optimization) and supports regulatory requirements. For result exposure, centralized storage simplifies access via standardized APIs, allowing third parties or internal systems to retrieve processed insights (e.g., obstacle maps, interference patterns) without raw data duplication. It also facilitates on-demand querying, reducing redundant data collection. By decoupling storage from processing, stored datasets enable scalable, repeatable analytics, ensuring high-quality sensing results are dynamically exposed for applications like autonomous mobility or predictive network maintenance.

Sensing services, as a type of data service, can share the same data collection and capability exposure architecture with other data services (e.g., AI, positioning). However, due to the unique requirements of sensing services, there are also dedicated operations like sensing entity selection and sensing data processing. Thus, sensing service management is jointly implemented by the DSMF (Data Service Management Function) and SSMF (Sensing Service Management Function). The DSMF, a common network element for all data services, manages unified data collection and capability exposure. The SSMF, dedicated to sensing services, handles sensing entity selection, sensing result generation, and storage.

This solution primarily focuses on a DSMF-based capability exposure framework, leveraging shared infrastructure for efficiency while ensuring specialized processing for sensing-specific tasks through the SSMF. By integrating sensing into the broader data service ecosystem, operators avoid redundant architectures while maintaining tailored support for sensing requirements like real-time entity coordination and environmental context handling.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-14.

* * * Start of Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 | Key Issue #4 | Key Issue #5 | Key Issue #6 |
| #1 |  |  |  |  |  |  |
| #2 |  |  |  |  |  |  |
| #X | X |  |  |  | X |  |

* * * Next Change (All new text)* * * *

## 6.X	Solution #X: Sensing result delivery to the consumer

### 6.X.1	Description

This solution is for Key Issue#1 and Key Issue#5 on Sensing result delivery to the consumer.

Sensing services, as a type of data service, can share the same data collection and capability exposure architecture with other data services (e.g., AI, positioning). However, due to the unique requirements of sensing services, there are also dedicated operations like sensing entity selection and sensing data processing. Thus, sensing service management is jointly implemented by the DSMF (Data Service Management Function) and SSMF (Sensing Service Management Function). The DSMF, a common network element for all data services, manages unified data collection and capability exposure. The SSMF, dedicated to sensing services, handles sensing entity selection, sensing result generation, and storage.

To enable sensing result exposure, the DSMF needs to:

- Determine the sensing result provider(s).

- Determine the sensing service consumer(s).

- Determine the method to deliver sensing results to the consumer(s): via Control Plane (CP) or User Plane (UP).

- Contral delivering the sensing results from the sensing result provider(s) to the sensing service consumer(s).

To enable sensing result storage, the DSMF needs to:

- Determine the sensing result provider(s).

- Determine the sensing result storage entity(ies).

- Determine the method to deliver sensing results to the sensing result storage entity(ies): via Control Plane (CP) or User Plane (UP).

- Contral delivering the sensing results from the sensing result provider(s) to the sensing sensing result storage entity(ies).

The sensing result provider(s) can be a centrual data collection entity, SSMF, UE, or gNB.

The sensing service consumer(s) can be a UE, AF, gNB, or CN NF.

The sensing sensing result storage entity(ies) can be a centrual data storage entity or SSMF

### 6.X.2	Procedures

Figure 6.X.2-1: Sensing result exposure and storage service flow

- Non-sensing service is activated and the measurement data (which includes sensing data) is reported to the data collection entity.

- DSMF receives a sensing service request, and the service request is successfully authorized. The request includes:

- Consumer ID/Address

- Sensing Service Type

- Expected Sensing Result，including sensing result format

- Capability of the sensing service consumer for transferring sensing result over UP

- DSMF checks with data collection/storage entity if there is available sensing result matching the service request, by sending the information received from the Sensing Consumer.

If there is matching sensing result, data collection/storage entity responds to DSMF including one or multiple following parameters:

- Sensing result

- Indication of transferring sensing result over UP

- data collection/storage entity address

- If no sensing result or indication of transferring sensing result over UP or data collection/storage entity address is provided in step 3, DSMF sends the sensing service request to the SSMF, including what is received in step 2

- If there’s no matching sensing result as requested in the sensing service request, the DSMF performs sensing transmitter/receiver discovery and selection.

- SSMF responds to DSMF one or multiple of following parameters:

- sensing result (in case there’s available sensing result)

- Indication of transferring sensing result over UP

- the receiver ID/address (in case there’s no available sensing result)

- Sensing result provider ID/address

- DSMF determines the sensing result provider based on the following considerations:

- Sensing consumer information: location, whether selected as Sensing Receiver, capability of acting as Sensing Receiver, capability of sensing result generation

- Sensing Receiver information: location, capability of sensing result generation, load for result generation

- SSMF load for result generation

- SSMF selected sensing result producer

- Operator configuration

- DSMF determines sensing result transferring mode, i.e. whether to transfer it over UP, based on the following info:

- Capability of the sensing result provider for transferring sensing result over UP

- Capability of the sensing service consumer for transferring sensing result over UP

- Sensing result format

- Sensing service type

- Operator configuration

- If sensing result is already received at step 3 or step 6, DSMF responds to sensing service consumer of the sensing result; otherwise, it responds with the Indication of transferring sensing result over UP and the sensing result provider ID/Address

- DSMF configures the sensing result provider with one or multiple following parameters:

- Indication of transferring sensing result

- Indication of transferring sensing result over UP

- the sensing service consumer ID/Address

- If Indication of transferring sensing result over UP is received, and there’s no available UP Connection, new UP Connection is established between sensing result provider and sensing service consumer

- SSMF triggers the sensing transmitter/receiver to perform sensing

- Sensing data collection and result generation are performed

- If Indication of transferring sensing result over UP is received, the sensing result is transferred to the sensing service consumer and the sensing result storage entity over UP. Sensing result is stored. Step 15-18 and Step 20-21 are skipped.

- Sensing receiver or SSMF notifies the DSMF about the sensing result

- DSMF notifies the sensing service consumer about the sensing result.

- Alternative to step 15 and step 16, Sensing receiver or SSMF directly notifies sensing service consumer about the sensing result

- Sensing receiver notifies SSMF about the sensing result

- Sensing result is stored at SSMF

- Sensing receiver/SSMF notifies data collection/storage entity about the sensing result

- Sensing result is stored at data collection/storage entity

### 6.X.3	Impacts on services, entities and interfaces

DSMF:

  - Determine the sensing result provider(s).

  - Determine the sensing service consumer(s).

  - Determine the method to deliver sensing results to the consumer(s): via Control Plane (CP) or User Plane (UP).

  - Contral delivering the sensing results from the sensing result provider(s) to the sensing service consumer(s).

  - Determine the sensing result storage entity(ies).

  - Determine the method to deliver sensing results to the sensing result storage entity(ies): via Control Plane (CP) or User Plane (UP).

  - Contral delivering the sensing results from the sensing result provider(s) to the sensing sensing result storage entity(ies).

SSMF:

  - Support sensing result generation

  - Support sensing result storage

Receiver:

  - Support sensing data collection to SSMF

  - Support sensing data collection to data collection entity

  - Support sensing data collection configuration by NF managing non sensing service

  - Support sensing measurement

NF managing non-sensing service:

  - Support sensing data collection configuration to the Sensing Receiver

Data collection/storage entity:

  - Support sensing data collection

  - Support sensing result storage

* * * End of Change * * * *